3GPP TSG-RAN WG2 Meeting #127bis	R2-2408780

Hefei, China, Oct 14~18, 2024

Agenda item:	8.7.6

Source: 	Huawei, HiSilicon

Title: 	Discussion on XR rate control

Document for:	Discussion and Decision

1. Introduction

During RAN#105 meeting, a new objective aimed to specify MAC layer XR rate control for uplink congestion adaption was introduced [1].

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In this contribution, we will analyse the legacy MAC layer XR rate control mechanism, i.e. the recommended bit rate procedure, to see whether there is anything not suitable for XR services, and give our view on the potential enhancements.

2. Discussion

2.0	Requirements for the perQoS/perDRB rate recommendation

During Rel-18 XR, RAN2 has discussed about the potential bit rate range for XR services when designing the new BS table. The related agreements are excerpted below. According to the agreements, it can be seen that the minimum bit rate for XR is assumed as 10Mbps while the maximum is 60Mbps.

RAN2#124 agreements: 9. The maximum buffer size can be determined based on the ratio between maximum link rate (60Mbps) and minimum frame rate (15 fps), which is 750KB. 10. The minimum buffer size can be determined based on the ratio between minimum bit rate (10 Mbps) and maximum frame rate (120 fps), which is 5 KB

For rate recommendation in DRB/QoS flow level, we think the same agreement also applies, that the range of applicable rate should fall into this region.

Proposal1: The range of rate applicable for per QoS flow/per DRB rate recommendation is 10 Mbps to 60 Mbps, same as that of refined BSR.

2.1 Per QoS flow bit rate recommendation

The Access Network Bitrate Recommendation (ANBR) procedure is used to provide the UE with the information about the bit rate which the gNB recommends. It can be used to enable the rate adaptation in the application layer according to the congestion status.

Currently, the gNB can use the Recommended bit rate MAC CE to indicate the recommended bit rate for the UE for a specific LCH. However, such per LCH indication may be not refined enough for XR services. Considering it is possible that multiple QoS flows are mapped to the same LCH, when the gNB indicates a recommended bit rate for the LCH, it is confusing for the UE to determine the bit rate for each QoS flow over the LCH. Some companies may think it can be resolved based on the UE implementation. For example, the UE can divide the recommended bit rate evenly among the multiple QoS flows. However, this is not always reasonable and may be not according to the network expectation.

To make the recommendation more flexible and accurate, it is better to support per QoS flow indication, so that the gNB can determine the recommended bit rate for each QoS flow based on their requirements and characteristics.

Proposal2: RAN2 to support per QoS flow level bit rate recommendation.

2.2 Granularity of the recommended bit rate

Another issue worth noting is the granularity of the recommended bit rate indication. In NR Rel-15 Recommended bit rate MAC CE, 56 codepoints are used to represent the recommended bit rate values from 0 to 8000kbps. In NR Rel-16 E_FLUS [3], in order to support uplink streaming services with extremely high data rates, a multiplier factor which can take a value from 40/70/100/200 was introduced to scale up the recommended bit rate values. With such a multiplier, RAN can indicate a recommended bit rate up to 1.6Gbps, which should be sufficient for XR services.

However, 56 codepoints combined with only 1 configured multiplier may not be able to reflect a bit rate with sufficient granularity. For example, when RAN estimates an available data rate for an LCH is 23Mbps, RAN may indicate a recommended bit rate 20Mbps by using codepoint 30, i.e. 500kbps if the multiplier 40 is configured for the LCH. This incurs some inaccuracy which may affect the service’s experience.

RAN2 may consider some solutions to solve the above issue. From our point of view, the following alternatives can be taken into account.

- Alternative 1: Define a new recommended bit rate table for XR.

The current recommended bit rate table defined in TS 38.321 is not suitable for XR services. When no multiplier is used, the range of the table is 0~8 Mbps which is too small. While when a multiplier is used, the range may be too large, making the granularity rough. For example, assuming the multiplier is set to 40, the gNB can indicate recommended bit rate from 0 to 320 Mbps with only 17 codepoints within the range 0~60 Mbps, i.e. the indexes 24~40.

Similar to Rel-18 BSR enhancement, to make the recommendation more useful for XR, RAN2 can consider to design a new table which specifically matches the potential bit rate range of XR services. One reserved bit in the recommended bit rate MAC CE can be used to indicate whether the new table is used.

- Alternative 2: Use dynamic multiplier in the recommended bit rate MAC CE.

Currently, the multiplier can only be semi-statically configured to a value from 40/70/100/200. The limited values of multiplier make the recommended bit rate indication inflexible. A potential enhancement is to support dynamic multiplier, so that the gNB can indicate the recommended bit rate as accurately as possible by selecting the proper bit rate index and multiplier. For example, to indicate 23Mbps, the index 36 (corresponding to 1.1Mbps) and multiplier 21 can be used together.

The dynamic multiplier can be explicitly indicated in the MAC CE. Or to reduce the MAC CE overhead, the gNB can pre-configure several multipliers, and indicate which one is used in the recommended bit rate MAC CE. The reserved bits in the current MAC CE can be reused.

Proposal3: RAN2 to refine the recommended bit rate granularity by enhancing the recommended bit rate table or MAC CE. The following alternatives can be considered:

- Alternative 1: Define a new recommended bit rate table for XR.

- Alternative 2: Use dynamic multiplier in the recommended bit rate MAC CE.

2.3 UL MAC CE for UE desired bit rate

In ANBR procedure, besides the DL recommended bit rate MAC CE sending by the gNB to the UE, there is also an UL MAC CE which is used to indicate the UE desired bit rate. The UL MAC CE has the same format as the DL MAC CE. It is also per LCH indication and refers to the same bit rate table as the DL MAC CE.

If the DL recommended bit rate MAC CE is enhanced from the above aspects, e.g., to support per QoS flow indication and to support refined bit rate granularity, there should be corresponding enhancements for the UL MAC CE to allow the UE to indicate the desired bit rate.

Proposal4: For ANBR, the UL MAC CE should be enhanced in the same way as the DL MAC CE to allow the UE to indicate the desired bit rate.

3. Conclusion

In this contribution, we analyzed the potential enhancements for XR rate control. The following proposals were made:

Proposal1: The range of rate applicable for per QoS flow/per DRB rate recommendation is 10 Mbps to 60 Mbps, same as that of refined BSR.

Proposal2: RAN2 to support per QoS flow level bit rate recommendation.

Proposal3: RAN2 to refine the recommended bit rate granularity by enhancing the recommended bit rate table or MAC CE. The following alternatives can be considered:

- Alternative 1: Define a new recommended bit rate table for XR.

- Alternative 2: Use dynamic multiplier in the recommended bit rate MAC CE.

Proposal4: For ANBR, the UL MAC CE should be enhanced in the same way as the DL MAC CE to allow the UE to indicate the desired bit rate.

4. Reference

[1]. RP-241771, Nokia (Rapporteur), Revised WID on XR (eXtended Reality) for NR Phase 3.

[2]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[3]. R2-1911499, LS on Recommended Bit Rate/Query for FLUS and MTSI, 3GPP TSG RAN WG2#107.