# 3GPP TSG RAN WG2 Meeting #127bis                          	   R2-2408785

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SSB SCell Operation

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives of on-demand SSB SCell are outlined with the follows [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and the potential RAN2 specification work.

# Discussion

## Use cases and Scenarios for On-demand SSB

At RAN1 #116, the following aggreements were made on the following use cases and scenarios for further study [2].

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission

Additionally, the following scenarios with Case #1 (no always-on SSB) and Case #2 (always-on SSB) respectively were agreed at RAN1 #116-bis [3].

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

On-demand SSB may be cell defining (i.e., SSB with associated SIB1) or non-cell defining (i.e., SSB without associated SIB1).

At RAN1#116-bis, the following agreement was made for always-on SSB (Case #2) and no always-on SSB (Case #1).

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, down select between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

At RAN1#118, the following agreement was made for always-on SSB (Case #2) and no always-on SSB (Case #1) [5] with non-cell-defining SSB that is not on synchronization raster.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

However, RAN1 could not conclude if cell-defining on-demand SSB on sync-raster is supported.

The WID has a note that “On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation.”

Furthermore, on-demand SSB in Case #1 (no always-on) is expected to be transmitted in a bursty or semi-persistent manner over a short period. The bursty or semi-persistent cell-defining SSB is not useful for both legacy UEs’ and Release 19 UEs’ operations, at RRC Idle or Inactive state, which rely on periodic cell-defining SSB.

On the other hand, non-cell defining on-demand SSB has no impact to both legacy UEs’ and Release 19 UEs’ operations at RRC Idle/Inactive state.

Observation 1. Cell-defining on-demand SSB for Case #1 (no always-on SSB) has negative impact to both legacy UEs’ and Release 19 UEs’ operations at RRC Idle or Inactive state.

Proposal 1. RAN2 may wait for RAN1’s further concludes before discussing on-demand SSB’s impact to UE operations at RRC Idle or Inactive state.

## Indication for On-demand SSB transmission

On-demand SSB is transmitted when it is triggered, e.g., for SCell time/frequency synchronization, L1/L3 measurements, SCell selection or activation, etc.

There are two options for triggering on-demand SSB transmissions: triggered by the network or triggered by a UE.

At RAN1#116, the option of network triggering was agreed [2].

Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE

Additionally, the following options were agreed at RAN1 #116-bis [3] for on-demand SSB signaling from gNB.

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. FFS: Details on On-demand SSB transmission indication

Furthermore, the following agreement on gNB’s indication of on-demand SSB transmission was agreed at RAN1 #117 [4].

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. FFS: Scenarios where the above signalings are applicable

Figure 1: On-demand SSB Cases and Scenarios

As shown above in Figure 1, to support on-demand SSB, UE may expect on-demand SSB transmissions at least for the following on-demand SSB transmission scenarios for both on-demand SSB cases.

- On-demand SSB transmission Scenarios 2: before UE receives SCell activation command.

- On-demand SSB transmission Scenario 2A: when UE receives SCell activation command.

At RAN2 #127, the following agreements were made for indication of on-demand SSB transmission [6].

Scenarios on OD-SSB: 1. RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: 2. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 3. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

However, the signaling optimization for scenario 2A is still FFS.

To support on-demand SSB transmission Scenario 2A for both on-demand SSB Case #1 and Case #2, a single signaling with both SCell activation/deactivation and on-demand SSB transmission indication (i.e., Option 2 in RAN1 #116bis agreement) is more reliable with simultaneous reception and same decoding of Scell activation/deactivation and on-demand SSB transmission indication. Furthermore, a single signaling is more efficient with only one MAC CE transmission.

Observation 2. Extending the current Scell activation MAC CE for OD-SSB transmission indication is more reliable for supporting SSB transmission Scenario 2A for both on-demand SSB Case #1 and Case #2.

Observation 3. Extending the current Scell activation MAC CE for OD-SSB transmission indication is more efficient for supporting SSB transmission Scenario 2A for both on-demand SSB Case #1 and Case #2.

Proposal 2. RAN2 discusses how to enhance the current Scell activation MAC CE for OD-SSB transmission indication.

## L3 measurement with on-demand SSB

At RAN2 #127, the following agreement was made for L3 measurement on on-demand SSB [6].

Measurement on OD-SSB: 4. Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). 5. For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. 6. RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. 7. RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

For Case 1 (no always-on SSB), L3 measurement may be event triggered, e.g., triggered by the indication of the on-demand SSB transmission.

Observation 4. For Case 1 (no always-on SSB), L3 measurement may be event triggered.

For Case 2 (always-on SSB), L3 measurement is already supported with SMTC configured per always-on SSB. In our view, the on-demand SSB with Case 2 is more for SCell time/frequency synchronization, L1 measurements and SCell activation, instead of L3 measurement.

Observation 5. For Case 2 (always-on SSB), L3 measurement is already supported, and no more optimization is needed.

Based on the observation, RAN2 may focus on L3 measurement for Case 1. For example, the event triggering of L3 measurement, as well as the triggered L3 measurement report.

Proposal 3. RAN2 discusses how to support L3 measurement and report for Case 1.

Proposal 4. RAN2 agrees that no more specification work is needed for Case 2.

# Conclusion

In this contribution, we discussed on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and potential RAN2 specification work and concluded with the following observations and proposals.

Use cases and scenarios for On-demand SSB

Observation 1. Cell-defining on-demand SSB for Case #1 (no always-on SSB) has negative impact to both legacy UEs’ and Release 19 UEs’ operations at RRC Idle or Inactive state.

Proposal 1. RAN2 may wait for RAN1’s further concludes before discussing on-demand SSB’s impact to UE operations at RRC Idle or Inactive state.

Indication for on-demand SSB transmission

Observation 2. Extending the current Scell activation MAC CE for OD-SSB transmission indication is more reliable for supporting SSB transmission Scenario 2A for both on-demand SSB Case #1 and Case #2.

Observation 3. Extending the current Scell activation MAC CE for OD-SSB transmission indication is more efficient for supporting SSB transmission Scenario 2A for both on-demand SSB Case #1 and Case #2.

Proposal 2. RAN2 discusses how to enhance the current Scell activation MAC CE for OD-SSB transmission indication.

L3 measurement with on-demand SSB

Observation 4. For Case 1 (no always-on SSB), L3 measurement may be event triggered.

Proposal 3. RAN2 discusses how to support L3 measurement and report for Case 1.

Observation 5. For Case 2 (always-on SSB), L3 measurement is already supported, and no more optimization is needed.

Proposal 4. RAN2 agrees that no more specification work is needed for Case 2.

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “Chair notes RAN1#116 eom0” RAN1 #116	, Athens, Greece, February 26th – March 1st, 2024.

[3] “Chair notes RAN1#116bis eom0” RAN1 #116bis	, Changsha, China, April 15th – 19th, 2024

[4] “Chair notes RAN1#117 eom0” RAN1 #117, Fukuoka, Japan, May 20th – 24th, 2024, 2024

[5] “Chair notes RAN1#118 eom0” RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

[6] R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127, Maastricht, NL, August 19th – 23rd, 2024