# 3GPP TSG RAN WG2 Meeting #127bis                          	   R2-2408786

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.5.3

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

At RAN #105, it was agreed that on-demand SIB1 is specified for Case #2. The updated objects are outlined with the follows [1].

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective. - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1. - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE. - RAN1 specification impact to support this feature should be minimized

Additionally, the following agreements were made at RAN2 #127 [6] for on on-demand SIB1.

Agreements on OD-SIB1 Scenarios: 1. No consensus for RAN1 case 3 in RAN2. OD-SIB1 Validity: 2. We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell). 3. Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement). WUS configuration of NES cell from NES cell: 4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures. MSG3 based OD-SIB1 REQ: 5. Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell). Access restriction for legacy UEs: 6. Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells. Impacts to RRC connected UEs: 7. RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell). OD-SIB1 SI conclusion: 8. RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this contribution, we further discuss design considerations for on-demand SIB1.

# Discussion

## Uplink WUS acquisition at NES cell

For on-demand SIB1, a UE needs to send UL WUS to trigger NES cell’s on-demand SIB1 transmission. Currently, it is agreed that a Cell-A may provide UL WUS configuration for a UE is camping on the Cell-A. However, if a UE is camping on a NES cell already, will the UE need to get UL WUS configuration from Cell A for cell reselection with another NES cell?

Figure 1 Example of a UE moving from a NES cell to another NES cell

As shown in Figure 1, if a UE is camping on a NES cell (NES Cell 1) and desire to select a best cell which is a NES cell (NES Cell 2), the UE needs to know a valid UL WUS configuration to trigger the on-demand SIB1 from candidate NES cell (NES Cell 2).

Observation 1. When a UE is camping on a NES cell, the UE needs the UL WUS configuration of a neighboring NES cell for selecting the neighboring NES cell.

Based on the current agreement, the UE has to go back to the Cell -A that the UE previously camped on or go to another Cell-A to acquire UL WUS configuration in order to trigger the on-demand SIB1 transmission from the candidate NES cell (NES2). Always returning to a Cell-A for UL WUS configuration is not efficient and may cause more power consumption for both network and UE.

Observation 2. When a UE is camping on a NES cell, turning to a Cell-A for the UL WUS configuration of a neighboring NES cell for selecting the neighboring cell may cause more power consumption.

Instead of turning to a Cell-A for UL WUS configuration, if a UE can obtain the UL WUS configuration of the candidate NES cell (NES Cell 2) from its NES cell (NES Cell 1), the UE will not need to turn to Cell-A for UL WUS configuration for the candidate NES cell. This may save time and energy for cell reselection with a NES cell.

Observation 3. UL WUS acquisition at a NES cell is beneficial for both network and UE power saving when a UE is camping on a NES cell and wants to select or enter another NES cell.

For a NES cell supporting its neighboring NES cells’ UL WUS configuration, the NES cell may need to communicate with its neighboring NES cells via

- Xn for inter-gNB or inter-CU messaging for UL WUS configuration,

- mid-haul for intra-gNB or intra-CU messaging for UL WUS configuration, or

- front-haul for intra-DU messaging for UL WUS configuration.

There may be traffic load concern on Xn if many NES cells communicate with each other on Xn for UL WUS configurations. But the number of NES cells under a gNB or CU or DU is limited, and thus the traffic load for communicating UL WUS configurations on mid-haul or front-haul may be ignorable, especially comparing the overhead for each UE to turn to a Cell-A for each cell reselection with a NES cell.

Observation 4. The traffic load for communicating UL WUS configurations on mid-haul or front-haul may be ignorable.

Additionally, when a UE leaves a NES cell that it is currently camping on, the UE may receive the latest UL WUS configuration for the NES cell’s on-demand SIB1 via the NES cell’s SIB or RRC release. Since the UL WUS configuration is not expected to change frequently, the UE may store the UL WUS for triggering on-demand SIB1 when the UE wants to reselect or reenter the NES cell later if its stored on-demand SIB1 is not valid.

Proposal 1. Support that a NES cell provides the UL WUS configurations of its neighboring NES cells via SI or RRC release at least for intra-gNB case. FFS: inter-gNB case.

## Uplink WUS configuration

The following agreements were made at RAN2 #125bis for the contents of UL WUS configuration [2].

Agreements on contents of UL WUS configuration: 1. RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1. 2. UL WUS configuration includes at least below information: • RACH configuration

More agreements were made at RAN2 #126 for UL WUS configuration [3].

Agreements on on-demand SIB1: Further clarification 1. Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells: - More than one Cell A may provide configuration for the same NES cell. - The same Cell A may assist more than one NES Cells. 2. RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage Information of WUS configuration 3. Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell. 4. For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline. Transmission of WUS configuration 5. Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Additionally, the following agreement was made at RAN2 #127 for UL WUS update indication from a NES cell [4]. This implies that a NES cell may contain its UL WUS configuration in an SI.

WUS configuration of NES cell from NES cell: 4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

To limit the impact to legacy UE and unify the UL WUS configuration design for both Cell A and NES cell, it’s a clean solution to specify a new SIB (e.g., SIBx) for UL WUS configuration at both a Cell-A and a NES cell.

With this approach, it is easier for a NES cell to transmit the SIBx containing UL WUS configuration on the Xn interface to a Cell-A.

Proposal 2. Support that a new SIB is specified for UL WUS configuration for both a Cell-A and a NES cell.

A Cell-A may receivd multiple UL WUS configurations from multiple NES cells respectively. The Cell A may include all the received UL WUS configurations in a new SIB (SIBx) which may include a list of NES cells and a list of RACH configurations each corresponding to a NES cell on the NES cell list. The RACH configuration may include the configuration of Message 1 (PRACH) and Message 2 (RAR) including the on-demand SIB1 monitoring. The Message 1 configuration may be baselined from the current on-demand SI request in SIB1 (i.e., IE SI-RequestConfig) which may include

- dedicated RACH Occasions (ROs) for on-demand SIB1 if supported, and

- SIB1 request resources.

Proposal 3. Support that a new SIB for UL WUS configurations may include at least the follows.

- A NES cell list

- A UL WUS configuration list corresponding to the NES cell list.

If dedicated PRACH is supported for transmitting an UL WUS (e.g., a dedicated preamble sequence or dedicated ROs), the configuration may be further optimized, e.g., including a common PRACH configuration that is common to the NES cells providing their UL WUS configurations to a Cell-A.

Observation 5. Further optimization to UL WUS configuration may be possible with dedicated PRACH for UL WUS transmission.

Proposal 4. RAN2 discusses whether dedicated PRACH is supported for UL WUS transmission.

# Conclusion

In this contribution, we discussed scenarios or options for on-demand SIB1 operation and concluded with the following observations and proposals.

Uplink WUS acquisition at NES cell

Observation 1. When a UE is camping on a NES cell, the UE needs the UL WUS configuration of a neighboring NES cell for selecting the neighboring NES cell.

Observation 2. When a UE is camping on a NES cell, turning to a Cell-A for the UL WUS configuration of a neighboring NES cell for selecting the neighboring cell may cause more power consumption.

Observation 3. UL WUS acquisition at a NES cell is beneficial for both network and UE power saving when a UE is camping on a NES cell and wants to select or enter another NES cell.

Observation 4. The traffic load for communicating UL WUS configurations on mid-haul or front-haul may be ignorable.

Proposal 1. Support that a NES cell provides the UL WUS configurations of its neighboring NES cells via SI or RRC release at least for intra-gNB case. FFS: inter-gNB case.

Uplink WUS configuration

Proposal 2. Support that a new SIB is specified for UL WUS configuration for both a Cell-A and a NES cell.

Proposal 3. Support that a new SIB for UL WUS configurations may include at least the follows.

- A NES cell list

- A UL WUS configuration list corresponding to the NES cell list.

Observation 5. Further optimization to UL WUS configuration may be possible with dedicated PRACH for UL WUS transmission.

Proposal 4. RAN2 discusses whether dedicated PRACH is supported for UL WUS transmission.

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “R2_125bis_SL_R19 NES_MOB_Session Notes (Kyeongin Jeong) _EoM”, RAN2 #125bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

[3] “R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong) _EoM”, RAN2 #126, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

[4] R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127, Maastricht, NL, August 19th – 23rd, 2024.