# 3GPP TSG RAN WG2 Meeting #127bis                          	   R2-2408795

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.12.2

Source:	Qualcomm Incorporated

Title:	Discussion on design considerations for MIMO Phase 5

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

In this contribution, we discuss potential RAN2 work for supporting Release 19 MIMO Phase 5.

# Discussion

## UE-initiated/event-driven beam management

For UE-initiated/event-driven beam management, the following subject was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

RAN1 has made progress and decisions on the following design subjects [2] [3].

Beam reporting modes - Mode A: dynamically scheduling UCI by gNB - Mode B: UCI in pre-configured resource(s) for second UL channel #### UL signaling content(s) of L1-RSRP report - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB Beam reporting trigger events - Event-1: Quality of the current beam is worse than a certain threshold. - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. RS measurement for the current beam for Event-2 - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. RS measurement for the new beam for Event-2 - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

With the above decissions, RAN1 has not concluded if the triggering event determination is handled at PHY or MAC layer.

Observation 1. RAN1 has not conclude whether PHY or MAC handles the event evaluation.

At RAN2 #127, the following agreements were made for LTM event evaluation [4].

Agreements on L1 MR event evaluation 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering.

Observation 2. It has been agreed that MAC handles the event evaluation and measurement report triggering for LTM.

There are many commonalities between UE initiated beam reporting and LTM, e.g., based on L1 RSRP measurement. It would be beneficial to have a unified framework for event evaluation based on L1 measurements, e.g., MAC handles the event evaluation for triggering UE initiated beam reporting.

Observation 3. It is beneficial to have a unified framework with LTM for event evaluation based on L1 measurements.

Proposal 1. RAN2 discusses how MAC handles the event evaluation for triggering the UE initiated beam measurement report.

## CSI enhancement with 128 ports

For CSI enhancement with up to 128 CSI-RS ports, the updated subject (addition of SRS port grouping) was outlined in the WID of NR MIMO Phase 5” [1].

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook. - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

RAN1 has made progress with most design decisions impacting RAN1’s specifications [2] [3]. RAN2 may wait for RAN1’s further progress, especially the decisions on RRC parameters.

Proposal 2. RAN2 waits for RAN1's progress on supporting 128 ports.

## Enhancement for CJT

For UE reporting enhancement for CJT with non-ideal synchronization and backhaul, the following subject was outlined in the WID of NR MIMO Phase 5” [1].

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

RAN1 has made progress with most design decisions impacting RAN1’s specifications [2] [3]. RAN2 may wait for RAN1’s further progress.

Proposal 3. RAN2 waits for RAN1's progress on supporting enhanced CJT with non-ideal back-haul.

## Three antenna ports

For non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, the updated subject was outlined in the WID of NR MIMO Phase 5” [1].

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

RAN1 has made progress with most design decisions impacting RAN1’s specifications [2] [3]. RAN2 may wait for RAN1’s further progress, especially the decisions on RRC parameters.

Proposal 4. RAN2 waits for RAN1's progress on supporting non-coherent UL codebook to facilitate 3-antenna-port.

## Asymmetric DL sTRP/UL mTRP

For asymmetric DL sTRP/UL mTRP deployment scenarios, the updated subject (addition of two TA) was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

As shown in Figure 1, UEs may have both DL and UL with the macro cell sTRP but only UL with the micro cell mTRP.

UL transmissions from different UEs to the UL only mTRP need to be aligned at the UL only mTRP receiver, thus timing advance (TA) for UL mTRP is added to the WI at RAN #105 [1].

Figure 1. Asymmetric DL sTRP/UL mTRP deployment scenarios

Due to the limitation of the UL only mTRP, the schemes of two TAs of Rel-18 multi-DCI-based multi-TRP cannot be applied directly for asymmetric DL sTRP and UL mTRP.

For example, since there is no DL from UL only mTRP, the reference for a TA used for an UL only mTRP may be also based on the DL of sTRP.

Observation 4. The TA used for an UL only mTRP may be referenced from the DL of sTRP.

For another example, if RACH procedure is used for a UE to obtain a TA for an UL only mTRP, the RAR with TA may also be transmitted on the DL of sTRP.

Observation 5. With RACH procedure for TA, the RAR with TA may be transmitted on the DL of sTRP.

RAN2 may start to discuss how RACH procedure is used for asymmetric DL sTRP and UL mTRP so that a UE can obtain a TA for an UL only mTRP.

Proposal 5. RAN2 discusses how RACH procedure can be used for a UE to obtain a TA for an UL only mTRP.

# Conclusion

In this contribution, we discussed potential RAN2 work for supporting Release 19 MIMO Phase 5 and concluded with the following observations and proposals.

UE-initiated/event-driven beam management

Observation 1. RAN1 has not conclude whether PHY or MAC handles the event evaluation.

Observation 2. It has been agreed that MAC handles the event evaluation and measurement report triggering for LTM.

Observation 3. It is beneficial to have a unified framework with LTM for event evaluation based on L1 measurements.

Proposal 1. RAN2 discusses how MAC handles the event evaluation for triggering the UE initiated beam measurement report.

CSI enhancement with 128 ports

Proposal 2. RAN2 waits for RAN1's progress on supporting 128 ports.

Enhancement for CJT

Proposal 3. RAN2 waits for RAN1's progress on supporting enhanced CJT with non-ideal back-haul.

Three antenna ports

Proposal 4. RAN2 waits for RAN1's progress on supporting non-coherent UL codebook to facilitate 3-antenna-port.

Asymmetric DL sTRP/UL mTRP

Observation 4. The TA used for an UL only mTRP may be referenced from the DL of sTRP.

Observation 5. With RACH procedure for TA, the RAR with TA may be transmitted on the DL of sTRP.

Proposal 5. RAN2 discusses how RACH procedure can be used for a UE to obtain a TA for an UL only mTRP.

# References

[1] RP-242394, “WID revision: NR MIMO Phase 5”, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[2] RP-241775, Status Report of NR MIMO Phase 5, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[3] RP-241051, Status Report of NR MIMO Phase 5, RAN Meeting #104, Shanghai, China, June 17-20, 2024.

[4] R2-2407571, “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, RAN2 Meeting #127, Maastricht, Netherlands, Aug 19th – 23rd, 2024.