3GPP TSG-RAN WG2 #127bis	R2-2408825

Hefei, China, 14th – 18th October 2024

Agenda Item:	8.3.3

Source:	Ericsson

Title:	AI based measurement events prediction: Use cases, and simulations

Document for:	Discussion

# 1	Introduction

In RAN2#125bis meeting, the following agreements have been taken regarding the AI/ML based measurement events prediction:

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

In this contribution, we discuss further aspects of the indirect measurement event prediction, where the RRM measurement predictions in time, space and frequency domains are used to predict mobility related events.

# 2	Discussion

This contribution is twofold. In the following section we discuss the general use cases and conceptual aspects of mobility event predictions. In addition, we discuss which events to consider for the study item. Later we provide our view on how to proceed with the simulation and evaluation for the provided use cases.

## 2.1 Use case discussion

Mobility events and measurement reports are in principle designed to provide measurement reports to the network which the network can use when taking handover decisions based on the specific event triggering the measurement report.

In general, each mobility related event in 3GPP is designed for specific purposes. For example, a report triggered by event A1 may enable the network to relax the UE from performing measurements while a report triggered by event A2 can be an indication that UE needs to start more measurements for a better serving cell in neighbouring frequencies. In a typical use case, event A3 is used to ensure that the UE stays in the best cell in the serving frequency. For example, absence of an A3 based measurement report (i.e., no better cell in serving frequency) and reception of a measurement report triggered by event A2 (indicating poor quality in serving cell) indicates the necessity to initiate the inter-frequency measurements and reporting.

In the light of the above and given that the time and frequency domain RRM measurement predictions are prioritised in the study, we think at least event A3 and A5 should be considered for the evaluation of event predictions and RAN2 should use at least temporal and frequency domain predictions (and spatial prediction if time allows) for the event prediction.

The network normally uses A3 and A5 events as input to L3 mobility decisions.

A3 event is typically used for intra-frequency handover decisions, while A5 event is typically used for inter-frequency handover decisions.

Temporal, frequency (and spatial) domain RRM predictions can be used by the UE to evaluate predicted A3 and A5 events.

### 2.2.1 Predictions concerning A3 event.

In general, we think temporal and spatial domain predictions can be used by the UE in predicting an A3 event. We describe our view in the following.

#### 2.2.1.1 Temporal prediction in evaluating/predicting A3 event

Event A3 is defined in the RRC TS 38.331 according to the following, wherein the event is fulfilled if all the measurements in the (TTT) period fulfil the entry condition (Inequality A3-1) of the A3 event.

#### 5.5.4.4 Event A3 (Neighbour becomes offset better than SpCell) The UE shall: 1> consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled; 1> consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled; 1> use the SpCell for Mp, Ofp and Ocp. NOTE 1: The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR. Inequality A3-1 (Entering condition) Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off Inequality A3-2 (Leaving condition) #### Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

Mn and Mp represent the measurements of a neighbouring cell and an SpCell respectively at the present time instance (t0).

Therefore, prediction of the A3 event can be seen as evaluating the A3 entry condition based on the predicted RSRP value in the future time instance (temporal RSRP prediction), instead of the actual measurements. We describe the details in the following.

#### 5.5.4.4 Predicted Event A3 (Neighbour becomes offset better than SpCell in the future time instance t) The UE shall: 1> consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled; 1> consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled; 1> use the SpCell for Mp, Ofp and Ocp. NOTE 1: The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR. Inequality A3-1 (Entering condition) Mn[t] + Ofn + Ocn – Hys > Mp[t] + Ofp + Ocp + Off Inequality A3-2 (Leaving condition) #### Mn[t] + Ofn + Ocn + Hys < Mp[t] + Ofp + Ocp + Off

[t] denotes the prediction step the UE should take into account when perform the temporal RRM predictions and evaluate the A3 entry conditions and Mn[t] and Mn[t] denote the temporal prediction of the neighbouring and serving cells for the future time instance t, respectively. Once the evaluation of the A3 entry condition based on the temporal domain predictions of the serving and neighbouring cell indicates fulfilment of the predicted A3 event for a time period of Time-To-Trigger (TTT), the UE can trigger a report to the network including the available measurements and predictions. This is schematically shown in the Figure 1.

Figure 1. Using temporal RRM measurement prediction in predicting A3 event, in a future time instance (i.e., monitoring fulfilment of the A3 entering condition in the predicted A3 event).

According to the analysis above, it is proposed to reuse the temporal event prediction with the purpose of mobility event predictions.

Temporal RRM predictions (of serving and neighbouring cells) are reused for the A3 event predictions, i.e., the entry condition of the A3 event is evaluated based on the temporal RRM predictions of the serving and neighbouring cells.

#### 2.2.1.2 Spatial RRM predictions in evaluating/predicting A3 event.

Since event A3 is typically used for intra-frequency handover procedure, the A3 entering and leaving conditions can be also fed by spatial predictions if such spatial predictions are enabled and available at the UE. By using the measurements of some cells, the UE can predict the quality of other neighbour cells, via spatial prediction (i.e., in the same frequency and at the current time). Then the UE uses the predicted neighbour cell measurements in the entry condition (e.g. Inequality A3-1) and if all the predicted measurements fulfil the entry condition in the TTT period, the predicted event A3 is considered fulfilled.

Therefore, the same A3 entry and leaving conditions are used, but the spatial predicted neighbour cell quality is used, instead of the actual measurements, as shown below.

#### 5.5.4.4 Predicted Event A3 (Predicted Neighbour becomes offset better than SpCell) The UE shall: 1> consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled; 1> consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled; 1> use the SpCell for Mp, Ofp and Ocp. NOTE 1: The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR. Inequality A3-1 (Entering condition) Mn[s] + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off Inequality A3-2 (Leaving condition) #### Mn[s] + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

Here, Mn[s] indicates the predicted measurement quality of the neighbouring cell using the spatial prediction.

In the second part of the inequalities, Mp (i.e. the measurement result of the SpCell) is the actual measurement (not a prediction). Although it would be possible for the UE to use predictions for the serving cell Mp, this is not recommended: it is not costly (in terms of usage of resources) for the UE to perform measurements of the serving cell, so it is preferable to use the actual measurements instead of the predictions, which would reduce the accuracy of the event. Using spatial domain prediction in A3 event evaluation is a valid use case for evaluation, although it is not an event prediction as such i.e., in future time instance. Therefore, we propose the following.

Spatial RRM prediction of neighbouring cell is reused in predicting A3 event, i.e., the entry condition of the A3 event is evaluated based on spatial RRM predictions of the neighbouring cells.

However, given that the spatial domain prediction is not yet studied in RAN2, the use of spatial domain prediction in mobility event prediction can be postponed to the time when evaluation is done with results reflecting the required accuracy for such predictions.

### 2.2.2 Predictions concerning A5 event

#### 2.2.2.1 Temporal RRM predictions in evaluating/predicting A5 event

Similar to the A3 event we believe the time domain RRM prediction can be used by the UE to evaluate the A5 event predictions in a future time instance. More precisely, the UE can perform measurements on the serving and the configured neighbouring frequency and then use the available RRM measurements associated with the serving and neighbouring cell (in the neighbouring frequency) to predict the temporal RRM measurements in the future time instances. The way the UE can evaluate/predict an A5 event using the temporal predictions available in different frequencies is shown in the following.

#### 5.5.4.6 Event A5 (SpCell becomes worse than threshold1 and predicted neighbour becomes better than threshold2) The UE shall: 1> consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled; 1> consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled; 1> use the SpCell for Mp. NOTE 1: The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell. Inequality A5-1 (Entering condition 1) Mp[t] + Hys < Thresh1 Inequality A5-2 (Entering condition 2) Mn[t] + Ofn + Ocn – Hys > Thresh2 Inequality A5-3 (Leaving condition 1) Mp[t] – Hys > Thresh1 Inequality A5-4 (Leaving condition 2) #### Mn[t] + Ofn + Ocn + Hys < Thresh2

Here, Mn[t] and Mp[t] depicts the predicted measurement result of the neighbouring cell using the time domain for the neighbouring and serving cells, respectively.

Temporal RRM measurement predictions (inferred based on the available RRM measurements) for the serving and neighbouring cells can be reused by the UE in evaluating/predicting A5 event.

#### 2.2.2.1 Frequency RRM predictions in evaluating/predicting A5 event

Since the event A5 is typically used for inter-frequency handovers, it is reasonable to use the frequency predictions of neighbour cells quality to predict the event A5.
By measuring of some cells, the UE can predict the quality of neighbour cells in a different frequency, at the current time instance. Then the UE uses the predicted neighbour cell measurements to evaluate the entry condition (Inequality A5-2) and if all the predicted measurements fulfil the entry condition in the TTT period, the predicted event A5 is considered fulfilled and a report can be sent to the network.

#### 5.5.4.6 Event A5 (SpCell becomes worse than threshold1 and predicted neighbour becomes better than threshold2) The UE shall: 1> consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled; 1> consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled; 1> use the SpCell for Mp. NOTE 1: The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell. Inequality A5-1 (Entering condition 1) Mp + Hys < Thresh1 Inequality A5-2 (Entering condition 2) Mn[f] + Ofn + Ocn – Hys > Thresh2 Inequality A5-3 (Leaving condition 1) Mp – Hys > Thresh1 Inequality A5-4 (Leaving condition 2) #### Mn[f] + Ofn + Ocn + Hys < Thresh2

Here, Mn[f] indicates the predicted measurement result of the neighbouring cell using the frequency prediction. Mp (i.e. the measurement result of the SpCell) in the Inequality A5-1 and A5-4 is the actual measurement (not a prediction) of the SpCell.

Frequency domain RRM predictions for neighbouring cells can be reused by the UE in evaluating/predicting A5 event.

## 2.2 Simulation and evaluation

In the previous section we discuss the need for the simulation and evaluation of  RRM measurement predictions (in time, space and frequency domain) when used for the mobility event prediction (so-called indirect event prediction). We note that, one of the main goals for the study is to enhance the HO performance and the temporal domain prediction Case A is designed to evaluate the time domain prediction for the sake of HO performance improvement. In our understanding similar to the time domain prediction Case A, mobility event predictions targets to enhance the handover performance e.g., by preparing the HO in advance or by selecting a target cell for which the prediction shows a good post-handover performance. Therefore, we believe temporal prediction case A and the mobility event predictions follows the same goal i.e., improving handover performance.

Temporal prediction Case A and the mobility event predictions essentially follow the same goal i.e., improving handover performance.

Concerning the temporal prediction Case A, the preliminary simulation results show promising level of accuracy for the RRM measurement predictions. We note that the provided results are obtained based on very simple models (LSTM) and without leveraging advanced models e.g., Convolutional Neural Network or using any assistance information e.g., UE location information or speed etc.

Preliminary evaluation of the RRM measurement temporal predictions Case A (without using any advanced model e.g., CNN or assistance information such as UE location or UE speed, direction etc.) shows good accuracy level.

In the Table 1, some example results of time domain prediction for the Case A (wherein the goal of the study is to enhance the HO performance) is shown while more comprehensive results are provided in the [R2-2408747]. As shown in most scenarios the prediction error is around 1 dB for prediction window up to 400 ms.

Table 1. Case 2 – Case A: RSRP difference (prediction error) for Cell-level measurement prediction from cell level measurement results.

|  | RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prediction step (ms) | 80 | 160 | 240 | 320 | 400 |
| UE moving speed 60km/h | 0.217 | 0.467 | 0.767 | 1.139 | 1.497 |
| UE moving speed 120km/h | 0.269 | 0.571 | 0.971 | 1.356 | 1.732 |

Provided that the preliminary evaluation of the time domain prediction Case A (using very models and without any assistance information) already shows good performance (less than 1 dB error) we think the simulation setting for the temporal prediction case A can be used for the event predictions.

RAN2 consider the scheme of temporal domain prediction Case A, for evaluating A3 and A5 event predictions.

We think RAN2 should work on the observation and prediction windows (or prediction steps) needed for the mobility event prediction. For example, RAN2 should discuss what prediction window/step is needed in order to predict an A3 event fairly in advance.

RAN2 discuss and agree the observation window size and prediction window/step size of the temporal prediction Case A to use such predictions for the mobility event prediction.

As discussed above the temporal prediction Case A can be used to evaluate whether e.g., an A3 (or an A5) event entry condition remains fulfilled during the time period in which the timer TTT will be running. This example and how A3 event prediction can be mapped to the temporal prediction Cases A is shown in the Figure 2.

Figure 2. A3 event prediction using the temporal RRM predictions. Observation and prediction window should be discussed and agreed. Prediction step needs to be discussed.

Therefore, we propose RAN2 consider the TTT time window as prediction window wherein the UE predicts multiple instances of the RSRP value across the prediction time window.

For the A3 (or A5) event prediction evaluation using temporal prediction Case A, RAN2 consider at least TTT values as prediction window.

Beside the prediction window, the observation window should be discussed and agreed in RAN2. As starting point the observation window of the temporal RRM measurement Case A can be considered and based on the preliminary results we can discuss other observation window sizes (number of input samples) if needed.

For the A3 (or A5) event prediction evaluation, as staring point, RAN2 reuses the agreed observation window for evaluation of the Temporal prediction Case A.

In addition to the prediction and observation window, RAN2 can discuss the prediction step i.e., how long into the future the UE should perform the predictions.

For the A3 (or A5) event prediction evaluation, RAN2 discuss the prediction step i.e., how long into the future the UE should perform the predictions.

Moreover, we think RAN2 should discuss what assistance information is needed to enhance the performance of the predictions. Some assistance information such as UE speed, UE location can be used at start point.

RAN2 discuss the assistance information to enhance the prediction accuracy used for the mobility event prediction. UE speed and location can be used as starting point.

In the previous section we discussed that spatial and frequency domain predictions can be used to evaluate the mobility events (e.g., spatial prediction in A3 event evaluation and frequency domain prediction in A5 event prediction). However, we think the same evaluation used for the RRM measurement prediction is sufficient for RAN2 to decide whether to enable the UE to use such predictions in the mobility event prediction in the WI phase. Therefore, we have the following proposal.

The evaluation of the frequency (and spatial) RRM prediction (what is already agreed) is sufficient for RAN2 to decide whether to use such predictions in the mobility event prediction or not. i.e., if frequency/spatial prediction shows good accuracy, RAN2 proceed with enabling them in mobility event prediction. No more evaluation is needed.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The network normally uses A3 and A5 events as input to L3 mobility decisions.

Observation 2	A3 event is typically used for intra-frequency handover decisions, while A5 event is typically used for inter-frequency handover decisions.

Observation 3	Temporal prediction Case A and the mobility event predictions essentially follow the same goal i.e., improving handover performance.

Observation 4	Preliminary evaluation of the RRM measurement temporal predictions Case A (without using any advanced model e.g., CNN or assistance information such as UE location or UE speed, direction etc.) shows good accuracy level.

Based on the discussion in this document we propose the following:

Proposal 1	Temporal, frequency (and spatial) domain RRM predictions can be used by the UE to evaluate predicted A3 and A5 events.

Proposal 2	Temporal RRM predictions (of serving and neighbouring cells) are reused for the A3 event predictions, i.e., the entry condition of the A3 event is evaluated based on the temporal RRM predictions of the serving and neighbouring cells.

Proposal 3	Spatial RRM prediction of neighbouring cell is reused in predicting A3 event, i.e., the entry condition of the A3 event is evaluated based on spatial RRM predictions of the neighbouring cells.

Proposal 4	Temporal RRM measurement predictions (inferred based on the available RRM measurements) for the serving and neighbouring cells can be reused by the UE in evaluating/predicting A5 event.

Proposal 5	Frequency domain RRM predictions for neighbouring cells can be reused by the UE in evaluating/predicting A5 event.

Proposal 6	RAN2 consider the scheme of temporal domain prediction Case A, for evaluating A3 and A5 event predictions.

Proposal 7	RAN2 discuss and agree the observation window size and prediction window/step size of the temporal prediction Case A to use such predictions for the mobility event prediction.

Proposal 8	For the A3 (or A5) event prediction evaluation using temporal prediction Case A, RAN2 consider at least TTT values as prediction window.

Proposal 9	For the A3 (or A5) event prediction evaluation, as staring point, RAN2 reuses the agreed observation window for evaluation of the Temporal prediction Case A.

Proposal 10	For the A3 (or A5) event prediction evaluation, RAN2 discuss the prediction step i.e., how long into the future the UE should perform the predictions.

Proposal 11	RAN2 discuss the assistance information to enhance the prediction accuracy used for the mobility event prediction. UE speed and location can be used as starting point.

Proposal 12	The evaluation of the frequency (and spatial) RRM prediction (what is already agreed) is sufficient for RAN2 to decide whether to use such predictions in the mobility event prediction or not. i.e., if frequency/spatial prediction shows good accuracy, RAN2 proceed with enabling them in mobility event prediction. No more evaluation is needed.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR