3GPP TSG RAN WG2 Meeting #127bis	R2-2408826

Hefei, China, October 14 – 18, 2024

Source:	Inmarsat, Viasat

Title:	Discussion on Emergency Broadcast for NB-IoT

Agenda Item:	8.9.4

Document for:	Discussion

# Introduction

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

RAN#105 approved the extension of IoT_NTN_Ph3 scope to introduce support for Emergency Broadcast for NB-IoT NTN, reusing LTE PWS mechanisms:

In this contribution, we discuss the specification aspects.

# Discussion

The first consideration to be made is the target of applicability for this feature.  In the past 2 years, a significant application of NB-IoT NTN has been for support of direct-to-device applications to smartphones and other human carried devices or personal communications devices. With a number of new smartphones and dongles (e.g. Bluetooth dongles) supporting NB-IoT NTN for personal out-of-coverage and emergency messaging and SOS/distress.

For these users, PWS is of critical importance.

Observation 1: PWS support for NB-IoT NTN should be understood as primarily intended for a human-carried device such as a smartphone, dongle, wearable or other handheld.

The PWS architecture is a well-established system that has been supported for LTE since Rel-8/9.

The following picture outlines the overall PWS procedure for ETWS.  The procedure involves delivery of PWS messages via dedicated System Information Blocks (SIB) - e.g. SIB10 and SIB11 in the case of ETWS – and the triggering of UE re-acquisition of System Information via a special paging indication.

Currently, these mechanisms are supported for all LTE UE types except NB-IoT.

In order to support this capability for NB-IoT NTN, a few changes are required as a minimum:

- Introduction of necessary procedures for emergency cell broadcast

- Paging indications (e.g. etws-Indication, cmas-Indication-r9)

- SIB1 scheduling information of SIB10, 11 and 12

- Introduction of NB-IoT support for SIBs 10, 11 and 12 as new SIB-NB types, e.g.:

  - SystemInformationBlockType10-NB and SystemInformationBlockType11-NB for ETWS

  - SystemInformationBlockType12-NB for CMAS

It should be noted that, due to the reduced bandwidth of the NB-IoT NTN network, we may need to consider the procedure in order to make sure that the UE will be notified in time by paging, in order to be able to read all the SIB contents.

Proposal 1: Specify the necessary changes to support PWS for NB-IoT, such as, as a minimum:

- Introduction of necessary procedures for emergency cell broadcast

  - Note: the order of operations between Paging and SIB delivery may need to be considered

- Paging indications (e.g. etws-Indication, cmas-Indication-r9)

- SIB1 scheduling information of SIB10, 11 and 12

- Introduction of NB-IoT support for SIBs 10, 11 and 12 as new SIB-NB types, e.g.:

  - SystemInformationBlockType10-NB and SystemInformationBlockType11-NB for ETWS

  - SystemInformationBlockType12-NB for CMAS

In addition to the minimum required changes, there are a few considerations that apply both to NB-IoT in general, and to NB-IoT NTN more specifically.

First, the maximum SI/SIB size for NB-IoT is 680 bits. Given that the PWS message size can be larger, mechanisms to address this issue may need to be considered.  Some of the possible mechanisms are:

- Message segmentation in multiple SIBs

- Indications to the network to allow restricting the PWS message payload size

- PWS message compaction.

Observation 2: The NB-IoT maximum SIB/SI size is 680 bit. Segmentation may need to be considered.

In addition, NB-IoT DL throughput can be quite constrained due to challenging link budget. Given the criticality of PWS, the network will likely choose to deliver the SIBs at the lowest possible MCS and with repetitions, in order to maximize probability of reception.

This in turn might also mean that the time-to-delivery requirement for PWS (e.g. ETWS primary warning) currently specified (i.e. 4 seconds), might need to be relaxed.

The current TS 22.268 specification containing system requirements for PWS, includes the following note:

Note:	A bandwidth reduced low complexity UE or a UE supporting eDRX may not support all requirements for PWS, including ETWS, CMAS, EU-Alert and KPAS

Given the reduced capability and low-complexity nature of NB-IoT UE and radio access, we consider it to be falling into the above exception category.  Nonetheless, given the criticality of the service, RAN2 should strive to optimize the delivery mechanism in order to achieve a result as close as possible to compliance to the original requirement.

Observation 3: Whilst the PWS delivery time requirement could be relaxed for NB-IoT NTN, as per the following note in TS 22.268, RAN2 should still strive to achieve efficient and fast delivery, as close as possible to the original requirement:

Note:	A bandwidth reduced low complexity UE or a UE supporting eDRX may not support all requirements for PWS, including ETWS, CMAS, EU-Alert and KPAS

To address the aspects discussed above, RAN2 should consider the following:

Proposal 2: Consider the NB-IoT SI/SIB maximum size, as well as impact of time to reliably deliver the full PWS content in NB-IoT NTN with low MCS and repetitions, and identify possible solutions, such as (but not limited to):

- Message segmentation in multiple SIBs

- Indications to the network to allow restricting the PWS message payload size

- PWS message compaction.

In addition, the usage of eDRX, and in some cases PSM, is expected to be very common for NB-IoT UE, even those included in smartphones, wearables and other handhelds. Given the criticality of PWS, RAN2 should study the possibility of introducing mechanisms for UE to receive PWS messages even during eDRX, or if possible also PSM.

This could be achieved by reusing the Group Wake-Up Signal (WUS) mechanism, which could be used in conjunction with the paging indication in order to wake up the UEs from eDRX/PSM to receive the paging indication and subsequent PWS payload.

Proposal 3: Study support of reception of PWS during eDRX and, if possible, PSM, via group wake-up signal.

Another aspect that needs to be considered is the size of the NTN cell. NTN cells are expected to be significantly larger than TN cells.  The requirement of restricting the affected Notification Area, in order to make sure that only the affected users are notified of the warning, has already been considered a critical aspect of PWS in terrestrial networks. This problem is even more severe in NTN.

Furthermore, due to the large geographical areas and the high probability of uncertainty on the user’s location, targeting of groups of users rather than a geographical area can also in some cases be a more effective mechanism.

RAN2 should thus ensure that appropriate mechanisms are introduced in order to restrict the affected Notification Area or User Group for a PWS message.

Proposal 4: Introduce mechanisms to restrict the affected area and/or group of users for the PWS message.

Lastly, throughout the years, the security of PWS has been under increased scrutiny. In case of unsecured PWS, an attacker may gain the ability to send forged PWS messages, which may cause significant damage such as panic events or even injury or death.

RAN2 should send an LS to SA3 for them to consider the security aspects of PWS, including potentially mandating security.

Proposal 5: Send an LS to SA3 to consider security aspects of PWS and potentially mandating securing of PWS.

# Conclusions

Observation 1: PWS support for NB-IoT NTN should be understood as primarily intended for a human-carried device such as a smartphone, dongle, wearable or other handheld.

Proposal 1: Specify the necessary changes to support PWS for NB-IoT, such as, as a minimum:

- Introduction of necessary procedures for emergency cell broadcast

  - Note: the order of operations between Paging and SIB delivery may need to be considered

- Paging indications (e.g. etws-Indication, cmas-Indication-r9)

- SIB1 scheduling information of SIB10, 11 and 12

- Introduction of NB-IoT support for SIBs 10, 11 and 12 as new SIB-NB types, e.g.:

  - SystemInformationBlockType10-NB and SystemInformationBlockType11-NB for ETWS

  - SystemInformationBlockType12-NB for CMAS

Observation 2: The NB-IoT maximum SIB/SI size is 680 bit. Segmentation may need to be considered.

Observation 3: Whilst the PWS delivery time requirement could be relaxed for NB-IoT NTN, as per the following note in TS 22.268, RAN2 should still strive to achieve efficient and fast delivery, as close as possible to the original requirement:

Note:	A bandwidth reduced low complexity UE or a UE supporting eDRX may not support all requirements for PWS, including ETWS, CMAS, EU-Alert and KPAS

Proposal 2: Consider the NB-IoT SI/SIB maximum size, as well as impact of time to deliver the full PWS content in NB-IoT NTN, and consider at least the following possible solutions:

- Message segmentation in multiple SIBs

- Indications to the network to allow restricting the PWS message payload size

- PWS message compaction.

Proposal 3: Study support of reception of PWS during eDRX and, if possible, PSM, via group wake-up signal.

Proposal 4: Introduce mechanisms to restrict the affected area and/or group of users for the PWS message.

Proposal 5: Send an LS to SA3 to consider security aspects of PWS and potentially mandating securing of PWS.