3GPP TSG-RAN WG2 Meeting #127bis			        R2-2408828

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.4 (Netw_Energy_NR-enh-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on common signal and channel adaptation

Document for: 	Discussion and Decision

Introduction

In this contribution, we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

Discussion

- 2.1 Adaptation of PRACH in time domain

In RAN1#116bis meetings, it is agreed to define additional PRACH resource for NES-capable UEs, for PRACH adaptation in time domain. Further, in RAN1#118 meeting, it is agreed to use DCI for PRACH adaptation for NES-capable UE , e.g., to indicate activation/deactivation of NES PRACH resource.

[RAN1#116bis] Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode [RAN1#118] Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format. Agreement For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format.

Regarding configuration of additional PRACH resources, it should be provided via cell specific configuration (e.g., SIB1), given that additional PRACH resource can used by NES-capable UEs in RRC_IDLE/INACTIVE state. Since the legacy PRACH resource is provided via SIB1, it would be better to configure the additional PRACH resources for NES-capable UE by SIB1 as well.

Proposal 1. 	Additional PRACH resource for NES-capable UE is configured by SIB1.

In addition, according to the current specification, both 2-step RA and 4-step RA can be supported for legacy UE. In other words, if the additional PRACH resource for NES-capable UE is deactivated (e.g., by DCI) and legacy PRACH resource is configured for both 2-step RA and 4-step RA, NES-capable UE may perform 2-step RA based on the channel condition. Given that there is no critical reason to support PRACH adaptation mechanism only for 4-step RA resources, it would be beneficial to support additional PRACH resources for both 2-step RA and 4-step RA, in order to enable PRACH adaptation for both RA types.

Proposal 2. 	Support both 4-step RA and 2-step RA for PRACH adaptation in time domain.

- 2.2 Adaptation of paging occasions in the time domain

Regarding the adaptation of paging occasions, RAN2 made the following agreements so far:

RAN2#125bis:

1.	From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

2.	For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

3.	For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

RAN2#127:

1.	Option-a) is about the bundling of PF for R19 NES UEs.

2.	R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

In the e-mail discussion [POST127][109], the companies discussed and compared the impact on the specification and the NES performance between the following two options

Option-a) bundle paging frames.

Option-b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

Most companies agreed that there is no significant difference in performance between the two options, and that option b has significant less impact on specifications compared to option a. Therefore, it is easily assumed that the option b is preferred way to go.

Proposal 3	Support option b which extends the value of N to have increased interval between PFs and increases POs per PF.

It is also still FFS whether to allow legacy UEs to camp on the NES cell capable of adaptation of paging occasions. Option b achieves NES gain by reducing the number of total paging frames within a DRX cycle, resulting in some existing PFs where the NES cell does not transmit the paging. However, legacy UEs cannot identify which PFs are valid and which are not. Since UE can freely choose to monitor any one of the N-PFs within the DRX cycle, if it selects an invalid PF, it will miss the paging. Furthermore, such a bad choice could be repeated in the next DRX cycle and the following ones, which would severely degrade the paging reception performance of legacy UEs. Note that in NES WID, it is specified “there shall be no negative impact to legacy UEs, unless significant benefits are shown.”. Therefore, if we will go with option b, legacy UEs should not be allowed to camp on the cell capable of the adaptation of paging occasions.

Proposal 4	Network should not allow legacy UEs to camp on NES cell applying paging adaptation based on option b.

Conclusion

Regarding adaptation of PRACH in time domain

Proposal 1. 	Additional PRACH resource for NES-capable UE is configured by SIB1.

Proposal 2. 	Support both 4-step RA and 2-step RA for PRACH adaptation in time domain.

Regarding adaptation of paging occasions in the time domain

Proposal 3	Support option b which extends the value of N to have increased interval between PFs and increases POs per PF.

Proposal 4	Network should not allow legacy UEs to camp on NES cell applying paging adaptation based on option b.