3GPP TSG-RAN WG2 Meeting #127bis                                                             R2-2408829

Hefei, China, Oct 14th – 18rd, 2024

Agenda Item	:	8.2.2

Title	:	Discussion on functionality for ambient IoT

Source	:	Google Ireland Limited

Work Item	:	FS_Ambient_IoT_solutions

Document for	:	Discussion and Decision

# 1		Introduction

In this paper, we discuss how a reader obtains the device type of a paged device. We also discuss the lifetime of AS ID and propose that RAN2 discusses whether the lifetime of AS ID is one access frame or can be multiple access frames.

# 2	Discussion

In the RAN2 #127 meeting, RAN2 agreed that the following information is visible to reader from CN. Besides, RAN2 also discusses how the reader obtains the device type of a paged device.

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

RAN1 considers three device types given below. Devices of different types possess different capability for data processing and transmission. For example, device 1 does not support UL amplification and UL transmission is backscattered on carrier wave. This information helps reader allocate resources and communicate with devices.

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

In the RAN2 meeting, it seems there are different views on how to obtain the device type of a paged device. For example, there is a viewpoint that CN provides reader with a mapping between an application layer ID and a device type. Once device transmits an application layer ID to reader, the reader learns the device type from the mapping. It is assumed that the mapping is visible to reader from CN. The application layer should interact with MAC layer to figure out which device type for a paged device or MAC transmits application layer ID in MAC CE.

In other views, an access occasion or a random ID is associated with a device type. According to the device type, device selects a random ID or access occasion to transmit Msg1. In another view, device can just transmit the device type or a capability in Msg3. It is proposed that RAN2 discusses how a reader obtains the device type for a paged device.

Proposal 1:  RAN2 to discuss how a reader obtains the device type of a paged device.

In the RAN2 #127 meeting, RAN2 discussed the AS ID and most companies think AS ID is shorter than application layer ID. Meanwhile RAN1 also discussed the need for a device ID to receive R2D data or schedule D2R transmission.

In RFID, after reader transmits a Query command, there is only one access occasion following the Query command. Devices contend to transmit their random ID in the access occasion. At most one device will win the contention. If reader wants to transmit data to the winning device in the non-contention peirod, there is no need to explicitly appoint which device to receive the data.

Fig. 1: RFID channel access.

In ambient IoT, since there are more than one access occasion following the paging/R2D round trigger message, there may have more than one device winning the contention. After Msg2 transmission, if reader wants to transmit a command/data to one of the winning devices, the reader should address the command/data to the AS ID of the device. Similarly, if reader wants to schedule a D2R transmission for one of winning devices, reader should also address the resource allocation to the AS ID of the winning device.

Observation 1: AS ID is needed to distinguish different devices.

There are different views on how to obtain the AS ID. One of the options is that reader assigns an AS ID to a device. In other option, device reuses the random ID included in Msg1 as its AS ID. There are also discussion about how long the AS ID should be valid.

If AS ID is valid only in the access frame where the AS ID is obtained, both options work but it is simper to reuse the random ID. The terminology “access frame” refers to a contention period by zero or one non-contention period as illustrated in Fig. 2. If the AS ID is valid in more than one access frame, e.g. an access round, the AS ID should be assigned by a reader to prevent collision.

Fig. 2: Ambient IoT channel access.

After reader receives a Msg3, the reader may transmit a resource allocation to device to transmit the remaining data in buffer. In another case, the reader may transmit another command to device to perform some opearions. On the other hand, it should be discussed whether there is a case that the reader may transmit a command or resource allocation to a reader after several access frames. If there is such a case, the AS ID should be assigned by reader to prevent collision.

Proposal 2:  RAN2 to discuss whether the lifetime of AS ID is one access frame or can be multiple access frames.

# 3	Conclusions

In conclusion, we have the following proposals:

Proposal 1:  RAN2 to discuss how a reader obtains the device type of a paged device.

Observation 1: AS ID is needed to distinguish different devices.

Proposal 2:  RAN2 to discuss whether the lifetime of AS ID is one access frame or can be multiple access frames.