3GPP TSG-RAN WG2 Meeting #127bis	R2-2408832

Hefei, China, 14 – 18 October 2024

Agenda item:	8.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Support of PWS for NB-IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The R19 WID [1] for IoT NTN has been updated in RAN plenary#105 meeting to cover the objective for support of PWS (Public Warning System). In details, the support of PWS messages for NB-IoT is expected to reuse the LTE mechanisms. Furthermore, the support of geo-fencing function for ETWS will be considered based on NR NTN outcome.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide our view on how to support PWS for NB-IoT NTN.

# 2	Discussion

According to stage one description of the Public Warning System (PWS) Requirements [2], the PWS may cover the subsystem such as Earthquake and Tsunami Warning System (ETWS), the Commercial Mobile Alert System (CMAS), EU-ALERT, and Korean Public Alert System (KPAS). As indicated in [3], the E-UTRAN procedures defined for CMAS equally apply for EU-Alert and KPAS. Therefore, to support PWS for NB-IoT NTN, both the ETWS and CMAS defined for LTE should be supported.

Proposal 1: Both ETWS and CMAS mechanism defined in LTE should be reused to support PWS message broadcast for NB-IoT NTN.

As specified in LTE specifications, the ETWS warning notifications can either be a primary notification or secondary notification. The primary notifications are short notifications, whereas secondary notifications provide more detailed information. The ETWS primary notification is broadcast in SystemInformationBlockType10 (SIB-10) while the secondary notification is broadcast in SystemInformationBlockType11 (SIB-11). In contrast, the CMAS warning notifications, which are short text messages (CMAS alerts) are broadcasted through SystemInformationBlockType12 (SIB-12). To enable PWS functionality for NB-IoT NTN, a straight-forward approach would be to introduce analogous SIBs such as SIB10-NB, SIB11-NB, and SIB12-NB. For the specifics of these new NB-IoT SIBs and their acquisition procedures, the existing SIB10, SIB11, and SIB12 content and procedures can serve as baseline.

Proposal 2: RAN2 to introduce SIB10-NB , SIB11-NB and SIB12-NB for NB-IoT NTN, which mirror the roles of SIB10, SIB11, and SIB12 used for ETWS and CMAS in LTE.

The ETWS and CMAS notifications can occur at any point in time. In legacy, the Paging message is used to inform ETWS/CMAS-capable UEs in RRC_IDLE and UEs in RRC_CONNECTED (excluding those in Coverage Enhancement mode) about presence of one or more notifications. Furthermore, for RRC_CONNECTED UEs in CE mode, a Direct Indication information transmitted on MPDCCH using P-RNTI or SI-RNTI informs them about the presence of the notifications. The NW may set the etws-indication and cmas-indication bits in the Paging message or Direct Indication information to alert UEs to start receiving SIB1 and corresponding SIBs for ETWS and CMAS notifications. To enable PWS functionality for NB-IoT NTN, the similar mechanism should be introduced for Paging-NB message and Direct Indication Information transmitted on NPDCCH.

Proposal 3: RAN2 to introduce etws-indication and cmas-indication bits in Paging-NB message and Direct Indication information transmitted on NPDCCH.

As part of the PWS objective for IoT NTN, there is a Note to consider supporting indication of intended service area for ETWS based on NR NTN discussions/outcome.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In Rel-19 NR NTN, there are similar discussions for ETWS notifications regarding whether the geofencing for an area smaller than an NTN cell should be supported. As a result, RAN2 decided to send LS to SA1 [4] in RAN2-127 meeting and asked for feedback whether the capability to geofence emergency messages to an area should be supported for ETWS in an NTN cell, considering the delay requirements for ETWS primary notification. Therefore, for IoT NTN, we can wait for the conclusion from NR NTN first.

Proposal 4: Wait for NR NTN conclusion to decide whether to support indication of intended service area for ETWS for IoT NTN.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Both ETWS and CMAS mechanism defined in LTE should be reused to support PWS message broadcast for NB-IoT NTN.

Proposal 2: RAN2 to introduce SIB10-NB , SIB11-NB and SIB12-NB for NB-IoT NTN, which mirror the roles of SIB10, SIB11, and SIB12 used for ETWS and CMAS in LTE.

Proposal 3: RAN2 to introduce etws-indication and cmas-indication bits in Paging-NB message and Direct Indication information transmitted on NPDCCH.

Proposal 4: Wait for NR NTN conclusion to decide whether to support indication of intended service area for ETWS for IoT NTN.

# Reference

[1] RP- RP-242397 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] TS22.268 Public Warning System (PWS) requirements

[3] TS36.300 (E-UTRAN) Overall description Stage 2

[4] R2-2407823 LS on requirements for ETWS primary notification