3GPP TSG-RAN WG2 Meeting #127bis				R2-2408836

Heifei, China, 14 - 18 October, 2024

Title: 	Discussion on support of the Rel-18 leftovers

Source: 	Huawei, HiSilicon

Agenda item:	8.10.5

Document for:	Discussion/Decision

1   Introduction

The RAN2#127 meeting has achieved the following agreements:

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure.

Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

In this paper, we will discuss the leftover issues from Rel-18 SON/MDT.

2   Discussion

2.1 RACH optimization for SDT

RA report for SDT has been standardized in Rel-18, which is one means to detect the SDT related RACH resource issues. The Rel-19 WID allows the further development of RACH optimization for SDT.

RAN2 has agreed to introduce the downlink RSRP and buffered data volume at the time of deciding whether to perform SDT or not in order to optimize the condition parameters for triggering the SDT. Some company were also interested in the RSRP and data volume at the time of SDT failure. Between the time of SDT initiation and SDT failure, the RSRP and buffered data volume may change a lot, which is dynamic random variation. In our understanding, the RSRP value and data volume when SDT failure happens is not helpful for network to adjust the condition parameter for triggering SDT or not.

Proposal 1: It is proposed RAN2 to investigate the necessity of including RSRP threshold and data volume threshold when SDT failure happens.

Per current specification TS 38.331, it could be considered as SDT failure under the following circumstances:

- T319a expires; or

- random access problem indication received from MCG MAC while SDT procedure is ongoing; or

- indication received from MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing; or

- indication from the lower layers that the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; or

- integrity check failure indication received from lower layers while SDT procedure is ongoing; or

From our point of view, it is important to detect the random access issue, the RLC re-transmission issue and T319a setting issue, becase we think they should be the main reasons for leading SDT failures. Other failure type could be de-prioritized, as they are relatively less likely to happen. Therefore, it is suggested to indicate the failure causes for random access, RLC re-transmission and T319a in RA report.

Proposal 2: Enhance the following failure cause to RA report: random access, RLC re-transmission and T319a.

2.2 MRO for MR-DC SCG failure

Rel-17 SON/MDT WI developed the solution for MRO for NR-DC SCG failures according to TS 38.331 specification [2]. In Rel-17, SON/MDT WI considered primarily two SCG failure scenarios as follows:

- SCG RLF, and

- Legacy PSCell change or addition failure.

In Rel-19, we further discuss the MRO for SCG failure in other DC options.

EN-DC

For EN-DC scenario, the UE report the NR SCG failure information to LTE MN node, and then the LTM MN node may further forward the NR SCG failure information to NR SN node. The SCGFailureInformationNR message reported from UE to LTE node needs to be enhanced in TS 36.331 accordingly to align with the SCGFailureInformation defined in TS 38.331.

Table 1 Enhancement to align with the NR-DC SCG failure report in TS 38.331

| Parameter | IE/value & condition |
| --- | --- |
| failedPSCellId | Frequency and PCI |
| previousPSCellId | Frequency and PCI |
| timeSCGFailure | 0-1023 |
| perRAInfoList | Only can be included if failureType is set to synchReconfigFailureSCG, or is set to randomAccessProblem while T304 is running |

RAN2 has already agreed to include the former three items. We continued to discuss the necessity of adding the last item: perRAInfoList. If only the SCG failure is caused by random access problem or PSCell change failure, the RA info need to be logged by UE to reflect the RACH configuration issue to the network. We believe that, there is still huge interest in making proper RACH optimization for SCG mobility. Therefore, we propose to add the perRAInfoList to SCGFailureInformationNR message in TS 36.331.

Proposal 3: Add the perRAInfoList to SCGFailureInformationNR message in TS 36.331.

NE-DC

For NE-DC scenario, the UE report the LTE SCG failure information to NR MN node, and then the NR MN node may further forward the LTE SCG failure information to LTE SN node. Therefore, the SCGFailureInformationEUTRA message in TS 38.331 needs to be enhanced accordingly.

NGEN-DC

Similar as EN-DC case, the SCGFailureInformationNR message defined in TS 36.331 needs to be enhanced accordingly, as above discussed.

Proposal 4: The enhancement of SCGFailureInformationNR message for EN-DC applies also to NGEN-DC.

Proposal 5: For NE-DC, the enhancement to SCGFailureInformationEUTRA message in TS 38.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.

3   Conclusion

In this paper, we provide the following proposals:

RACH optimization for SDT:

Proposal 1: It is proposed RAN2 to investigate the necessity of including RSRP threshold and data volume threshold when SDT failure happens.

Proposal 2: Enhance the following failure cause to RA report: random access, RLC re-transmission and T319a.

MRO for MR-DC SCG failure:

Proposal 3: Add the perRAInfoList to SCGFailureInformationNR message in TS 36.331.

Proposal 4: The enhancement of SCGFailureInformationNR message for EN-DC applies also to NGEN-DC.

Proposal 3: For NE-DC, the enhancement to SCGFailureInformationEUTRA message in TS 38.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.