3GPP TSG-RAN WG2 Meeting #127bis   								   R2-2408837

Hefei, China, October 14 - 18, 2024

Agenda Item:	8.1.2.3

Source:	Huawei, HiSilicon

Title:	Discussion on LCM for positioning use case

Document for:	Discussion and decision

Introduction

At the RAN#102 meeting, a new R19 WI on AI for air interface [1] was approved and revised in the RAN#103 meeting [2]. One objective is specification support for positioning accuracy enhancements.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

Following the RAN2#127bis agenda, this paper is to provide RAN2 impacts analysis on LCM for positioning accuracy enhancements 1st priority use case.

#### 8.1.2.3 LCM for Positioning use case Contributions should focus on LCM for UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases. Aspects related to data collection should be covered in 8.1.3

Discussion

In the following sections, we will discuss the potential RAN2 impacts to support the LCM of the above positioning use case for both the UE-sided and NW-sided models, respectively.

For UE-sided model, Case 1 is analyzed.

For NW-sided models, Case 3b (LMF-sided model) and Case 3a (gNB-sided model) are analyzed.

UE-sided model for positioning

In the objective, there are two type of UE-sided model for positioning accuracy enhancements, as highlighted with yellow marker [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In the RAN2 #125b meeting, there are the following agreements for the UE-sided model.

Agreements: 1. Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available. FFS what functionality refers to. 2. Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. Agreements for positioning and beam management - Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message. - Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality 3 FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed) Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In the RAN2 #126 meeting, the following agreements are achieved:

Agreements

- RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b. (for NW-sided model)

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

Based on the latest RAN1 and RAN2 progress, we are to discuss possible RAN2 impacts for Case 1, such as UE capability reporting, functionality management, monitoring, positioning methods.

UE capability reporting for positioning

For legacy positioning mechanisms, UE capability reporting has been defined, and LMF may decide on a positioning method and send configurations to UE for initiating the positioning procedure.

For AI/ML positioning Case 1, whether/how the UE should indicate its capability to LMF depends on solution details.

For BM, RAN2 has discussed UE capability aspect and there are static capability and applicable functionality reporting as the applicability of functionality may dynamically change. Then, RAN2#127 discussed more details of applicable functionality reporting, e.g. how the UE should report this to gNB. Unlike BM, for legacy positioning mechanisms, the LPP message ProvideCapabilities can support both static and dynamic capability reporting (e.g. see the text from TS 37.355).

*******************************************

5.1.2	Capability Indication procedure

The Capability Indication procedure allows the target to provide unsolicited capabilities to the server and is shown in Figure 5.1.2-1.

Figure 5.1.2-1: LPP Capability Indication procedure

*******************************************

Observation 1: For positioning Case 1, the LPP messages RequestCapabilities/ProvideCapabilities messages can be reused to carry static and dynamic positioning capabilities.

Proposal 1: If dynamic positioning capabilities are to be supported, the LPP messages RequestCapabilities/ProvideCapabilities can also be reused to carry dynamic positioning capabilities.

For dynamic information reporting, at RAN2#127, RAN2 sent a LS to RAN1 for asking some questions about applicable functionality reporting for BM. For positioning, we think RAN1 is still discussing whether applicable functionality reporting is needed or not, and if so, what are the requirements. From RAN2 point of view, we can wait a bit for more RAN1 progress on applicable functionality reporting for positioning.

Proposal  2: For positioning Case 1, for applicable functionality reporting for positioning, RAN2 can wait for more RAN1 progress on the necessity and requirements.

For static capability aspect, normally it can be discussed once basic solutions are clear enough. In addition, it is also related to functionality identification.

In TR 38.843 [3], the definition of functionality identification is captured below:

Functionality identification: A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification. Where AI/ML functionality resides depends on the specific use cases and sub use cases.

For each use case, the standard definition of functionality is still in the RAN1 discussion. In general, we think the functionality can refer to a group of AI models, which are designed to realize an AI based function. In the positioning case, for example, the UE-sided functionality can refer to direct AI/ML positioning (case 1), or UE-assisted AI/ML positioning, from the sub-use case perspective.

Besides referring to a sub-use case, specific functionality may further describe other common information among a group of models, e.g. the input/output-related information. The combination of sub-use case and input/output can be regarded as the granularity to distinguish the supported functionalities. For the direct positioning, the input can be either CIR (Channel Impulse Response), PDP (Power Delay Profile), or DP (Delay Profile), which can respectively refer to different functionalities. For the assisted positioning, the output can be either LOS/NLOS identification, ToA (Time of Arrival), which also respectively refers to different functionalities.

In our view, the supported functionality indicates that the UE implements a specific functionality. However, due to the variable hardware and software conditions (i.e. UE side additional conditions), in a certain time, the UE may currently be able to execute only a subset of all the supported functionalities, which is defined as applicable functionality.

Observation 2: For positioning use cases, the UE-sided functionality can be categorized based on sub-use cases, input, output, etc.

Proposal 3: For the UE-sided model for positioning, it is proposed that RAN2 discuss the following two options for the supported functionality category:

(1) sub-use cases, e.g. Case 1

(2) a combination of sub-use cases and the input/output-related information

Functionality management

In TR 38.843 v18.0.0, section 7.2.4 has defined some requirements regarding functionality management:

-	Management:

o	For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE.

o	For gNB-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB.

o	The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE.

For UE-sided model, some actions may be performed by the UE, e.g. (de)activation, fallback. Currently, LPP message RequestLocationInformation is from LMF to UE, which is to request location information at UE. This LPP message can be used to carry functionality management related indications, e.g. whether the UE is allowed to do AIML based positioning.

The  existing LPP message RequestLocationInformation can be considered as the baseline for enabling functionality management. Details are pending for functionality definitions (including granularity), which can be further discussed.

Proposal 4: For positioning Case 1, for supporting functionality management, existing LPP messages RequestLocationInformation can be considered.

Monitoring aspect

For the UE-sided model positioning, in the RAN1 116b meeting, the following agreements have been achieved.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

We can observe that for the UE-sided model positioning case, the monitoring related information exchange is between UE and LMF. Meanwhile, the functionality management triggers and commands are also between UE and LMF. The UE and LMF communicate via LPP signaling. The main procedures should include the following.

- Report configurations from LMF to UE. According to the RAN1 116b meeting, in option A, it is the UE to calculate the monitoring metrics while in option B, it is the LMF to perform the calculation based on UE reported inference result. Whichever option is chosen, the LMF should configure the UE about how to acquire calculation assistance data (option A) or what inference results to report (option B).

- Report from UE to LMF. Based on the configuration from LMF, the UE reports to LMF via LPP signaling. The UE may be configured to report with trigger conditions or periodic. After receiving reports from UE, the LMF may further send another configuration to the UE, to modify the previous configuration or cancel the UE reporting.

For now, we see that RAN1 still needs more time to check different options, details and they may do down-selection of some options. So we think RAN2 can wait for more RAN1 progress on monitoring.

Proposal 5: For positioning Case 1, RAN2 can wait for more RAN1 progress on monitoring aspect.

About positioning methods

Since R16, some positioning methods have been introduced. We think that AI/ML based positioning is based on existing positioning method, and RAN1 should have the same assumption. For UE-sided model for positioning, the existing UE-based positioning methods can be considered, e.g. DL-TDoA and DL-AoD; for NW-sided model, the existing NW-based positioning methods can be considered, e.g. UL-TDoA and UL-AoA.

Proposal 6: For positioning Case 1, it is proposed to use existing UE-based positioning methods (e.g. DL-TDoA and DL-AoD) for supporting AIML-based positioning.

NW-sided model for positioning

In the objective, there are three types of NW-sided model for positioning accuracy enhancements, as highlighted with a yellow marker [2]. In this sub-section, we shall analyze the 1st priority cases, i.e. case 3a and case 3b.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In the last RAN2 125b meeting, the following agreements are achieved for LCM for NW-sided model of positioning.

Agreements: - RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training - For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference. FFS if further enhancements are needed - There is no specification impact associated to gNB-side model inference, depending on further RAN1 input. - FFS whether there is specification impact associated to gNB-side model monitoring. - For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs - For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

For Case 3a, gNB firstly performs NW-based positioning, and then provide assistance information to LMF for AI/ML based positioning. For functionality identification and functionality management, we think they can be left to implementation, so there should be no specification impacts. For Case 3b, we have the similar analysis as for Case 3a.

For the monitoring and inference, we quote the related conclusions from TR 38.843 [3] as the follows.

Model monitoring: - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - gNB at least for Case 3a (with gNB-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - For AI/ML based positioning, LMF for Case 2a (with UE-side model) and Case 3a (with gNB-side model) is identified as the entity to derive the monitoring metric at least when monitoring is based on provided ground-truth label (or its approximation). - Data for computing monitoring metric: - If monitoring based on model output: e.g., estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground-truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input: e.g., measurement corresponding to model inference input. - Assistance signalling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring. - Assistance signalling from UE/PRU for NW-side model monitoring. Model Inference related: - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signalling overhead - Potential new measurement: CIR/PDP - Existing measurement: e.g., RSRP/RSRPP/RSTD - Note: Details of potential new measurement and/or potential enhancement to existing measurement is to be studied. - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a): - Measurement report to carry model output to LMF - New measurement report: e.g., ToA, path phase - Existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - Enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - At least the following types of model inference output are identified as candidates providing performance benefits: - Timing estimation - Note: the report to LMF is derived based on and maybe different from the model inference output - LOS/NLOS indicator - Assistance signalling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations The specification impact related to the following items is assessed: - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signalling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signalling - Note: Whether such assistance signalling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed

For the monitoring metrics deriving, at least the gNB executes the deriving for case 3a, and LMF for case 3b. Further, in case 3a, the LMF will first require the gNB to derive the monitoring metrics. Then gNB will request the LMF to provide assistance signaling, e.g. the ground-truth labels. Based on the assistance signaling and model outputs, the gNB can derive the monitoring metrics and report to the LMF, while the LMF will decide the functionality management. In case 3b, the LMF will require gNB to report measurement results and then derive the monitoring metrics. Therefore, the monitoring for case 3a/3b only involves gNB and LMF, and the related procedures can be realized based on the NRPPa protocol.

Observation 3: For monitoring in cases 3a and 3b, we observe no RAN2 impacts for now, and RAN3 can discuss NRPPa impacts.

For the inference in case 3a, the gNB will provide model inference output, where new measurement reports and (enhancement of) existing measurement reports are included. For the inference in case 3b, the gNB will provide measurements as LMF-sided model inference input, where potential new measurements and existing measurements are included. The above-mentioned information provided from gNB to LMF can be carried by NRPPa protocol and bring no RAN2 impact.

We note that RAN1#116bis had some discussions on inference inputs for 3a/3b, e.g. SRS measurements. For RS configuration, whether existing RS configuration can be re-used or not is still under RAN1 discussions. So we think the only potential impact is that to acquire these measurements, whether a new RS configuration should be introduced or the current RS configuration should be enhanced.

Observation 4: For inference in cases 3a and 3b, we observe no RAN2 impacts for now, and RAN3 can discuss NRPPa impacts.

Conclusions

In this contribution, we share our views on LCM for Positioning use cases. Our observations and proposals are summarized as follows:

For UE-sided model for positioning

Observation 1: For positioning Case 1, the LPP messages RequestCapabilities/ProvideCapabilities messages can be reused to carry static and dynamic positioning capabilities.

Observation 2: For positioning use cases, the UE-sided functionality can be categorized based on sub-use cases, input, output, etc.

Proposal 1: If dynamic positioning capabilities are to be supported, the LPP messages RequestCapabilities/ProvideCapabilities can also be reused to carry dynamic positioning capabilities.

Proposal  2: For positioning Case 1, for applicable functionality reporting for positioning, RAN2 can wait for more RAN1 progress on the necessity and requirements.

Proposal 3: For the UE-sided model for positioning, it is proposed that RAN2 discuss the following two options for the supported functionality category:

(1) sub-use cases, e.g. Case 1

(2) a combination of sub-use cases and the input/output-related information

Proposal 4: For positioning Case 1, for supporting functionality management, existing LPP messages RequestLocationInformation can be considered.

Proposal 5: For positioning Case 1, RAN2 can wait for more RAN1 progress on monitoring aspect.

Proposal 6: For positioning Case 1, it is proposed to use existing UE-based positioning methods (e.g. DL-TDoA and DL-AoD) for supporting AIML-based positioning.

For NW-sided model for positioning

Observation 3: For monitoring in cases 3a and 3b, we observe no RAN2 impacts for now, and RAN3 can discuss NRPPa impacts.

Observation 4: For inference in cases 3a and 3b, we observe no RAN2 impacts for now, and RAN3 can discuss NRPPa impacts.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RP-240774, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- TR 38.843 v18.0.0