3GPP TSG-RAN WG2 Meeting #127bis   								   R2-2408839

Hefei, China, October 14 - 18, 2024

Agenda Item:	8.1.5

Source:	Huawei, HiSilicon

Title:	Discussion on model transfer and delivery

Document for:	Discussion and decision

Introduction

After Rel-18, RAN1 has agreed the following candidate cases for model transfer/delivery [1].

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

Currently [4], RAN1 is still discussing how to completely design the model transfer/delivery mechanism.

Agreement RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

Following the RAN2#127-bis agenda, this paper is to provide a comprehensive analysis on model transfer/delivery, based on both RAN1 and RAN2 progresses.

### 8.1.5 Model transfer/delivery Contributions should at least provide further analysis of the solutions captured in the TR (section 7.2.1.4 in the TR).

Discussion

As we shown in the introduction part, RAN1 has concluded that model transfer is mainly for CSI compression using two-sided model (if supported), and CSI compression will continue study until the end of Rel-19. So we can directly use this RAN1 conclusion for RAN2 discussions.

Observation 1: Model transfer/delivery is mainly for two-sided model. From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported).

In the following sections, we will discuss the potential RAN2 impacts based on latest RAN1 progress.

Analysis on model transfer/delivery candidate cases

In TR 38.843 [1], the model transfer/delivery is discussed based on the model training and storage side:

## 4.3 Collaboration levels … For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE. - For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. - Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage. - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

In general, as in the highlight parts above, we provide our analysis on the perspective the model is trained at NW side or the UE side/neutral site.

Model(s) trained at the NW side

When the model is trained at the NW side, case y, z2, z4 and z5 are agreed as the candidate options in Rel-18. Then according to the agreements in RAN1 #116 [2] and #116b meeting [3], the case z5 and z2 are deprioritized in Rel-19.

RAN1 #116: Conclusion: From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. RAN1 #116b: From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration

In RAN1 #118 meeting [4], RAN1 made more agreements regarding case z4. And we think that case y is still open.

Agreement From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information

Before starting the model transfer/delivery procedures, the NW and UE need to achieve an alignment about the model structure details as mentioned in RAN1 agreement. If this alignment is achieved via offline, considering the various software/hardware capabilities and different scenarios for model application, the NW may need to previously train a large number of models with different structures, in order to match the UE capabilities and scenarios. This will introduce significant burdens on NW vendors, e.g. calculation complexity and storage.

On the other hand, if the alignment can be achieved via 3GPP (i.e. specified structure can be defined in 3GPP by RAN1 or RAN4 so that there will be no offline alignment), there may be less NW burdens as NW may decide to train models under the limited structures. From RAN2 point of view, there may be impacts on signalling part as the signalling may be needed for model transfer that is non-transparent to 3GPP.

At RAN1#116 meeting, the following agreements were made. We observe that model transfer is also being discussed under the feature model identification (which is also required for more study until the end of Rel-19), and there are also alternatives. In this case, we do think that more RAN1 progress is needed, e.g. whether model transfer is necessary or not.

Agreement For model identification of UE-side or UE-part of two-sided models, further clarification is made as follows. - The following are example use cases Type B1 and B2 - Model identification in model transfer from NW to UE - Model identification with data collection related configuration(s) and/or indication(s) and/or dataset transfer - Note: Other example use cases are not precluded. - Note: Offline model identification may be applicable for some of the above example use cases

For now, we believe that the discussion on RAN1 has just begun, and more requirements and options need to be discussed. And then RAN2 could discuss potential RAN2 impacts (maybe impacts on other WGs).

Observation 2: For model transfer/delivery for two-sided model, RAN1 will only discuss case y and z4 for model(s) trained at the NW side.

Observation 3: For case z4, if alignment of the model structure is done offline, it brings significant burdens on NW vendors (i.e. via training of different model structures). On the other hand, if alignment of the model structure is defined in the specification, the burdens on NW vendor may be less as NW may decide to train models under the limited structures. The decision on this lies in RAN1 and more RAN1 inputs will be helpful for RAN2 discussions.

Model(s) trained at the UE side

When the model is trained at the UE side, case y, z1 and z3 are agreed as the candidate options in Rel-18. Then in RAN1 #116b meeting [3], case z3 is deprioritized.

RAN1 #116b: Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network

Therefore, we think that RAN1 will only discuss case y and z1. In our opinion, case z1 has similar issues as case z4 with model structure alignment via offline, e.g. significant burdens on NW vendors and extra RAN2 impacts.

Observation 4: For model transfer/delivery for two-sided model, RAN1 will only discuss case y and z1 for model(s) trained at the UE side.

Observation 5: For case z1, we observe that it brings significant burdens on NW vendors and extra RAN2 impacts. More RAN1 inputs will be helpful for RAN2 discussions.

Analysis on RAN2

In the clause 7.2.1.4 of TR 38.843 [1], RAN2 has clarified several candidate solutions for model transfer/delivery.

#### 7.2.1.4 Model transfer/delivery Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study. Nonetheless, to support AI/ML model transfer/delivery, the following solutions are considered: - Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

We think RAN2 has done sufficient evaluations on candidate solutions. More RAN1 inputs will be helpful for RAN2 discussions, such as more analysis on model transfer cases (as mentioned in section 2.1 in this paper) and possible content. Once RAN2 can have sufficient RAN1 inputs, RAN2 may discuss possible RAN2 impact, comparison and other aspects if possible.

Proposal 1: For now, RAN2 can wait for more RAN1 inputs on model transfer/delivery, such as more analysis on model transfer cases and possible content.

Conclusions

In this contribution, we share our further consideration about model transfer/delivery, based on RAN1 latest progress. Our observations and proposals are summarized as follows:

Observation 1: Model transfer/delivery is mainly for two-sided model. From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported).

Observation 2: For model transfer/delivery for two-sided model, RAN1 will only discuss case y and z4 for model(s) trained at the NW side.

Observation 3: For case z4, if alignment of the model structure is done offline, it brings significant burdens on NW vendors (i.e. via training of different model structures). On the other hand, if alignment of the model structure is defined in the specification, the burdens on NW vendor may be less as NW may decide to train models under the limited structures. The decision on this lies in RAN1 and more RAN1 inputs will be helpful for RAN2 discussions.

Observation 4: For model transfer/delivery for two-sided model, RAN1 will only discuss case y and z1 for model(s) trained at the UE side.

Observation 5: For case z1, we observe that it brings significant burdens on NW vendors and extra RAN2 impacts. More RAN1 inputs will be helpful for RAN2 discussions.

Proposal 1: For now, RAN2 can wait for more RAN1 inputs on model transfer/delivery, such as more analysis on model transfer cases and possible content.

- References

- 3GPP TR 38.843 v18.0.0

- RAN1 Chair’ s Notes RAN1 #116 eom0

- RAN1 Chair’ s Notes RAN1 #116b eom2

- RAN1 Chair’ s Notes RAN1 #118 eom1