3GPP TSG-RAN WG2 #127bis	R2-2408853

Hefei, China, Oct 14th – 18th 2024

Agenda Item:	6.1.1

Source:	Ericsson

Title:	Enhanced channel raster for (e)RedCap UE

Document for:	Discussion

# 1	Introduction

At RAN2#127, RAN2 received LS [1] from RAN2 on mandating enhanced channel raster for (e)Redcap UE.

In the email discussion [POST127][032][Channel Raster], RAN2 attempted to agree on related CRs to TS 38.306, but due to different understandings among companies on the need for UE capability signaling, the discussion was postponed.

In this document we share our understanding and propose way forward.

# 2	Discussion

## 2.1	Enhanced Channel Raster from RAN2 point of view

In Rel-18 WI [2], an enhanced channel raster with 10 kHz granularity for both the UE and BS was specified to meet the objective such that “…configuring a narrower UE channel BW inside a wider gNB channel BW is always possible”. The concrete problem was to handle UE implementations only capable of locating their channel bandwidth on the “legacy” 100 kHz channel raster.

RRC signalling already supported sufficient granularity to configure a narrower UE channel BW on the enhanced channel raster.

#### –	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2).

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

txDirectCurrentLocation Indicates the downlink Tx Direct Current location for the carrier. A value in the range 0..3299 indicates the subcarrier index within the carrier. The values in the value range 3301..4095 are reserved and ignored by the UE. If this field is absent for downlink within ServingCellConfigCommon and ServingCellConfigCommonSIB, the UE assumes the default value of 3300 (i.e. "Outside the carrier"). (see TS 38.211 [16], clause 4.4.2). Network does not configure this field via ServingCellConfig or for uplink carriers.

subcarrierSpacing Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

In the Rel-18 WI, RAN4 defined the related UE performance requirements. Without those, configuring the UE outside the legacy 100 kHz channel raster would in principle not be supported by the UE, although allowed in configuration signalling.

RAN2 introduced the following Rel-18 UE capability.

| enhancedChannelRaster-r18 Indicates whether the UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 [2] and TS 38.101-5 [34]. It is mandatory with capability signalling for all Rel-18 UEs for certain bands as defined in TS 38.101-1 [2] and TS 38.101-5 [34]. Otherwise, it is optional. | Band | CY | N/A | FR1 only |
| --- | --- | --- | --- | --- |

The enhanced channel raster is also an Early Implementable Feature [TS 38331, Annex C].

| RP-240667: UE capability for Enhanced channel raster | 4445 | 3 | Release 16 |  |
| --- | --- | --- | --- | --- |

## 2.2	Status in RAN4

RAN4 discussed in several meetings the issue with RedCap UEs in narrow-band network NR deployments. If the enhanced channel raster is not supported by RedCap UE, the network operator will need to make compromises on capacity (e.g. PDCCH), performance and spectrum utilization configuring a narrower RedCap UE-specific channel bandwidth within a wider BS channel BW, if even possible (see [3] for further details).

From LS [1]:

<cut>

RedCap UEs must always be configured with BWPs and associated UE-specific channel bandwidths smaller than the carrier bandwidth when operated in BS channel bandwidths greater than 20 MHz. RAN4 therefore consider support of the enhanced raster by RedCap UEs essential and should therefore be mandated for all RedCap UEs from Rel-17, eRedCap UEs from Rel-18. To this end, RAN4 has introduced the enhanced raster for RedCap in the Rel-17 and Rel-18 versions of 38.101-1 in a new sub-clause 5.4I for RedCap (see R4-2414434), consistent with the enhanced raster introduced for all UEs in the Rel-18 version of 38.101-1 (2024-03).

<cut>

:

To avoid this problem, RAN4 agreed to mandating rel-17 Redcap UE to support the 10 kHz channel raster, and RAN4 sent the LS [1] to RAN2, where the message is clear:

Potential compatibility issues for existing RedCap UEs were discussed at length in RAN4. Although this is a late new requirement for Rel-17 RedCap UE, no company expressed any problems with existing Rel-17 RedCap UEs or existing RedCap deployments.

NOTE:	The RedCap UE, which complies with Rel-17 specifications of version 17.14.0 and before version 17.14.0, may support the legacy channel raster only

Still, the following note was introduced in TS 38.101-1:

RAN4 indicated enhanced channel raster should be mandated for RedCap UEs from Rel-17, eRedCap UEs from Rel-18.

Note in RAN4 spec indicate a UE capability bit may be defined for RedCap UE.

## 2.3	Way forward in RAN2

Traditionally, RAN2 is very careful to avoid potential compatibility issues when new features are introduced. RAN2 typically introduces UE capability bits to avoid compatibility problems with existing UEs that do not support the new feature.

However, in this case of enhanced channel raster for RedCap UE, the added feature does not involve any modifications to existing signalling. The new feature means the UE ensures it complies to RAN4 performance requirements when using existing signalling to position the UE-specific channel BW according to the 10 KHz channel raster. This could be the reason why no issues have been observed e.g. at IODT. Existing RedCap UEs might already support this without any upgrade, and they do not reject such reconfiguration (by triggering RLF).

There are in our understanding two ways forward:

- Mandate enhanced channel raster for Rel-17 RedCap UE without UE capability signalling

  - In our understanding this was the intention of RAN4.

  - Existing RedCap UEs already support enhanced channel raster.

  - There is no additional requirement on existing RedCap UE that is already compliant to set a UE capability bit.

  - Potential problems with RedCap UEs not supporting the enhanced channel raster can be taken care of by the Network deployment.

- Mandate enhanced channel raster for Rel-17 RedCap UE with UE capability signalling

  - It seems feasible to use the existing enhancedChannelRaster-r18 bit for this purpose.

  - The bit will be an “IOT-bit”, that shall be set by the UE to indicate that “…the feature has been successfully tested”.

  - To make sure the bit is useful, UE vendors need to ensure the bit is set by Rel-17 RedCap UEs.

    - A Nw can, based on deployment, decide whether to rely on the bit or not.

    - The bit can be used to detect non-compliant RedCap UEs if any.

  - The bit should be set by Rel-17/18 RedCap UEs and Rel-18 eRedCap, to follow the principle that a UE capability remains consistent across releases.

RAN2 to discuss and decide whether to mandate Enhanced channel raster for Rel-17 RedCap UE with or without UE capability signalling.

If RAN2 concludes on Mandatory with UE capability signalling, the enhancedChannelRaster-r18 is used by RedCap UE from Rel-17 and eRedCap UE from Rel-18.

- ¨

- Draft TPs for TS 38.306 for 1 and 2 are provided in Appendix A and Appendix B.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	RAN4 indicated enhanced channel raster should be mandated for RedCap UEs from Rel-17, eRedCap UEs from Rel-18.

Observation 2	Note in RAN4 spec indicate a UE capability bit may be defined for RedCap UE

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss and decide whether to mandate Enhanced channel raster for Rel-17 RedCap UE with or without UE capability signalling.

# 4	References

[1]	R2-2407826	LS on mandated support of the enhanced channel raster by RedCap UEs from Rel-17, Source RAN4

[2]	RP-231469, “New WID on Channel raster enhancement”, Ericsson

[3]	R4-2411670, Mandated support of the enhanced channel raster by RedCap UEs from Rel-17, Ericsson

# Appendix A

TP to TS 38.306 for “Mandatory without capability signalling”

| enhancedChannelRaster-r18 Indicates whether the UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 [2] and TS 38.101-5 [34]. It is mandatory with capability signalling for all Rel-18 UEs for certain bands as defined in TS 38.101-1 [2] and TS 38.101-5 [34]. Otherwise, it is optional. This capability is not applicable to RedCap or eRedCap UEs, see 4.2.21.1 and 4.2.22.1. | Band | CY | N/A | FR1 only |
| --- | --- | --- | --- | --- |

#### 4.2.21.1	Definition of RedCap UE

RedCap UE is the UE with reduced capability:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	The maximum mandatory supported DRB number is 8;

-	The mandatory supported PDCP SN length is 12 bits while 18 bits being optional;

-	The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional;

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs;

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as other UEs, unless indicated otherwise.

RedCap UE supports the channel raster as specified in TS 38.101-1 [2], clause 5.4I, for all bands supported by the UE.

#### 4.2.22.1	Definition of eRedCap UE

eRedCap UE is the UE with reduced peak data rate and, with or without reduced baseband bandwidth in FR1:

-	The maximum bandwidth is 20 MHz for FR1. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 are not supported by eRedCap UEs. eRedCap UEs do not support operation in FR2 and in FR1 60kHz SCS.

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the eRedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by eRedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for eRedCap UEs same as other UEs, unless indicated otherwise.

eRedCap UE supports the channel raster as specified in TS 38.101-1 [2], clause 5.4I, for all bands supported by the UE.

# Appendix B

TP to TS 38.306 for “Mandatory with capability signalling”

| enhancedChannelRaster-r18 Indicates whether the UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 [2] and TS 38.101-5 [34]. It is mandatory with capability signalling for all Rel-18 UEs for certain bands as defined in TS 38.101-1 [2] and TS 38.101-5 [34]. Otherwise, it is optional. Indicates whether the (e)RedCap UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified specified in TS 38.101-1 [2], clause 5.4I, for all bands supported by the UE. It is mandatory with capability signalling for all (e)RedCap UEs. | Band | CY | N/A | FR1 only |
| --- | --- | --- | --- | --- |

#### 4.2.21.1	Definition of RedCap UE

RedCap UE is the UE with reduced capability:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	The mandatory support (with capability signalling, enhancedChannelRaster-r18) of the channel raster as specified in TS 38.101-1 [2], clause 5.4I, for all bands supported by the UE

-	The maximum mandatory supported DRB number is 8;

-	The mandatory supported PDCP SN length is 12 bits while 18 bits being optional;

-	The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional;

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs;

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as other UEs, unless indicated otherwise.

#### 4.2.22.1	Definition of eRedCap UE

eRedCap UE is the UE with reduced peak data rate and, with or without reduced baseband bandwidth in FR1:

-	The maximum bandwidth is 20 MHz for FR1. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 are not supported by eRedCap UEs. eRedCap UEs do not support operation in FR2 and in FR1 60kHz SCS.

-	The mandatory support (with capability signalling, enhancedChannelRaster-r18) of the channel raster as specified in TS 38.101-1 [2], clause 5.4I, for all bands supported by the UE

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the eRedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by eRedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for eRedCap UEs same as other UEs, unless indicated otherwise.