3GPP TSG-RAN WG2 Meeting #127bis	R2-2408857

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.7.4.1

Source:	China Telecom

Title:	Discussion on Leftover Issues for LCP Prioritization

Document for:	Discussion and Decision

- Introduction

In the RAN2#127 meeting, it was agreed that the LCP restriction enhancement is no longer considered as a candidate solution for LCP enhancement in Rel-19 XR Enhancement WI. In the RAN2#126 meetings, the following agreements were made regarding LCP prioritization.

- For LCP enhancements, LCP Prioritisation: Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR; The solution should consider impact on UE complexity (as already indicated in SI objective description); For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data. FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH. LCP prioritization within a logical channel will not be considered in RAN2 discussions

The intention of this paper is to discuss the UL scheduling enhancements concerning LCP prioritization for XR.

2 Discussion

To prioritize the transmission of the data with low remaining time, one efficient solution is to introduce additional priority configuration for delay-critical data. However, whether the priority only applies to the delay-critical data within the LCH or to the whole LCH is undetermined. In the last meeting, it was agreed that the LCH prioritization within a LCH is not considered, which means that the current LCP is based on LCH granularity. Therefore, it is not reasonable to configure the additional high priority to delay-critical data and configure the legacy priority to the other data within a LCH. In other words, it is more preferable to apply the additional priority to the whole LCH.

Proposal 1: Agree to introduce the additional priority to the whole LCH.

If there is delay critical data, the additional priority should be applied to the LCH. When all delay-critical data is assembled, the legacy priority should be applied during the remaining resource allocation procedure. Otherwise, it will be unfair for other LCHs without delay-critical data whose priority remains the same all the time. For instance, it is assumed that the legacy priority of LCH1 and LCH2 are 5 and 3, respectively. In case there is delay-critical data in LCH1, its priority will become 2. Based on the LCH priority, the delay-critical data will be assembled first during the resource allocation procedure. After all delay-critical data is assembled, it is more reasonable for LCH1 to use the legacy priority 5 during the remaining resource allocation procedure.

Proposal 2: The additional priority is applied in case the delay-critical data exists.

Based on the above analysis, we can observe that the priority may be reconfigured/revised based on whether the delay-critical data exist. If there is delay-critical data, the additional priority is applied for the LCH. After all delay-critical data is assembled, the MAC entity will adjust the LCH priority to the legacy one before the remaining resource allocation procedure. In other words, the LCH priority may be adjusted before the resource allocation based on whether there is delay-critical data.

Proposal 3: After all delay-critical data is assembled, the additional priority may be adjusted to the legacy one.

It is undetermined how to configure the additional priority of LCH with delay-critical data. In legacy, the LCH priority is configured by RRC signalling. To reduce the specification impact, the additional priority of LCH with delay-critical data could also be configured by RRC signalling. In this way, the network can have more flexibility in prioritizing some important data, other than always prioritizing delay-critical data.

Proposal 4: The additional priority of LCH with delay-critical data could be configured by RRC signalling.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: Agree to introduce the additional priority to the whole LCH.

Proposal 2: The additional priority is applied in case the delay-critical data exists.

Proposal 3: After all delay-critical data is assembled, the additional priority may be adjusted to the legacy one. Proposal 4: The additional priority of LCH with delay-critical data could be configured by RRC signalling.

- References

- R2-24079xx, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands