3GPP TSG-RAN WG2 Meeting #127bis	R2-2408860

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.6.2

Source:	Samsung

Title:	Further Considerations to Support Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, RAN2 reached the following agreements for inter-CU LTM procedure and some details:

Agreements on inter-CU LTM:

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In this contribution, we will further address inter-CU LTM in details.

Discussion

Issue 1: Early Sync RACH resource allocation

During Rel-18 discussion, RAN2 made the following agreement

- RAN2#121bis-e From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RAN2#123-bis RAN2 understands that the source DU needs to know the early RACH configuration for each candidate cell, so that the source cell can know how to set the PDCCH order information for early RACH; The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g. preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE. RAN3 can design the necessary network signalling.

The above agreements indicated that the early sync RACH resource allocated by one candidate cell is shared by multiple UEs. This sharing method can save the RACH resource at each LTM candidate cell. Following this RAN2 agreement, RAN3 designed that the early sync resource is shared among UEs in the same gNB-DU. Thus, the early sync. RACH resource at each candidate cell is allocated per gNB-DU, as shown by an example in Fig. 1. Specifically, the Cell1 allocates three early sync. RACH resource sets, i.e.,

- Set0: used for the LTM cell switch from Cell0 to Cell1

- Set1: used for the LTM cell switch from Cell2 to Cell1

- Set2: used for the LTM cell switch from Cell3 to Cell1

Fig. 1 Early sync. RACH resource sharing per gNB-DU

In Rel-19, the inter-CU LTM has the similar issue, i.e., whether the early sync. RACH resource is shared among multiple UEs or not. The motivation in Rel18 on shared resource is also applicable for inter-CU LTM case.

Proposal 1: RAN2 is kindly asked to agree that the early sync. RACH resource of each LTM candidate cell is shared among multiple UEs for inter-CU LTM.

To realize the resource sharing, two options can be considered, i.e., per gNB-DU sharing or per gNB sharing. However, this should be discussed in RAN3 since either options have RAN3 impact. RAN2 can make decision on the sharing early sync. RACH resource, and then RAN3 handles the detailed solutions.

Issue 2: CFRA resource in LTM cell switch MAC CE

In RAN2#126 meeting, the following agreement were reached:

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

In Rel18, the CFRA resource can be included in the LTM cell switch MAC CE. This can help the UE perform CFRA if the MAC CE does not contain the TA value. In inter-CU LTM, the TA value of the candidate cell is transferred from the candidate gNB-DU to the source gNB-DU via candidate gNB-CU and source gNB-CU in sequence. Thus, it can be expected that the TA transfer will be delayed compared to intra-CU case (which is conveyed via gNB-CU only). In other words, the LTM cell switch MAC CE without TA value may happen with higher possibility than intra-CU case. In this sense, it is beneficial to support the CFRA resource indication in the LTM cell switch MAC CE for inter-CU LTM.

Proposal 2: RAN2 is kindly asked to agree that the CFRA resource can be included in the LTM cell switch MAC CE for inter-CU LTM.

Issue 3: failure case handling

In Rel18, the failure case handling was defined as follows:

- If the attemptLTM-Switch IE is configured and the selected cell is one of the LTM candidate cell, the UE can perform the LTM cell switch procedure directly;

- If the attemptLTM-Switch IE is not configured or the selected cell is not one of the LTM candidate cell, the UE perform the RRC reestablishment. Meanwhile, the LTM configuration for both MCG and SCG is released

For inter-CU case, the above scheme on the LTM attempt can be reused if the UE selects an intra-CU cell after the failure. Otherwise (the UE selects an inter-CU LTM candidate cell), how to handle the security key needs further discussion. This depends on the SA3’s solution. If SA3 determines to use the LTM MAC CE to deliver NCC, the failure recovery via an inter-CU LTM candidate cell cannot work, and the UE should not perform the recovery via the selected cell.

Proposal 3: the Rel18 handling on failure (i.e., LTM switch attempt over the selected cell, MCG/SCG LTM configuration release) is reused in R19 if the UE selects an intra-CU LTM candidate cell after the failure; otherwise (UE selects an inter-CU LTM candidate cell after failure), the failure handling is FFS (related to SA3’s inputs).

Issue 4: Coexistence of inter/intra-CU LTM

In RAN2#126 meeting, the following agreement were reached

An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

According to the above agreement, the simultaneous MCG/SCG inter-CU LTM configuration is not allowed. However, the intra-CU LTM configuration does not have such restriction, which means the following cases can be supported:

- Case 1: intra-CU MCG LTM + inter-CU SCG LTM

- Case 2: inter-CU MCG LTM + intra-CU SCG LTM

Proposal 4: RAN2 is kindly asked to confirm the support of the following cases for simultaneous MCG/SCG LTM configuration:

- Case 1: intra-CU MCG LTM + inter-CU SCG LTM

- Case 2: inter-CU MCG LTM + intra-CU SCG LTM

Issue 5: Support of MIMO with 2TA

In R18, the MIMO with 2TA was supported for intra-CU LTM. The UE can determine the TAG based on the received TCI state in the LTM cell switch MAC CE. This feature can be also supported for inter-CU LTM.

Proposal 5: The MIMO with 2TA is supported for inter-CU LTM by reusing the Rel18 scheme.

Conclusion

Proposal 1: RAN2 is kindly asked to agree that the early sync. RACH resource of each LTM candidate cell is shared among multiple UEs for inter-CU LTM.

Proposal 2: RAN2 is kindly asked to agree that the CFRA resource can be included in the LTM cell switch MAC CE for inter-CU LTM.

Proposal 3: the Rel18 handling on failure (i.e., LTM switch attempt over the selected cell, MCG/SCG LTM configuration release) is reused in R19 if the UE selects an intra-CU LTM candidate cell after the failure; otherwise (UE selects an inter-CU LTM candidate cell after failure), the failure handling is FFS (related to SA3’s inputs).

Proposal 4: RAN2 is kindly asked to confirm the support of the following cases for simultaneous MCG/SCG LTM configuration:

- Case 1: intra-CU MCG LTM + inter-CU SCG LTM

- Case 2: inter-CU MCG LTM + intra-CU SCG LTM

Proposal 5: The MIMO with 2TA is supported for inter-CU LTM by reusing the Rel18 scheme.

References