3GPP TSG-RAN WG2 Meeting #R2-127bis		R2-2408865 Hefei, China, Oct 14-18, 2024

Agenda Item:	8.5.3

Source: 	Rakuten

Title:  	Detection and access of NES cells with OD-SIB1

Document for:	Discussion and decision

# Introduction

RAN#105 approved the Rel-19 WI on enhancements of network energy savings for NR (RP-242354) and it includes the following objective highlighted below:

2. Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

# Discussion

This contribution focuses on the second objective highlighted in the above WID.

## Progress from RAN1

In this section, we quote some agreements from RAN1 to further our analysis in RAN2.

RAN1#116 (February 2024) made the following agreements:

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options.

On target cell of UL WUS transmission:

- Option 1: UE transmits UL WUS to NES Cell

- Option 2: UE transmits UL WUS to Cell A

On configuration provision for UL WUS transmission

- Option A: UE obtains the UL WUS configuration from NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

Other options are not precluded

Agreement

- For study of UL WUS design, consider at least PRACH as a starting point

  - FFS: Whether there is dedicated PRACH resource for SIB1 request

- Other option(s) not precluded

RAN1 #116-bis made the following agreements.

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS.

Based on the above RAN1 agreements, the Cell A and NES cell could be depicted using the below figure 1.

The cell A in the below example is a macro cell and the NES cell is a small cell which has at least partial overlapping coverage with the macro cell.

Figure 1: NES Cell and Anchor cell topography

## UL WUS configuration in RRC Connected mode

There were 3 scenarios discussed during the SI phase and Case 1 was one among them,

  - Option 1: UE transmits UL WUS to NES Cell

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option X: UE receives on-demand SIB1 from NES Cell

Case 1 included a sub use-case where UE obtains the UL-WUS configuration from the NES cell directly where the UL-WUS configuration of the NES cell is provided to the UE when it was in RRC Connected before being moved to IDLE/INACTIVE.

This sub use-case could prove very useful as it ensures that the UE receives the UL WUS configuration already in RRC Connected and hence doesn’t have to visit Cell A before acquiring the UL WUS configuration.

Although the UL WUS configuration is provided to the UE from NES cell, since this is in RRC Connected mode, it is not clear from the WID whether this sub use-case is in the scope of the WI or excluded.

RAN2 sends an LS to RAN1 to clarify if providing UL WUS configuration to the UE while in RRC Connected mode is in scope of the WI.

## Detection and access of NES cell in Case 2(Option 1+B+X)

Case 2 is,

  - Option 1: UE transmits UL WUS to NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

  - Option X: UE receives on-demand SIB1 from NES Cell

A pre-Rel-19 UE can determine from the Master Information Block (MIB) that the cell is not broadcasting SIB1 periodically. Such a UE will therefore not perform cell reselection to a NES cell, at least in periods when the cell is not transmitting SIB1.

A cell that is not transmitting SIB1 periodically could be an NES cell or a non-NES cell. Hence it is important for UEs to identify whether a detected cell is an NES cell and whether it can request OD-SIB1 to camp/enter the NES cell in IDLE/INACTIVE modes.

Since option B is about UE obtaining UL WUS configuration of NES cell from cell A, it is imperative that the NES cell IDs and their corresponding UL-WUS configurations are broadcasted in one of the SIBs of cell A and a pointer to that SIB is provided in SIB1.

The NES cell ID list and their corresponding UL-WUS configurations are broadcasted in one of the SIBs of Cell A.

From a UE pov, before coming to NES cell, it has already performed a cell reselection to the cell A, as it could not find the SIB1 to camp/enter the NES cell. Moreover, the OD-SIB1 is a feature designed to enhance network energy savings. Hence, we think that a UE should be permitted to go to NES cell, only when really needed. This ensures that the energy savings gains are maximised. Some additional RRM criteria may be imposed before a UE is redirected to the NES cell in case 2. Some example criteria are,

- Cell A is overloaded with RRC IDLE/INACTIVE UEs.

- Radio conditions in Cell A is poor compared to NES cell.

- Radio condition in NES cell is better than a predefined threshold.

- UE is intending to move to RRC Connected for data transmission and NES cell can offer capacity.

These criteria could be predefined in RAN2, and the UE performs the RRM criteria check based on indication in Cell A SIB1.

In Case 2, UE is camped on cell A before it can retrieve UL-WUS configuration of an NES cell.

RAN2 discusses and agrees the minimum criteria that a UE should satisfy in cell A, before requesting OD-SIB1 transmission from NES cell to camp there.

Once the UE satisfies the minimum criteria, it performs RACH, based on the UL-WUS configuration and requests OD-SIB1. The NES cell starts broadcasting OD-SIB1 and the UE can perform cell reselection.

# Conclusion

We have the following observations:

Observation 1	In Case 2, UE is camped on cell A before it can retrieve UL-WUS configuration of an NES cell.

We have the following proposals:

Proposal 1	RAN2 sends an LS to RAN1 to clarify if providing UL WUS configuration to the UE while in RRC Connected mode is in scope of the WI.

Proposal 2	The NES cell ID list and their corresponding UL-WUS configurations are broadcasted in one of the SIBs of Cell A and a pointer to that is provided in SIB1.

Proposal 3	RAN2 discusses and agrees the minimum criteria that a UE should satisfy in cell A, before requesting OD-SIB1 transmission from NES cell to camp there.

# Reference

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

RAN1 Chairman Notes : RAN1 #116bis