3GPP TSG RAN WG2 #127bits                                                                R2-2408868

Hefei, China, Oct 14h –18rd, 2024

Agenda Item:	8.2.5

Source:	TCL

Title:	Considerations on Topology 2 for Ambient IoT

Document for:	Discussion and Decision

- Introduction

Topology 2 and UE as intermediate node is another scope in the Rel-19 Ambient IoT (A-IoT) SID [1], where the A-IoT device communicates bidirectionally with an intermediate node between the device and base station (BS). RAN2#127 meeting has achieved some initial agreements about topology 2 for A-IoT [2]:

Agreements - RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. - A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device. - RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress. - RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied) - NW is in control of resources to be used by AIoT air interface. - Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

It can be found that in either topology 1 and topology 2, there is only one type A-IoT air interface. That is, the A-IoT device communicates the UE reader or BS reader using the same A-IoT air interface, and the A-IoT device does not need to know its topology. In this contribution, we will discuss the following aspects related to topology 2 for Ambient IoT, which focuses on Uu interface between UE and gNB.

- System architecture

- AIoT data transfer

- Resource allocation

- RRC state of UR

- Stage 2 general procedure for Topology 2

- Discussion

2.1 System architecture

A logical system architecture for topology 2 has been discussed in SA2 [3] and RAN3[4]. Based on the meeting progress and input, we depict the reference system architecture for topology 2, shown in Figure 1, where we assume that AMF with the AIoT logical function. As can be seen from the figure, UE acts as the reader to communicate with other nodes through different solutions

- For A-IoT device: the UE Reader communicates with the A-IoT device via AIoT-Uu interface.

- For NG-RAN:  the UE Reader communicates with the NG-RAN node via Legacy NR Uu interface.

- For CN Function: the UE Reader communicates with the CN functions can be via N1 interface, or or NG /a new interface.

Figure 1. Reference system architecture for topology 2

Observation 1: According to SA and RAN3, in the proposed system architecture, UE acts as the reader to communicate with other nodes through different solutions:

  - For A-IoT device: the UE Reader communicates with the A-IoT device via AIoT-Uu interface.

  - For NG-RAN: the UE Reader communicates with the NG-RAN node via Legacy NR Uu interface.

  - For CN Function: the UE Reader communicates with the CN functions can be via N1 interface, or NG /a new interface.

2.2 AIoT data transmission

SA2 is discussing the protocol stacks for topology 2, the various potential protocol stacks are depicted in the following figures, which include the CP-based protocol stack and UP-based protocol stack.

Figure 2. CP-based protocol stack for Topology 2 (AS-based)

Figure 3. CP-based protocol stack for Topology 2 (NAS-based)

Figure 4. UP-based protocol stack for Topology 2 (PDU-based)

Based on the above protocol stacks, there are serval options for the AIoT data transmission accordingly.

- Option1 (AS-based solution): In Figure 2, the AIoT data is carried in AS signaling (e.g., RRC message) between UE reader and the gNB, therefore, the gNB has the capability to decode the AS signaling and NG-AP message. Moreover, gNB can do some control for the AIoT data transmission. Take an example for the UL data transmission, the UE reader receives the AIoT data from AIoT device and forwards the data to the gNB via AS signaling, then the gNB transfers the AIoT data to CN functions via NG-AP or new defined interface.

- Option2 (NAS-based solution): In Figure 3, the AIoT data is carried in NAS signaling between UE reader and the CN functions (e.g., AMF, or AIoT Function). The AIoT information are contained in NAS messages and be forwarded to UE reader via gNB transparently.  In this option, CN handles the AIoT data transmission.

- Option3 (PDU-based solution): In Figure 4, the AIoT data is carried in PDU session between UE reader and the CN functions (e.g., UPF), which is user plane mode. The AIoT information is transparent for UE reader and gNB, the corresponding data transmission between UE reader and RAN can based on the legacy PDU session transmission.

Observation 2: Based on the protocol stack proposed by SA2, there are serval options for the AIoT data transmission:

  - Option 1: AS-based solution.

  - Option 2: NAS-based solution.

  - Option 3: PDU-based solution.

For Option1 (AS-based solution), it will raise some specific impacts for RAN2. On the contrary, as for Option 2 (NAS-based solution) and Option 3 (PDU-based solution), the specification efforts are out of RAN2’s scope. From our perspective, since the data transmission highly depends on the protocol stacks, it is safe to postpone the AIoT data transmission at current stage until the protocol stacks for Topology 2 are determined by SA2.

Proposal 1: RAN2 is kindly asked to postpone the discussion of AIoT data transmission, until the protocol stacks for topology 2 are determined by SA2.

2.3 Resource Allocation

In topology 2, gNB is responsible for allocating the radio resource, which is used for the transmission over AIoT-Uu interface and Uu interface. Since various AIoT services has different requirements, it is a good idea that some assistance information related AIoT services is visible to gNB to help gNB assign the appropriate resources and avoid resource waste. From our perspective, there are three possible ways of resource allocation.

  - Option 1: gNB receives some assistance information related AIoT services from CN firstly, and then allocates the appropriate resources to one or more UE readers.

  - Option 2: gNB receives some assistance information related AIoT services from UE reader firstly, and then allocates the appropriate resources to the UE reader.

  - Option 3: gNB assigns a common resource pool to one or more UE readers in advance.

Proposal 2: gNB is responsible for allocating the radio resource for AIoT-Uu interface and Uu interface.

Proposal 3: RAN2 is kindly asked to discuss the resource allocation for topology 2 by considering the following two options:

  - Option 1: gNB receives some assistance information related AIoT services from CN firstly, and then allocates the appropriate resources to one or more UE readers.

  - Option 2: gNB receives some assistance information related AIoT services from UE reader firstly, and then allocates the appropriate resources to the UE reader.

  - Option 3: gNB assigns a common resource pool to one or more UE readers in advance.

2.4 RRC state of UE

Legacy UE supports three kinds of RRC states, including RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED connected, whether UE of different RRC states can serve as an intermediate node in Topology 2 needs to be discussed.

It is natural that the RRC_CONNECTED UE serves as the intermediate node in Topology 2, as it is always connected with the NW and is capable of transmitting the AIoT data in real-time. However, considering some special features of AIoT services, such as small data volume and low latency requirement, if UE remains in the RRC_CONNECTED state only for AIoT data transmission, it will increase the UE power consumption. On the other hand, AIoT device has no RRC state concept and does not know its topology, so AIoT device may not acquire and effect the intermediate node RRC state. Therefore, in our understanding, the RRC_INACTIVE UE can be regarded as the intermediate node in Topology 2. For example, RRC_INACTIVE UE can use the SDT scheme to transfer the AIoT data. While for RRC_IDLE UE, because the NW does not have UE’s context information, it may generate some security problems. So, we think that the RRC_IDLE UE as the intermediate node is not preferred.

Proposal 4: The UE can serve as an intermediate node in the RRC_CONNECTED state or RRC_INACTIVE state in Topology 2.

Proposal 5:  The RRC_IDLE UE serves as an intermediate node in Topology 2 is not preferred.

- Conclusion

We have the following observations and proposals

Observation 1: According to SA and RAN3, in the proposed system architecture, UE acts as the Reader to communicate with other nodes through different solutions:

  - For A-IoT device: the UE Reader communicates with the A-IoT device via AIoT-Uu interface.

  - For NG-RAN: the UE Reader communicates with the NG-RAN node via Legacy NR Uu interface.

  - For CN Function: the UE Reader communicates with the CN functions can be via N1 interface, or NG /a new interface.

Observation 2: Based on the protocol stack proposed by SA2, there are serval options for the AIoT data transmission:

  - Option 1: AS-based solution.

  - Option 2: NAS-based solution.

  - Option 3: PDU-based solution.

Proposal 1: RAN2 is kindly asked to postpone the discussion of AIoT data transmission, until the protocol stacks for topology 2 are determined by SA2.

Proposal 2: gNB is responsible for allocating the radio resource for AIoT-Uu interface and Uu interface.

Proposal 3: RAN2 is kindly asked to discuss the resource allocation for topology 2 by considering the following two options:

  - Option 1: gNB receives some assistance information related AIoT services from CN firstly, and then allocates the appropriate resources to one or more UE readers.

  - Option 2: gNB receives some assistance information related AIoT services from UE reader firstly, and then allocates the appropriate resources to the UE reader.

  - Option 3: gNB assigns a common resource pool to one or more UE readers in advance.

Proposal 4: The UE can serve as an intermediate node in the RRC_CONNECTED or RRC_INACTIVE state in Topology 2.

Proposal 5:  The RRC_IDLE UE serves as the intermediate node in Topology 2 is not preferred.

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, RAN#103, March, 2024

- RAN2 #127 Chair Notes

- RAN3 #125 Meeting Report

- 3GPP TR 23.700-13 V1.0.0 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)