3GPP TSG-RAN WG2 Meeting #127bis	R2-2408882

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.7.3

Source:	Ericsson

Title:	XR – RRM Measurement Gap/Restriction Enhancements

Document for:	Discussion, Decision

# 1	Introduction

The following are the agreements from the RAN1#118 on measurement gap enhancements.

Option 1: Support Alt. 1-1: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. Option 2: Support Alt. 3-1: - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions.

In addition, an LS was also sent out to RAN4 with RAN2 in CC, as follows:

RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information

In this contribution, we discuss the two options including the aspects as mentioned in the LS and provide our views.

# 2	Discussion

## 2.1 	Measurement Gap Enhancements

As shown in the above section, RAN1 has discussed two options for the measurement gap enhancements namely Option 1, dynamic indication and Option 2, semi-static solution with RAN1/RAN4 currently evaluating these two solutions. RAN2 is unable to evaluate the two options on account of it being outside RAN2’s expertise. And the impact on RAN2 depends on the outcome of the discussions, thus we should wait for progress in RAN1/RAN4.

Wait for RAN1/RAN4 progress on the solution for measurement gap enhancements.

## 2.2 	On the LS

In addition to the agreements, an LS was sent to RAN4 from RAN1 with RAN2 in CC list. The LS discusses whether to introduce new UE assistance information for solutions to enable Tx/RX gaps/restrictions caused by RRM measurements. The information relevant for RAN2 are the following:

- FFS: UE assistance information related to traffic:

  - FFS: PSI (PDU set importance);

- FFS: UE assistance information related to UE mobility:

  - FFS: L3 parameters related to mobility, e.g., static or not

For the UE assistance information related to traffic, although technically the PSI information can be used by the network to select whether to skip a particular measurement gap for e.g., only skip measurement gaps (or prioritize scheduling) for high priority data. Given the dynamic nature of this information i.e., on a per-packet basis, we believe it is not feasible for the UE to provide such information in the UAI. In addition, the PSI in the UL is up to UE implementation, i.e., not all the UEs can support it and it is not a reliable criterion.

- Determination of PSI in UL is up to UE implementation i.e., not all UEs support it and thus, an unreliable criterion.

Do not support including PSI in the UAI for measurement gap enhancements.

On UE assistance information related to UE mobility e.g., whether the UE is static, this information could potentially be useful for the network in deciding the measurement gap configuration/skipping occasions.

- UE mobility information could potentially be useful to configure measurement gaps.

It could also be simple to reuse the framework of the relaxed measurement criterion for a stationary (e)RedCap UE. In the current procedure, the network provides an RRM-MeasRelaxationReportingConfig for determination of stationarity and the UE can report its rrm-MeasRelaxationFulfilment status in the UAI on whether the stationary criterion is met. However, RAN4 should check the feasibility of this approach and evaluate the impact on measurements.

- RAN4 should evaluate the impacts of reusing the stationary criterion for (e)RedCap UE on measurements.

Consider reusing the RRM measurement relaxation criterion for a stationary (e)RedCap UE. Send an LS to RAN4 to evaluate the impact on measurements.

# 3 	Conclusion

Based on the discussion in the previous sections, we draw the following observations:

Observation 1	Determination of PSI in UL is up to UE implementation i.e., not all UEs support it and is an unreliable criterion.

Observation 2	UE mobility information could potentially be useful to configure measurement gaps.

Observation 3	RAN4 should evaluate the impacts of reusing the stationary criterion for (e)RedCap UE on measurements.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Wait for RAN1/RAN4 progress on the solution for measurement gap enhancements.

Proposal 2	Do not support including PSI in the UAI for measurement gap enhancements.

Proposal 3	Consider reusing the RRM measurement relaxation criterion for a stationary (e)RedCap UE. Send an LS to RAN4 to evaluate the impact on measurements.