3GPP TSG-RAN WG2 Meeting #127bis	R2-2408904
Hefei, P. R. of China, October 14th – 18th, 2024

Agenda Item:	8.5.2

Source:	InterDigital

Title:	On demand SSB transmission for SCell

Document for:	Discussion, Decision

# Introduction

The Rel-19 New WID on Network Energy Savings for NR has the following objective on on-demand SSB SCell [1]:

The WID includes the following objective accordingly:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In RAN2#127, the objective was discussed for the first time and the following was agreed:

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In the last RAN1 meeting, RAN2 has received the following LS from RAN1 [3]:

# 1 Overall description RAN1 has discussed signaling mechanisms to indicate on-demand SSB transmission and has reached agreements to support indication by both RRC and MAC CE signalling. Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement (RAN1#116bis) For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication To provide more context for RAN2, the following are relevant RAN1 agreements on Cases and Scenarios for on-demand SSB SCell operation. Agreement (RAN1#116) Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement (RAN1#116) For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement (RAN1#116bis) For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. RAN1 respectfully requests RAN2 to design MAC CE based signaling to indicate on-demand SSB transmission for Scenarios #2 and #2A. RAN1 is working on the further details needed for RRC and MAC CE based signaling to indicate on-demand SSB transmission and will inform RAN2 as further agreements are made. # 2 Actions To TSG RAN2 ACTION: RAN1 respectfully requests RAN2 to design MAC CE based signaling to indicate on-demand SSB transmission for Scenarios #2 and #2A.

Full agreements made in RAN1#116 meeting are provided in the appendix. In this contribution, we discuss the enhancements for enabling on-demand SSB SCell along with the MAC CE design to indicate it.

# Discussion

For CA deployments, SCells with SSB-less Scell has been supported since Rel-15 for both FR1 and FR2. In Rel-18 the support for SSB-less Scell is extended for collocated cells in FR1 that are in inter-band CA (e.g. within max receive time, frequency and power difference). In the inter-base CA case, the NW signals to the UE which cell is the reference cell (e.g. a Pcell or another Scell), where the reference cell acts as the timing reference and AGC source of the SSB-less Scell.

Supporting inter-band CA for FR2 comes with challenges, e.g. achieving synchronization, L1/L3 measurements and beam management, especially when the cells are non-collocated and only the Scells operate in FR2 while other cells in FR1. For energy saving purposes in CA, it is important for the Scells to operate in a NES mode as long as possible without or with limited SSB transmission.

## Signalling for OD-SSB reception

MAC CE signalling

During Rel-18 SI, it was discussed that the Scells which are used as capacity cells and operating in NES mode are not expected to transmit SSBs or only transmit sparse SSBs. gNB initiated triggering of OD-SSB in the context of Scell activation procedure has been agreed by R1. The following options were discussed in RAN1 for the indication of reception of OD-SSB; the selection of which option was left for RAN2:

- MAC CE is separate from the legacy MAC CE for SCell (de)activation

- MAC CE can be also used for SCell (de)activation

- MAC CE is the same as the legacy MAC CE for SCell (de)activation with enhancements

The third option is ruled out by R2 agreements last meeting, the first option is agreed, and it is FFS whether the second option can be used for Scenario 2A (when both OD-SSB and SCell are activated together).

Since RAN1 agreed to support OD-SSB before SCell activation, at least a MAC CE separate from the legacy MAC CE is needed; this was agreed last meeting. Further, since the activation of the SCell itself is based on the network’s decision, i.e. based on the reception of UE measurement and L1 CSI, the activation decision may lag the indication for the reception of an OD-SSB.

Observation 1:	Per R2 agreement, reception of on-demand SSB on an SCell is indicated by a new MAC CE signalling separate from the legacy MAC CE for SCell (de)activation.

One FFS from last meeting is “FFS if we need further optimization for scenario 2A”. In our view, it is beneficial to keep the design of the MAC CE fixed, simple and applicable to all scenarios, rather than having fields that are specific only to Scenario 2A, thus considered as overhead for Scenario 2. For signalling OD-SSB for Scenario 2A, the network can ensure that both the new OD-SSB indication MAC CE and the legacy SCell activation MAC CE are multiplexed in the same PDSCH. Therefore, there is no need for any specific optimization for Scenario 2A

Observation 2:	it is beneficial to keep the design of the MAC CE simple, fixed and applicable to all scenarios, rather than having fields that are specific only to Scenario 2A (e.g. to indicate SCell (de)-activation).

Proposal 1:	The OD-SSB indication MAC CE signalling does not indicate SCell (de)-activation. Reuse legacy MAC CE for SCell (de)activation.

Proposal 2:	For signalling OD-SSB for Scenario 2A (when both OD-SSB and SCell are activated together), confirm the understanding that both the new OD-SSB indication MAC CE and the legacy SCell activation MAC CE are multiplexed in the same PDSCH.

Per RAN1 agreements, MAC CE based signalling is used to indicate on-demand SSB transmission on an SCell supporting on-demand SSB SCell operation. RAN1 further agreed that for a cell supporting on-demand SSB SCell operation, multiple candidate values can be configured by RRC for the periodicity of the on-demand SSB, and the applicable value can be indicated by MAC CE. It is FFS whether there are any other relevant parameters. However, looking at the list of parameters from RAN1, it appears like there are no other parameters that need to be indicated dynamically.

Proposal 3:	The OD-SSB indication MAC CE signalling includes the SCell index, and an index for the applicable periodicity among those configured. No other parameters need to be included in the MAC CE.

Given R1 agreed to indicate a value associated with the OD-SSB periodicity, R2 can assume that there will be one octet to indicate the periodicity index from the values configured in RRC.

Proposal 4:	One octet in the OD-SSB indication MAC CE is used to indicate the periodicity index from the values configured in RRC.

From a MAC CE design perspective, two assumptions can be discussed in RAN2:

- Option 1: OD-SSB is indicated for at most one SCell at a time. A “Serving Cell ID” field is used to indicate the identity of the Serving Cell for which the OD-SSB is indicated. The OD-SSB MAC CE therefore has a fixed size. In R1 discussion so far, not much discussion has assumed that OD-SSB is indicated for multiple SCells simultaneously. A sample MAC CE is illustrated in Figure 1 for such assumption. With such design, there are some reserved bits in the first octet that can be used for SCell activation or for the (de)-activation of the OD-SSB transmission.

Figure 1: illustration of a fixed-size OD-SSB MAC CE for Option 1

- Option 2: OD-SSB can be indicated for more than one SCell in the MAC CE simultaneously. A bit map is used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable periodicity index. The OD-SSB MAC CE therefore has a variable size. A sample MAC CE is illustrated in Figure 2 for such assumption, at least for the case of up to 7 SCells.

Figure 2: illustration of a variable-size OD-SSB MAC CE for Option 2

Provided that the MAC CE does not provide (de)-activation of SCells, it is beneficial to keep the MAC CE simple and fixed in size.

Proposal 5:	RAN2 to discuss the following options for the design of the OD-SSB MAC CE:

- Option 1: OD-SSB is indicated for at most one SCell at a time. A “Serving Cell ID” field is used to indicate the identity of the Serving Cell for which the OD-SSB is indicated. The OD-SSB MAC CE therefore has a fixed size.

- Option 2: OD-SSB can be indicated for more than one SCell in the MAC CE simultaneously. A bit map is used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable periodicity index. The OD-SSB MAC CE therefore has a variable size.

It is not clear whether the MAC CE should support activation only of the OD-SSB or whether it should also support deactivation. In the RAN1 agreement, the following note mentions: “Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.” One can rely on RRC reconfiguration to turn off OD-SSB transmission, e.g. per proposal 6. However, if only the Serving Cell ID is indicated in the MAC CE, there will likely be 3 spare bits in that octet, as the Serving Cell ID is only 5 bits, and one of those R bits can be used to indicate whether it is activation of deactivation of the OD-SSB.

Proposal 6:	RAN2 to discuss whether the OD-SSB MAC CE supports activation-only or both deactivation/activation of OD-SSB transmission:

- Option 1: the MAC CE supports only activation of an OD-SSB for a given SCell.

- Option 2: the MAC CE supports both activation and deactivation of transmission of an OD-SSB for a given SCell.

RRC signalling

In RAN2#127, it was agreed that “RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.” While MAC CE is typically used for dynamic activation and deactivation, the initial RRC configuration can include a flag to indicate the initial activation status of the on-demand SSB. However, this status can be modified or reconfigured later. There is no need to restrict the usage of such flag however, as the same flag can be used both for reconfiguration and initial configuration.

Proposal 7:	A one-bit field is configured by RRC to indicate the activation status of OD-SSB for a given SCell, which can be used both for reconfiguration and initial configuration.

In terms of what additional parameters can be configured by RRC, some parameters can be configured for the structure and expected OD-SSB parameters. Other parameters can be inferred from the SCell configuration (e.g. Sub-carrier spacing of the OD-SSB, Gap length between SSB bursts etc). Parameters may be added in addition to those used for legacy SCell activation, per RAN1’s discussions and agreements, as listed in proposal 8. Since these parameters are likely to differ for different frequency bands, they can be configured per frequency range.

Proposal 8:	At least the following parameters can be configured by RRC per frequency range to support OD-SSB reception on SCells:

  - Frequency of the OD-SSB

  - SSB positions within an OD-SSB burst

  - Periodicity of OD-SSB

## Indication of OD-SSB by WUS

In addition to OD-SSB reception triggered by MAC CE, the request for on-demand SSB can be conveyed by the UE, as likely the condition for requesting such demand is known at the UE. The UE can therefore send the UL signal to gNB, corresponding to UL wake-up signal (WUS), to request for OD-SSB transmission. Using UL WUS as a triggering method for OD-SSB transmission also results in considerable NES gains. The Rel-18 TR [2] mentions that with sparse SSB of 160ms periodicity transmitted before gNB reception of WUS, 5 sources show technique A-3-1 of UE WUS triggering gNB for SSB/SIB/RACH, with the assumption of ideal detection of UL WUS, could achieve BS energy savings by 6.2% – 80.7% in range with UPT loss by 0%-24.2%, where the gain increases as a function of the WUS period. Given the benefits in terms of low delay, energy savings and low UPT losses, UL WUS can be supported as a triggering method for OD-SSB at SCell.

Options considered in the R18 SI for UL WUS include using PRACH resource and PUCCH resource (e.g. SR). Using a PUCCH resource for triggering on demand SSBs can be useful for UEs in connected mode, but this may require introducing a new procedure for triggering the transmission and ensuring sufficient reliability/coverage when receiving at Scell and hence, less preferable for UL WUS. Using PRACH is thus simpler, since the gNB monitors PRACH on the PCell regardless of NES states.

Contention-based PRACH resources for UL WUS can be configured to consist of a set of PRACH preambles and RACH occasions. Given there is no CBRA on the Scell, the UL WUS would have to be transmitted by the UE on the PCell.

The resources for UL WUS can be provided by the PCell to the NES-capable UEs as a separate configuration or a sub-configuration of the RACH config.

Proposal 9:	Support using PRACH preamble on the PCell as UL WUS for triggering OD-SSB on the SCell. FFS: Conditions for triggering UL WUS.

# Conclusion

The following proposals are provided for on demand SSB reception on SCells:

Observation 1:	Per R2 agreement, reception of on-demand SSB on an SCell is indicated by a new MAC CE signalling separate from the legacy MAC CE for SCell (de)activation.

Observation 2:	it is beneficial to keep the design of the MAC CE simple, fixed and applicable to all scenarios, rather than having fields that are specific only to Scenario 2A (e.g. to indicate SCell (de)-activation).

Proposal 1:	The OD-SSB indication MAC CE signalling does not indicate SCell (de)-activation. Reuse legacy MAC CE for SCell (de)activation.

Proposal 2:	For signalling OD-SSB for Scenario 2A (when both OD-SSB and SCell are activated together), confirm the understanding that both the new OD-SSB indication MAC CE and the legacy SCell activation MAC CE are multiplexed in the same PDSCH.

Proposal 3:	The OD-SSB indication MAC CE signalling includes the SCell index, and an index for the applicable periodicity among those configured. No other parameters need to be included in the MAC CE.

Proposal 4:	One octet in the OD-SSB indication MAC CE is used to indicate the periodicity index from the values configured in RRC.

Proposal 5:	RAN2 to discuss the following options for the design of the OD-SSB MAC CE:

- Option 1: OD-SSB is indicated for at most one SCell at a time. A “Serving Cell ID” field is used to indicate the identity of the Serving Cell for which the OD-SSB is indicated. The OD-SSB MAC CE therefore has a fixed size.

- Option 2: OD-SSB can be indicated for more than one SCell in the MAC CE simultaneously. A bit map is used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable periodicity index. The OD-SSB MAC CE therefore has a variable size.

Proposal 6:	RAN2 to discuss whether the OD-SSB MACE CE supports activation-only or both deactivation/activation of OD-SSB transmission:

- Option 1: the MAC CE supports only activation of an OD-SSB for a given SCell.

- Option 2: the MAC CE supports both activation and deactivation of transmission of an OD-SSB for a given SCell.

Proposal 7:	A one-bit field is configured by RRC to indicate the activation status of OD-SSB for a given SCell, which can be used both for reconfiguration and initial configuration.

Proposal 8:	At least the following parameters can be configured by RRC per frequency range to support OD-SSB reception on SCells:

  - Frequency of the OD-SSB

  - SSB positions within an OD-SSB burst

  - Periodicity of OD-SSB

Proposal 9:	Support using PRACH preamble on the PCell as UL WUS for triggering OD-SSB on the SCell. FFS: Conditions for triggering UL WUS.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023

R1-2407438, “LS to RAN2 for on-demand SSB SCell operation”, RAN1

# Appendix: Agreements on OD-SSB SCell operation

RAN1#116 Agreements

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

RAN1#116bis Agreements

- Scenarios where OD-SSB can be triggered by gNB

  - Scenario #2 with Case #1 (no always-on SSB) and Case #2 (always-on SSB is periodically transmitted)

  - Scenario #2A with Case #1 and Case #2

  - FFS: Scenario 3A with Case 1 & 2

  - FFS: Scenario 3B with Case 1 & 2

    - For Case #1, once OD-SSB is triggered, its transmission is in a periodic manner

    - Notes:

      - Scenario #2A: When UE receives SCell activation command

      - Scenario #3A: After UE receives SCell activation command until SCell activation is completed

      - Scenario #3B: When SCell activation is completed, or after SCell activation is completed

- Type of SSBs for OD-SSB

  - Down-select between the following:

    - Alt-1: It is up to gNB implementation whether OD-SSB is CD-SSB or not

    - Alt-2: OD-SSB is limited to NCD-SSB.

      - FFS: Further limitations to OD-SSB

- Type of measurements

  - L1 and/or L3 measurements based on OD-SSB is supported for the cell

  - FFS further details on L1 and/or L3 measurement (e.g. timing)

- Study signalling for indicating OD-SSB

  - Option 1: Separate signalling for providing SCell (de)activation with legacy signalling (e.g., RRC, MAC CE) and for providing OD-SSB transmission indication

  - Option 2: A single signalling providing both SCell (de)activation and OD-SSB transmission indication

RAN1#117 Agreements

- Type of signaling for indicating OD-SSB

  - Support RRC based signaling

  - Support MAC based signaling

  - FFS: DCI based signaling

    - DCI does not provide Scell (de)activation

    - If supported, details on whether to use UE-specific or group-common DCI

  - FFS: Scenarios where the signaling is applicable

- Configuration info provided for OD-SSB:

  - Frequency of the OD-SSB

  - SSB positions within an OD-SSB burst (by using signaling similar to ssb-PositionsInBurst)

  - Periodicity of OD-SSB

  - Sub-carrier spacing of the OD-SSB

  - Physical Cell ID of the OD-SSB

  - Location of on-demand SSB burst (e.g. offset, half frame)

  - Downlink transmit power of OD-SSB

  - FFS: Further config details:

    - Whether more than one OD-SSB configs can be configured for the cell to UE

    - Whether RRC parameters are newly introduced or existing are reused

  - FFS: Other parameters, e.g.

    - How many OD-SSB burst(s) are transmitted once indicated

    - Offset between HARQ-ACK corresponding to OD-SSB transmission indication signaling and OD-SSB transmission

    - Gap length between SSB bursts

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

- Timing of OD-SSB transmission upon indication by MAC CE

  - Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted OD-SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signaling from gNB to indicate OD-SSB transmission

    - SSB time domain positions of OD-SSB burst are configured by gNB.

  - FFS: Details of value of T (≥ 0) including possibility of T comprising of multiple components

    - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

- Support L1 measurements on OD-SSB

  - Periodic, semi-persistent [and aperiodic] L1 measurements based on existing CSI framework

  - FFS: Potential enhancements of CSI report config and/or triggering/activation mechanisms for L1 measurements on OD-SSB

- RAN1#118

- Signaling of OD-SSB

  - Support RRC based signaling to indicate OD-SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides OD-SSB configuration.

    - FFS: Whether to support RRC based signaling for other cases.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

  - Note: Deactivation and adaptation of on-demand SSB transmission to be separately discussed.

- Signaling of OD-SSB parameters

  - Multiple candidate values (of parameter(s) of OD-SSB) can be configured by RRC and the applicable value indicated by MAC CE

  - MAC CE indicates: Periodicity of OD-SSB

  - FFS: Other relevant parameters

- SSB type for OD-SSB

  - Support: OD-SSB on the cell is not located on synchronization raster

  - Support: OD-SSB on the cell is NCD-SSB

  - FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

  - [JR] Companies (HW, vivo, Samsung, Apple, QC, Ericsson) were against supporting OD-SSB that is on sync-raster (i.e. CD-SSB) due to possible impacts on idle/inactive UEs.

- Timeline of OD-SSB transmission

  - Time instance A: Beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of OD-SSB burst which is at least T slots after the slot where UE receives a signaling from gNB to indicate OD-SSB transmission

  - WA: T is not less than T_min = m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - WA: T=T_min

- L1 measurements on OD-SSB

  - Support aperiodic measurements based on existing CSI framework

  - FFS: potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

  - Support of LTM is a separate discussion point

- L3 measurements on OD-SSB

  - Support. Further work is up to RAN2/RAN4

- Other topics discussed (with no agreements)

  - Scenarios for OD-SSB

    - Scenario 3A and 3B are objected by Apple and vivo

  - Whether/how to support multiple OD-SSB configs

    - All NW vendors wanted to support multiple OD-SSB configs (e.g. each containing different parameter set), but no consensus on the need for > one config.

    - Discussion direction then changed to first identify what is included in one OD-SSB config

  - Contents of OD-SSB configuration

    - Parameters identified: Frequency of the OD-SSB, SSB position in burst, periodicity, SCS, PCI, Tx power, number of bursts, gap between bursts, T value

  - Transmission pattern of OD-SSB

    - All time spent on discussing/clarifying views on timeline (T value, time instance A) than on the pattern given the multiple understandings on start time

  - L1/L3 measurements on OD-SSB

    - Whether to support OD-SSB for BFD-RS (LG, Goog, DCM, NEC are supportive)

    - Whether/how to configure OD-SSB in CSI-RS resource config

      - Companies mentioned to first discuss whether UE should measure always-on SSB and OD-SSB separately or jointly to derive one CSI (e.g. for Case #2)

  - UE-triggered OD-SSB

    - Tagged as low priority proposal in FL summary