3GPP TSG-RAN WG2 Meeting #127bis	R2-2408907

Hefei, China, Oct 14th – 18th, 2024

Source:				ETRI

Title:					Discussion on additional LCP handling

Agenda Item:		8.7.4.1	LCP enhancements

Document for:		Discussion and Decision

1. Introduction

During the study of LCP enhancements by the RAN2#127 meeting, agreements on LCP enhancements have been made as the followings:

LCP restrictions

- RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

LCP enhancements – LCH prioritization

- Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR.

- The solution should consider impact on UE complexity (as already indicated in SI objective description)

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

- Use additional priority configured to LCHs in case of these LCHs with delay-critical data

- FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- We try to avoid RAN1 impacts.

- RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

- For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

- The solutions should not disallow non-delay critical data from using an UL grant.

- LCP prioritization within a logical channel will not be considered in RAN2 discussions

- FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

LCP enhancements – solution directions

- RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied:

- Alternative 1: Enhance LCP restrictions/LCH selection.

- Alternative 2: Enhance LCH prioritization.

- RAN2 should consider potential impact on traffic from SRBs.

In this contribution, we provide discussions on the LCP enhancements for XR service. For delay-critical data, additional priority is given during resource allocation within given radio resources, and methods for providing this additional priority are discussed.

2. Discussion

Additional LCH priority

When the UE transmits data on the uplink grant, the UE MAC entity performs a LCP  procedure to multiplex and transmit data. The LCP procedure is configured for a logical channel and a LCH priority is used during resource allocation for data in the buffer. The LCH priority remains fixed until the LCH is released.

In order to accommodate multi-modality services, a method of allocating resources with additional priority given to delay-critical data was discussed. There was a consensus on adopting the delay-aware LCP procedure and on providing additional LCH priority by recognizing delay-critical data. It was agreed to provide additional priority to LCHs that carry delay-critical data during the study of scheduling enhancements.

Observation 1) For delay-aware LCP enhancements, RAN2 has agreed that additional LCH priority for delay-critical data is introduced to a LCH.

Proposal 1) RAN2 confirms that an additional LCP is introduced to a LCH.

The limited transmission delay of the data over air interface is associated with the required QoS and PDU Set SN, etc. Data with remaining delay shorter than a time threshold is classified as delay-critical data and additional LCP is introduced for the LCH containing the delay-critical data. Non-delay critical data uses the legacy LCP, while delay-critical data uses the introduced additional LCP. By providing a remaining time threshold, the LCH may have flexibility in QoS management.

Observation 2) A time threshold is configured, enabling the classification of data into delay-critical data based on the remaining time.

The PDCP SDU is set either discardTimer or discardTimerForLowImportance and is associated a PDCP SN. The PDCP SN is used for in-order delivery operation. The RLC layer also assigns an RLC SN to the RLC PDU to provide reassembly and retransmission functionalities. Generally, the first transmission of data in PDCP and RLC layer is in order. If data transmission order changes in a LCH, the receiver entity could not the changed order and decides it as a reception error. Reassembly and recovery procedures are performed in which a status report message is transmitted to the transmitter entity.

Observation 3) PDCP and RLC transmit data sequentially, and if the order is changed, data recovery operations are performed through status report messages, etc.

A legacy LCP and an additional LCP are configured for the LCH. When data arrives, the legacy LCP is applied for the data. When delay-critical data occurs and is in the buffer, the LCH activates the additional LCP. Scheduling is performed with it and data is transmitted on the allocated radio resource within a UL grant. When all delay-critical data is transmitted and only non-delay critical data remains in the buffer, the LCH changes back to legacy LCP.

Proposal 2) Additional LCP is activated for the LCH when delay-critical data occurs. If there is no delay-critical data, the LCH changes back to legacy LCP.

It is necessary to discuss the specification impacts for transmitting data out of order. The delay-critical data is transmitted earlier than non-delay critical data transmission. This priority operation does not guarantee the sequential transmission and PDCP or RLC SDU order may be changed, resulting in a retransmission/recovery procedure. This is expected to have a specification impact because it affects functions such as data reassembly and re-ordering in the reception of PDCP or RLC.

Observation 4) When transmitting non-delay critical data on radio resources allocated as additional LCPs within the LCH, Priority mismatch between resource allocation and data transmission may occur, which results in unfairness in resource usage.

Proposal 3) RAN2 is expected to discuss specification impact when delay-critical data has a transmission priority to non-delay critical data and is transmitted earlier.

Resource allocation within UE

The Bj variable is used for resource allocation per LCH, and Bj is periodically calculated according to section 5.4.3.1. After LCH selection procedure is done, the procedure of allocating resources to LCH and transmitting data when Bj > 0 are followed. In the next step, the procedure for transmitting data according to LCH priority order is performed under the condition that there are remaining resources. In resource allocation, priority is not considered within LCH.

5.4.3.1 Logical Channel Prioritization 5.4.3.1.1 General … The following UE variable is used for the Logical channel prioritization procedure: - Bj which is maintained for each logical channel j. The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established. For each logical channel j, the MAC entity shall: 1> increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented; 1> if the value of Bj is greater than the bucket size (i.e. PBR × BSD): 2> set Bj to the bucket size. NOTE: The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP. … 5.4.3.1.3 Allocation of resources … The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

Radio resources are allocated to a LCH considering two priorities: additional LCP and legacy LCP. However, the changes above in Bj and resource allocation related to logical channel j do not reflect the characteristics of the additional LCP. Transmitting non-delay critical data over resources allocated to the additional LCP affects the calculation of Bj related to the legacy LCP. It reduces resource fairness among LCHs. A new Bja value and updated resource allocation scheme may be discussed to incorporate additional LCP and ensure resource fairness.

Proposal 4) RAN2 is expected to discuss how to update calculating Bj and resource allocation procedure of LCH j  for considering additional LCP.

3. Conclusion

In the discussion we made the following observations:

Observation 1) For delay-aware LCP enhancements, RAN2 has agreed that additional LCH priority for delay-critical data is introduced to a LCH.

Observation 2) A time threshold is configured, enabling the classification of data into delay-critical data based on the remaining time.

Observation 3) PDCP and RLC transmit data sequentially, and if the order is changed, data recovery operations are performed through status report messages, etc.

Observation 4) When transmitting non-delay critical data on radio resources allocated as additional LCPs within the LCH, Priority mismatch between resource allocation and data transmission may occur, which results in unfairness in resource usage.

We propose the followings:

Proposal 1) RAN2 confirms that an additional LCP is introduced to a LCH.

Proposal 2) Additional LCP is activated for the LCH when delay-critical data occurs. If there is no delay-critical data, the LCH changes back to legacy LCP.

Proposal 3) RAN2 is expected to discuss specification impact when delay-critical data has a transmission priority to non-delay critical data and is transmitted earlier.

Proposal 4) RAN2 is expected to discuss how to update calculating Bj and resource allocation procedure of LCH j  for considering additional LCP.

4. References

RAN2#127 meeting report

R2-2406202, “Report of 3GPP TSG RAN WG2 meeting #126,” ETSI MCC

R2-2404102, “Report of 3GPP TSG RAN WG2 meeting #125bis,” ETSI MCC