3GPP TSG-RAN WG2 #127-bis	Tdoc R2-2408911

Hefei, China, 14-18 October 2024

Agenda Item:	5.1.3.2

Source:	Ericsson

Title:	Missing NR-DC parameters branches and DCI format 2_3

Document for:	Discussion, Decision

# 1	Introduction

This contribution further discusses missing branches for UE capabilities under NR-DC parameters and DCI format 2_3 and related SRS-CarrierSwitching configuration and capability.

# 2	Discussion

## 2.1	Missing NR-DC parameters branches

CA capabilities applicable when the UE is configured with NR-DC are indicated under the NR-DC branch (CA-ParametersNRDC), which usually instantiates the same IE, and hence the same fields defined for CA (CA-ParametersNR). Currently, there are branches defined for CA that are missing in NR-DC. In 38.331, CA-ParametersNR-v1690 and CA-ParametersNR-v1740 were forgotten to be defined for NR-DC.

CA-ParametersNR-v1690 ::= SEQUENCE {

csi-ReportingCrossPUCCH-Grp-r16          SEQUENCE {

computationTimeForA-CSI-r16              ENUMERATED {sameAsNoCross, relaxed},

additionalSymbols-r16                    SEQUENCE {

scs-15kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-30kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-60kHz-additionalSymbols-r16          ENUMERATED {s14, s28, s56}       OPTIONAL,

scs-120kHz-additionalSymbols-r16         ENUMERATED {s14, s28, s56}       OPTIONAL

}                                                                             OPTIONAL,

sp-CSI-ReportingOnPUCCH-r16              ENUMERATED {supported}               OPTIONAL,

sp-CSI-ReportingOnPUSCH-r16              ENUMERATED {supported}               OPTIONAL,

carrierTypePairList-r16                  SEQUENCE (SIZE (1..maxCarrierTypePairList-r16)) OF CarrierTypePair-r16

}                                                                                 OPTIONAL

}

CA-ParametersNR-v1740 ::= SEQUENCE {

-- R1 33-5-1f: NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback

-- for SPS group-common PDSCH for multicast

nack-OnlyFeedbackForSPS-Multicast-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-1: PUCCH resource configuration for multicast feedback for dynamically scheduled multicast

singlePUCCH-ConfigForMulticast-r17                     ENUMERATED {supported}                         OPTIONAL

}

Hence, the UE cannot indicate support of the features in those branches when configured with NR-DC.

RAN2 missed to define CA-ParametersNR-v1690 and CA-ParametersNR-v1740 for NR-DC.

While this could be added as a non-critical extension to the corresponding versions of CA-ParametersNR, we think this can be addressed by simply including the missing branches for NR-DC in Rel-18, since the Rel-18 band combinations lists may anyway be extended. However, it should be noted that adding only the NR-DC branch would not work for all scenarios: i.e. this would be NBC if you consider a legacy UE that only includes the already-defined NR CA branches and consequently will never include the newly defined NR-DC branches, and thus a network that implements these new branches will consider this legacy UE as supporting the same features as indicated for NR CA also in case of NR-DC. To avoid NBC issues, the cleaner approach is to dummify the current NR CA fields and re-add them for both CA and DC. In this way, there should be no inter-operability issue since both NR-CA and NR-DC branches can be taken into account under the same extension. A TP is provided in the Annex I.

Support for both NR-CA and NR-DC is added in Rel-18 for the features defined in CA-ParametersNR-v1690 and CA-ParametersNR-v1740.

The legacy ca-ParametersNR-v1690 and ca-ParametersNR-v1740 fields are dummified.

## 2.2	DCI format 2_3

In RAN#127, the following was agreed:

SRS switching

R2-2406841	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-15	38.331	15.26.0	4893	-	F	NR_newRAT-Core

R2-2406842	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-16	38.331	16.17.0	4894	-	A	NR_newRAT-Core, TEI16

R2-2406843	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-17	38.331	17.9.0	4895	-	A	NR_newRAT-Core, TEI17

R2-2406844	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-18	38.331	18.2.0	4896	-	A	NR_newRAT-Core, TEI18

-	Ericsson and ZTE thinks there are many errors in this field description and they will send an LS. CATT wants to clarify this here now since it will help RAN1 discussions. Vivo agrees with CATTs intention. Nokia thinks something is wrong in the field description and want to fix it now. Huawei think we can remove something and refer to RAN1 specs instead.

RAN2 understands that the field description for typeA and typeB in SRS-CarrierSwitching are wrong and we will wait for input from RAN1.

RAN1 gave input on this topic in R1-2407534, where it was suggested that the description of SRS-CarrierSwitching should be updated so that this IE can be provided in two separate cases:

- SRS transmission on a PUSCH-less SCell

- SRS power control is not tied with PUSCH power control

For the first case, the UE support of the feature is clear from the capabilities below:

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

It is not crystal clear, however, which capabilities are used to indicate support of the second case. Support of DCI 2_3 is indicated by the capability field tpc-SRS-RNTI. While tpc-SRS-RNTI does not have a dependency on the srs-CarrierSwitch capability, the SRS-CarrierSwitching IE is used for this second case where the UE does not switch among carriers, and so UE support for fields of this IE that only relate to carrier switching is ambiguous at present. Since SRS-CarrierSwitching IE is revised, some clarification of UE capability for DCI 2_3 without carrier switching could be good to clarify what the UE should support in this case.

Although support of DCI 2_3 is indicated by support for tpc-SRS-RNTI and independent of the srs-CarrierSwitch capability, the use of srs-CarrierSwitch to configure DCI 2_3 based power control on a cell with PUSCH requires update of  tpc-SRS-RNTI capability.

Update tpc-SRS-RNTI capability to indicate support of SRS-CarrierSwitching configuration.

For the use of SRS-CarrierSwitching to configure the second case, it should also be clarified how to handle fields that are not applicable to this case (in yellow below):

#### –	SRS-CarrierSwitching

The IE SRS-CarrierSwitching is used to configure for SRS carrier switching when PUSCH is not configured and independent SRS power control from that of PUSCH.

SRS-CarrierSwitching information element

-- ASN1START

-- TAG-SRS-CARRIERSWITCHING-START

SRS-CarrierSwitching ::=            SEQUENCE {

srs-SwitchFromServCellIndex         INTEGER (0..31)                                                         OPTIONAL,   -- Need M

srs-SwitchFromCarrier               ENUMERATED {sUL, nUL},

srs-TPC-PDCCH-Group                 CHOICE {

typeA                               SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

typeB                               SRS-TPC-PDCCH-Config

}                                                                                                           OPTIONAL,   -- Need M

monitoringCells                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex               OPTIONAL,   -- Need M

...

}

SRS-TPC-PDCCH-Config ::=            SEQUENCE {

srs-CC-SetIndexlist                 SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex                                OPTIONAL    -- Need M

}

SRS-CC-SetIndex ::=                 SEQUENCE {

cc-SetIndex                         INTEGER (0..3)                                                          OPTIONAL,   -- Need M

cc-IndexInOneCC-Set                 INTEGER (0..7)                                                          OPTIONAL    -- Need M

}

-- TAG-SRS-CARRIERSWITCHING-STOP

-- ASN1STOP

For srs-SwitchFromServCellIndex, it is an optional field and thus could in principle be omitted. However, if this field is omitted, the UE may have no way to understand that the configuration being provided is not for SRS-carrier switching but rather for DCI format 2_3. Therefore, this field should be set to the SCellIndex of the scheduled cell (i.e. the one where SRS-CarrierSwitching is configured). In this way, it could be implicitly understood by the UE that this configuration is for DCI format 2_3 only.

For srs-SwitchFromCarrier, it is a mandatory field and thus the UE should ignore this field for independent SRS power control from that of PUSCH. This aspect should be clarified in the field description.

For independent SRS power control from that of PUSCH, srs-SwitchFromServCellIndex is set to the same value as the SCellIndex of the scheduled cell (i.e. the one where SRS-CarrierSwitching is configured).

For independent SRS power control from that of PUSCH, the UE ignores srs-SwitchFromCarrier.

For the field descriptions of typeA and typeB,  it should be clarified that those fields are also used in case of DCI format 2_3 only.

TypeA and typeB field descriptions are clarified to also be applicable for independent SRS power control from that of PUSCH.

A TP is provided in the Annex I and II capturing the proposed changes above. Note that Annex II also has an editorial correction to monitoringCells field description to change the reference from TS 38.213 clause 11.3 to clause 11.4.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 missed to define CA-ParametersNR-v1690 and CA-ParametersNR-v1740 for NR-DC.

Observation 2	Although support of DCI 2_3 is indicated by support for tpc-SRS-RNTI and independent of the srs-CarrierSwitch capability, the use of srs-CarrierSwitch to configure DCI 2_3 based power control on a cell with PUSCH requires update of  tpc-SRS-RNTI capability.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support for both NR-CA and NR-DC is added in Rel-18 for the features defined in CA-ParametersNR-v1690 and CA-ParametersNR-v1740.

Proposal 2	The legacy ca-ParametersNR-v1690 and ca-ParametersNR-v1740 fields are dummified.

Proposal 3	Update tpc-SRS-RNTI capability to indicate support of SRS-CarrierSwitching configuration.

Proposal 4	For independent SRS power control from that of PUSCH, srs-SwitchFromServCellIndex is set to the same value as the SCellIndex of the scheduled cell (i.e. the one where SRS-CarrierSwitching is configured).

Proposal 5	For independent SRS power control from that of PUSCH, the UE ignores srs-SwitchFromCarrier.

Proposal 6	TypeA and typeB field descriptions are clarified to also be applicable for independent SRS power control from that of PUSCH.

# Annex I (TP for 38.331 - Missing NR-DC parameters branches)

RF-Parameters ::=                                   SEQUENCE {

supportedBandListNR                                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList                        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter                           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540                  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested                          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550                  BandCombinationList-v1550                   OPTIONAL

]],

******omitted unchaged parameters*******

]],

[[

supportedBandCombinationList-v18xy                  BandCombinationList-v18xy                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v18xy   BandCombinationList-UplinkTxSwitch-v18xy    OPTIONAL]]

}

BandCombinationList-v18xy ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v18xy

BandCombinationList-UplinkTxSwitch-v18xy ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v18xy

BandCombination-v1690 ::=          SEQUENCE {

dummy              CA-ParametersNR-v1690                 OPTIONAL

}

BandCombination-v1740 ::=          SEQUENCE {

dummy              CA-ParametersNR-v1740                    OPTIONAL

}

BandCombination-UplinkTxSwitch-v18xy ::=    SEQUENCE {

bandCombination-v18xy                       BandCombination-v18xy              OPTIONAL

}

BandCombination-v18xy::=            SEQUENCE {

ca-ParametersNR-v1690             CA-ParametersNR-v1690                    OPTIONAL,

ca-ParametersNR-v1740             CA-ParametersNR-v1740                    OPTIONAL,

ca-ParametersNRDC-v18xy           CA-ParametersNRDC-v18xy                    OPTIONAL

}

CA-ParametersNRDC-v18xy ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1690                  CA-ParametersNR-v1690                        OPTIONAL,

ca-ParametersNR-ForDC-v1740                  CA-ParametersNR-v1740                        OPTIONAL

}

# Annex II (TP for 38.331/38.306 – DCI format 2_3)

## TP for 38.331

#### –	SRS-CarrierSwitching

The IE SRS-CarrierSwitching is used to configure SRS carrier switching when PUSCH is not configured or to configure independent SRS power control from that of PUSCH.

SRS-CarrierSwitching information element

-- ASN1START

-- TAG-SRS-CARRIERSWITCHING-START

SRS-CarrierSwitching ::=            SEQUENCE {

srs-SwitchFromServCellIndex         INTEGER (0..31)                                                         OPTIONAL,   -- Need M

srs-SwitchFromCarrier               ENUMERATED {sUL, nUL},

srs-TPC-PDCCH-Group                 CHOICE {

typeA                               SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

typeB                               SRS-TPC-PDCCH-Config

}                                                                                                           OPTIONAL,   -- Need M

monitoringCells                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex               OPTIONAL,   -- Need M

...

}

SRS-TPC-PDCCH-Config ::=            SEQUENCE {

srs-CC-SetIndexlist                 SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex                                OPTIONAL    -- Need M

}

SRS-CC-SetIndex ::=                 SEQUENCE {

cc-SetIndex                         INTEGER (0..3)                                                          OPTIONAL,   -- Need M

cc-IndexInOneCC-Set                 INTEGER (0..7)                                                          OPTIONAL    -- Need M

}

-- TAG-SRS-CARRIERSWITCHING-STOP

-- ASN1STOP

SRS-CC-SetIndex field descriptions

cc-IndexInOneCC-Set Indicates the CC index in one CC set for Type A (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA.

cc-SetIndex Indicates the CC set index for Type A associated (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA. The network does not configure this field to 3 in this release of specification.

SRS-CarrierSwitching field descriptions

monitoringCells A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4).

srs-SwitchFromCarrier The UE ignores this field in case of independent SRS power control from that of PUSCH.

srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less SCell. During SRS transmission on a PUSCH-less SCell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less SCell to transmit SRS. (see TS 38.214 [19], clause 6.2.1.3). If this field is set to the ScellIndex value of the serving cell in which PUSCH transmission and the SRS-CarrierSwitching field is configured, it indicates the configuration of independent SRS power control from that of PUSCH and that SRS carrier switching (TS 38.214 [19], clause 6.2.1.3) is not used.

srs-TPC-PDCCH-Group Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any.

typeA Type A trigger configuration for SRS transmission on a PUSCH-less SCell or independent SRS power control from that of PUSCH (see TS 38.213 [13], clause 11.4). In this release, the network configures at most one entry (the first entry) of typeA, and the first entry corresponds to the serving cell in which the SRS-CarrierSwitching field is configured. SRS carrier switching to SUL carrier is not supported in this version of the specification.

typeB Type B trigger configuration for SRS transmission on a PUSCH-less SCell or independent SRS power control from that of PUSCH (see TS 38.213 [13], clause 11.4).

SRS-TPC-PDCCH-Config field descriptions

srs-CC-SetIndexlist A list of pairs of [cc-SetIndex; cc-IndexInOneCC-Set] (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network does not configure this field for typeB.

## TP for 38.306

| tpc-SRS-RNTI Indicates whether the UE supports group DCI message based on TPC-SRS-RNTI for TPC commands for SRS and related configuration in the SRS-CarrierSwitching IE as specified in TS 38.331 [9]. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |