3GPP TSG-RAN WG2 Meeting #127bis	R2-2408916

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.7.4.1

Source:	Ericsson

Title:	LCP enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to LCP:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

In this contribution we continue the discussion on the Logical Channel priority handling. Furthermore, we provide a TP for how the enhanced LCP mechanism can be modelled.

# 2	Discussion on LCP enhancements

## 2.1 LCP enhancement considerations

When considering LCP enhancements to improve the prioritization of data with short remaining time there are some important aspects that should be taken into consideration:

- Certain data will always require highest priority, e.g. pose traffic, and should always be prioritized first, even when there exist other time critical data in the buffers (the negative impact on XR capacity by not doing such prioritization can be shown through system simulations).

- In current LCP mechanism not all time critical data may be given priority and instead other (less time-critical) data in other LCH will be prioritized.

- Packets may arrive at different times and PDU Sets may even become interleaved in the LCH queue, i.e. the remaining time of packets is not always strictly following the place in the queue. It is even possible that multiple different traffic flows are mapped to the same LCH, potentially increasing the issue with interleaved PDU Sets even more.

- It was agreed that prioritization inside a LCH should not be considered in RAN2 discussion but for completeness it should be noted that above makes it also possible to have both delay critical and non-delay critical packets in the same LCH. Even though configuration can avoid such cases by restricting to only map flows with similar characteristics and requirements to the same LCH, that is an implementation aspect. In the worst case scenario, a LCH can contain both delay critical and non-delay critical packets. Then it is possible that the non-delay critical packets are ahead in the queue (selected by transmission of a UE) and will get the benefit of increased priority for the LCH, see some examples in figure 1.

Figure 1: Examples of the worst case scenario where delay critical and non-delay critical packets can be in the same LCHs

If we only want delay critical packets to benefit from increased priority for an LCH, then delay critical and non-delay critical packets can be mapped into separate LCHs as shown in figure 2.

Figure 2: Delay critical and non-delay critical packets in same or separate LCHs

Another alternative is that the UE itself chooses to transmit delay critical data from a LCH before non-delay critical data, even if the non-delay critical data arrived earlier in the queue.. However, this is something that is left for UE implementation.

The above aspects and their impacts on the XR traffic KPI should be given thorough consideration when designing the new LCP solution.

To avoid negative XR capacity impacts a delay-aware LCP solution should allow protecting transmission of selected data, e.g. pose could be configured to always have a higher priority than the time critical data.

A delay-aware LCP solution should take into consideration that different PDU Sets and traffic flows may be interleaved in a LCH.

There can be both delay critical and non-delay critical packets in an LCH. However, how to handle intra-LCH prioritization is left to UE implementation.

If UE selects non-delay critical packets for transmission the delay critical data may be delayed.

## 2.2 Delay-aware LCP enhancement

A simple solution with low UE complexity is to adjust the priority of selected LCH based on the remaining time of the data in the LCH queue, i.e. introduce dynamic priority of selected LCH. This priority adjustment can be done in advance in the UE, i.e. before the actual selection of LCH and resource allocation and thus has low impact on the UE processing during the time critical procedures. The solution can work in the way that as soon as any data in the LCH pass the configured remaining time threshold, i.e. data becomes time critical, the scheduling priority for the LCH can be adjusted to a configured higher priority value. This priority value is then used in the legacy LCP mechanism when selecting between the LCHs based on the priority values. The priority values and time threshold should be configured for each LCH by the network which allows for the appropriate prioritization. When there is no more time critical data the priority of the LCH is reverted to the original priority value. The time threshold should be different than the threshold used in the DSR, since the threshold to trigger reporting of delay critical data in a DSR is used for different purpose. Early reporting of delay critical data through the DSR can be useful for the network to do appropriate scheduling and not necessarily to schedule the delay critical data in the UE directly. There may for instance still be time for other data to be served before the delay critical data. Furthermore, DSR will be enhanced with multiple reporting thresholds in Rel19, which is another indication that the DSR thresholds have a different purpose than the one used in the delay-aware LCP enhancement.

Lastly to make sure that there is timely transmission of the time critical data another stop condition should be introduced in the delay-aware LCP solution. This can be done by applying a new counter value to keep track of the transmission of the time critical data, so that only when all of the time critical data is transmitted from the LCH the resource allocation move on to another lower priority LCH. This solution assures that the time critical data is being included in the transmission before any other low important data.

The described delay-aware LCP solution make it possible to remedy the aspects brought up in 3.1, as with proper implementation and configuration the time critical data is prioritized and transmitted as quickly as possible but not at the expense of any higher priority data in the UE buffer. We thus propose following for the delay-aware LCP enhancements.

A delay-aware LCP solution can configure a LCH with a time threshold to determine if there is time critical data in the buffer. This threshold is different than the thresholds used in the DSR reporting.

A LCH can be configured with an alternative priority value to be applied when there is time critical data in the buffer.

A delay-aware LCP solution uses new counter variables to keep track of the scheduling of the time critical data.

A text proposal implementing above delay-aware LCP solution in TS 38.321 specification can be found in Appendix 1.

Implement the TP provided in Appendix 1.

## 2.3 Configuration of enhanced LCP

Each LCH that contains time critical data will need to be configured via RRC with an alternative priority value and a threshold for remaining time. This can be done in the same IE that is used for LCP configuration today, LogicalChannelConfig.

For each LCH that contains time critical data, it shall be possible to configure via RRC an alternative priority value and a remaining time threshold in the IE LogicalChannelConfig.

Some data, e.g. pose and signallling should always have higher priority than time critical data, even if the time critical data is close to be discarded. This can be solved by the network via configuration of the default priority levels and the alternative priority levels. Since we can’t predict exactly which data that need to be protected we shouldn’t specify how these priority levels will be configured, it can be left for network implementation.

How to configure priority values so that e.g. pose and signalling always have higher priority than time critical data can be left for network implementation.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	To avoid negative XR capacity impacts a delay-aware LCP solution should allow protecting transmission of selected data, e.g. pose could be configured to always have a higher priority than the time critical data.

Observation 2	A delay-aware LCP solution should take into consideration that different PDU Sets and traffic flows may be interleaved in a LCH.

Observation 3	There can be both delay critical and non-delay critical packets in an LCH. However, how to handle intra-LCH prioritization is left to UE implementation.

Observation 4	If UE selects non-delay critical packets for transmission the delay critical data may be delayed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A delay-aware LCP solution can configure a LCH with a time threshold to determine if there is time critical data in the buffer. This threshold is different than the thresholds used in the DSR reporting.

Proposal 2	A LCH can be configured with an alternative priority value to be applied when there is time critical data in the buffer.

Proposal 3	A delay-aware LCP solution uses new counter variables to keep track of the scheduling of the time critical data.

Proposal 4	Implement the TP provided in Appendix 1.

Proposal 5	For each LCH that contains time critical data, it shall be possible to configure via RRC an alternative priority value and a remaining time threshold in the IE LogicalChannelConfig.

Proposal 6	How to configure priority values so that e.g. pose and signalling always have higher priority than time critical data can be left for network implementation.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024

# Appendix 1

Changes to specification TS 38.321, additions highlighted in yellow:

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

-	timeCriticalPriority an alternative priority value.

-	timeCriticalTimeThreshold which set the remainingTimeThreshold (RETT).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

-	Rj which is maintained for each logical channel j.

The MAC entity shall initialize Bj and Rj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

1>	if the value of D is less than or equal to RETT, where D is the shortest remaining time until discard for any MAC SDU in logical channel j:

2>	set Rj to the size in bytes for MAC SDUs with remaining time less or equal to D.

2>	adjust priority of logical channel j to timeCriticalPriority

- if the value of D is greater than RETT, where D is the shortest remaining time until discard for any MAC SDU in logical channel j:

2>	set Rj to zero.

2>	adjust priority of logical channel j to priority.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or Rj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2> decrement Rj by the total size of MAC SDUs with remaining time less or equal to D served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.