3GPP TSG-RAN WG2 Meeting #127bis	R2-2408925

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.3.2

Source:	InterDigital Inc.

Title:	Simulation results for RRM measurement predictions

Document for:	Discussion

# 1. Introduction

In RAN2-126 and RAN2-127, several agreements were made regarding the simulation assumption for the RRM measurement predictions [1]:

Table 1: Evaluation Scenario Combinations

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In addition, three RRM prediction sub-cases were introduced [2]:

- Sub-case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Sub-case 2: To directly predict cell level results based on cell level results

- Sub-case 3: To directly predict cell level results based on beam level results

In this contribution, we provide some initial simulation results for the highlighted scenarios in the table above (Case 2, Case 3, and Case 4), focusing on Sub-case 1 and Sub-case 3 as indirect and direct approaches for predicting L3 cell-level measurements.

# 2. Discussion

### 2.1 Intra-cell intra-frequency FR1 to FR1 temporal domain case B

To generate data, we consider a sampling period of 40 ms for L1 beam-level RSRPs. Then, non-sliding L1 filtering is applied to obtain L1 filtered beam-level RSRPs. With L1 filtering window length that is equal to 5, the sampling period of non-sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 200 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 1: Non-sliding L1/L3 Filtering

We study 50% measurement reduction rate. For predicting cell-level or beam-level RSRPs at time instance T, we use input from 3 previous time instances, which are T-5, T-3, and T-1. It is necessary to use input from multiple time instances to enable the AIML model to learn information about UE trajectory. The RSRPs at time instances T-5, T-3, and T-1 are all measured RSRPs.

Figure 2: Temporal-domain Case B Observation Windows (OWs in yellow) and Prediction Windows (PW in green) Pattern with 50% Measurement Reduction Rate

In Case 2, we consider predicting L3 cell-level RSRPs directly and indirectly according to Sub-cases 1 and 3 where:

- Sub-case 1 (Indirect Approach): L1 filtered beam-level RSRPs from 3 time instances (T-5, T-3, and T-1) are input to AIML model to predict L1 filtered beam-level RSRPs at time instance T. Then, L3 cell-level RSRP at time instance T is derived using the L1 beam-level RSRPs at time instance T that were obtained from the AIML model.

- Sub-case 3 (Direct Approach): L1 filtered beam-level RSRPs from 3 time instances (T-5, T-3, and T-1) are input to AIML model to predict L3 cell-level RSRP at time instance T directly.

Simulation parameters of Case 2 are listed in Table A.1 [2]-[4]. Evaluation results are shown in Table 2.

Table 2: Evaluation results for intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameters | Sub-case 1 (Indirect), 30 Km/h | Sub-case 3 (Direct), 30 Km/h | Sub-case 1 (Indirect), 60 Km/h | Sub-case 3 (Direct), 60 Km/h |
| --- | --- | --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 | Option 3 | Option 3 |
| UE speed | 30 Km/h | 30 Km/h | 60 Km/h | 60 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A | N/A | N/A | N/A |
| Measurement reduction rate | 50% | 50% | 50% | 50% |
| Observation window | 800 ms | 800 ms | 800 ms | 800 ms |
| Prediction window | 200 ms | 200 ms | 200 ms | 200 ms |
| Any other parameters | N/A | N/A | N/A | N/A |
| Number of training examples | 14700 | 14700 | 14669 | 14669 |
| Number of validation examples | 2100 | 2100 | 2095 | 2095 |
| Number of testing examples | 4200 | 4200 | 4190 | 4190 |
| AIML model input | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 |
| AIML model output | L1 filtered beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) | L3 cell-level RSRPs at 1 time instance T | L1 beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) | L3 cell-level RSRPs at 1 time instance T |
| AIML model type | ResNet | ResNet | ResNet | ResNet |
| AIML model number of parameters | 1.51 M | 1.51 M | 1.51 M | 1.51 M |
| AIML-based Average L3 cell-level RSRP difference (dB) | 0.0149 dB | 0.0199 dB | 0.0346 dB | 0.0408 dB |
| AIML-based Average L1 beam-level RSRP difference (dB) | 0.0355 dB | N/A | 0.0791 dB | N/A |
| Baseline (Sample-and-Hold) Average L3 cell-level RSRP difference (dB) | 0.1119 dB | 0.1119 dB | 0.2263 dB | 0.2263 dB |

We notice that indirect L3 cell-level RSRP prediction via an AIML model that predicts L1 beam-level RSRPs (i.e., Sub-case 1) achieves a better performance than direct prediction (i.e., Sub-case 3). This is because the direct AIML model is required to predict a filtered L3 cell-level measurement given partial previous history (i.e., information from only 3 previous time instances). The performance of the direct model may be enhanced by providing either L1 beam-level RSRP input from more previous instances or previous measured and/or predicted L3 cell-level measurements.

We also notice that the AIML model achieve a lower prediction error as the UE speed becomes lower. In addition, AIML-based prediction outperforms baseline (sample-and-hold), in which L1-filtered beam-level RSRPs from latest measurement time instance before prediction are maintained as predicted measurements, by achieving less average prediction error.

Observation 1: For temporal domain case B, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 2: For temporal domain case B, the indirect prediction approach outperforms the direct prediction approach when given the same input (i.e., only L1 filtered beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 3: For temporal domain case B, AIML model prediction accuracy is higher than baseline (sample-and-hold)

Proposal 1: RAN2 to study the impact of how the measured RSRPs and predicted RSRPs are combined in cell level measurement derivation.

Proposal 2: RAN2 to study the impact of different input window lengths in temporal domain case B.

### 2.2 Intra-cell intra-frequency FR2 to FR2 temporal domain case A

To generate data, we consider a sampling period of 20 ms for L1 beam-level RSRPs. To measure RSRPs for 4 Rx beams, a measurement period of 80 ms (i.e., 4 sampling periods) is required. Then, sliding L1 filtering (with window length that is equal to 5) is applied to obtain L1 filtered beam-level RSRPs., the measurement period of sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 80 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 3: Sliding L1/L3 Filtering

In Case 4, we consider predicting L3 cell-level RSRPs directly and indirectly according to Sub-cases 1 and 3 where:

- Sub-case 1 (Indirect Approach): L1 filtered beam-level RSRPs from OW time instances are input to AIML model to predict L1 filtered beam-level RSRPs of PW time instances. Then, L3 cell-level RSRP of PW time instances are evaluated using the L1 filtered beam-level RSRPs of PW time instances that were obtained from the AIML model.

- Sub-case 3 (Direct Approach): L1 filtered beam-level RSRPs from OW time instances are input to AIML model to predict L3 cell-level RSRP of PW time instances directly.

Figure 4: Temporal-domain Case A Observation Window (OW) and Prediction Window (PW) Pattern

Simulation parameters of Case 4 are listed in Table A.2 [2]-[4]. Evaluation results are shown in Table 3 and following figures.

Table 3: Evaluation results for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameters | Sub-case 1, 60 Km/h |  |  |  | Sub-case 3, 60 Km/h |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE trajectory option [2] | Option 1 |  |  |  | Option 1 |  |  |  |
| UE trajectory boundary processing option [2] | Option 3 |  |  |  | Option 3 |  |  |  |
| UE speed | 60 Km/h |  |  |  | 60 Km/h |  |  |  |
| Inter-frequency correlation assumption (yes or no) | N/A |  |  |  | N/A |  |  |  |
| Measurement reduction rate | N/A |  |  |  | N/A |  |  |  |
| Observation window (OW) | 400 ms | 400 ms | 800 ms | 800 ms | 400 ms | 400 ms | 800 ms | 800 ms |
| Prediction window (PW) | 400 ms | 800 ms | 400 ms | 800 ms | 400 ms | 800 ms | 400 ms | 800 ms |
| Any other parameters | N/A |  |  |  | N/A |  |  |  |
| Number of training examples | 14581 |  |  |  | 14581 |  |  |  |
| Number of validation examples | 2083 |  |  |  | 2083 |  |  |  |
| Number of testing examples | 4164 |  |  |  | 4164 |  |  |  |
| AIML model input | L1 filtered beam-level RSRPs from OW time instances |  |  |  | L1 filtered beam-level RSRPs from OW time instances |  |  |  |
| AIML model output | L1 filtered beam-level RSRPs of PW time instances |  |  |  | L3 Cell-level RSRPs of PW time instances |  |  |  |
| AIML model type | ResNet |  |  |  | ResNet |  |  |  |
| AIML model complexity | Refer to Table 4 |  |  |  | Refer to Table 4 |  |  |  |
| AIML-based Average L3 cell-level RSRP difference (dB) | Refer to Table 5 and Figures |  |  |  | Refer to Table 5 and Figures |  |  |  |
| AIML-based Average L1 beam-level RSRP difference (dB) | Refer to Table 6 |  |  |  | N/A |  |  |  |
| Baseline (Sample-and-Hold) Average L3 cell-level RSRP difference (dB) | Refer to Table 7 and Figures |  |  |  | Refer to Table 7 and Figures |  |  |  |

Table 4: AIML model complexity summary for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Sub-case | UE Speed | OW, PW (ms) | AIML model number of parameters | AIML model number of FLOPs |
| --- | --- | --- | --- | --- |
| 1 | 60 Km/h | 400, 400 | 0.413 M | 25.96 M |
| 1 | 60 Km/h | 400, 800 | 0.415 M | 26.04 M |
| 1 | 60 Km/h | 800, 400 | 0.416 M | 26.14 M |
| 1 | 60 Km/h | 800, 800 | 0.417 M | 26.22 M |
| 3 | 60 Km/h | 400, 400 | 0.575 M | 25.96 M |
| 3 | 60 Km/h | 400, 800 | 0.656 M | 26.04 M |
| 3 | 60 Km/h | 800, 400 | 0.658 M | 26.14 M |
| 3 | 60 Km/h | 800, 800 | 0.669 M | 21.77 M |

Table 5: AIML model prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A (UE Speed = 60 Km/h)

| Sub-case | OW, PW (ms) | OW, PW (Measurement periods) | AIML-based Average L3 cell-level RSRP difference (dB) |
| --- | --- | --- | --- |
| 1 | 400, 400 | 5, 5 | [0.010, 0.018, 0.035, 0.064, 0.109] dB |
| 1 | 400, 800 | 5, 10 | [0.035, 0.045, 0.060, 0.082, 0.116, 0.162, 0.223, 0.297, 0.387, 0.492] dB |
| 1 | 800, 400 | 10, 5 | [0.018, 0.024, 0.041, 0.072, 0.120] dB |
| 1 | 800, 800 | 10, 10 | [0.023, 0.027, 0.039, 0.062, 0.098, 0.147, 0.208, 0.283, 0.372, 0.470] dB |
| 3 | 400, 400 | 5, 5 | [0.341, 0.357, 0.387, 0.440, 0.535] dB |
| 3 | 400, 800 | 5, 10 | [0.364. 0.377, 0.404, 0.440, 0.494, 0.561, 0.638, 0.724, 0.828, 0.953] dB |
| 3 | 800, 400 | 10, 5 | [0.387, 0.415, 0.459, 0.527, 0.628] dB |
| 3 | 800, 800 | 10, 10 | [0.379, 0.400, 0.430, 0.476, 0.538, 0.611, 0.696, 0.794, 0.909, 1.041] dB |

Table 6: AIML model beam-level prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A, Sub-case 1 (UE Speed = 60 Km/h)

| Sub-case | OW, PW (ms) | OW, PW (Measurement periods) | AIML-based Average L1 beam-level RSRP difference (dB) |
| --- | --- | --- | --- |
| 1 | 400, 400 | 5, 5 | [0.044, 0.102, 0.192, 0.314, 0.476] dB |
| 1 | 400, 800 | 5, 10 | [0.076, 0.131, 0.217, 0.332, 0.474, 0.634, 0.811, 1.001, 1.204, 1.427] dB |
| 1 | 800, 400 | 10, 5 | [0.047, 0.105, 0.197, 0.330, 0.504] dB |
| 1 | 800, 800 | 10, 10 | [0.061, 0.107, 0.182, 0.290, 0.430, 0.588, 0.763, 0.952, 1.155, 1.378] dB |

Table 7: AIML-based vs. baseline (Sample-and-Hold) prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A (UE Speed = 60 Km/h)

| Sub-case | OW, PW (ms) | OW, PW (Measurement periods) | AIML-based Average L3 cell-level RSRP difference (dB) | Baseline (Sample-and-Hold) Average L3 cell-level RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| 1 | 800, 800 | 10, 10 | [0.023, 0.027, 0.039, 0.062, 0.098, 0.147, 0.208, 0.283, 0.372, 0.470] dB | [0.189, 0.461, 0.765, 1.069, 1.373, 1.672, 1.971, 2.271, 2.578, 2.887] dB |
| 3 | 800, 800 | 10, 10 | [0.379, 0.400, 0.430, 0.476, 0.538, 0.611, 0.696, 0.794, 0.909, 1.041] dB |  |

Figure 5: AIML model prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A (UE Speed = 60 Km/h)

We notice that indirect L3 cell-level RSRP prediction via an AIML model that predicts L1 beam-level RSRPs (i.e., Sub-case 1) achieves a better performance than direct prediction (i.e., Sub-case 3). This is because the direct AIML model is required to learn both measurement prediction and selection/consolidation to predict a filtered L3 cell-level measurement which may be challenging for AIML regression models.

In addition, AIML-based direct and indirect prediction outperform baseline (sample-and-hold), in which L1-filtered beam-level RSRPs from latest measurement time instance before prediction are maintained as predicted measurements, by achieving less average prediction error.

To study the impact of changing observation window, we compare the AIML-based prediction for OW and PW pairs (400 ms, 400 ms) and (800 ms, 800ms) since both have 1:1 ratio between OW and PW with different window lengths. Using shorter OW at 60 Km/h, with 1:1 ratio between OW and PW, results in better prediction performance since it seems PW measurements can be better inferred from more recent history at such UE speed, i.e., older OW instances do not help enhance the prediction performance.

We can also compare the AIML-based prediction for OW and PW pairs (400 ms, 800 ms) and (800 ms, 800ms) since both have same PW length, but with 1:2 and 1:1 ratio between OW and PW, respectively. We notice that the performance gap between both case is small (0.22 dB and 0.87 dB for Sub-cases 1 and 3, respectively) which indicates that AIML-based prediction did not gain much advantage by having a longer OW for a constant PW.

To study the impact of changing prediction window, we compare the AIML-based prediction for OW and PW pairs (400 ms, 400 ms) and (400 ms, 800ms) since both have same OW length, but with 1:1 and 1:2 ratio between OW and PW, respectively. Increasing the length of PW results in a minor increase in prediction error (~0.4 dB), however, it results in less measurement overhead. It is expected that the prediction error gap shall increase with higher UE speeds.

Observation 4: For temporal domain case A, the indirect prediction approach outperforms the direct prediction approach when given the same input (i.e., only L1 filtered beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 5: For temporal domain case A, AIML model prediction accuracy is higher than baseline (sample-and-hold)

Observation 6: For temporal domain case A, using shorter OW at 60 Km/h for a fixed ratio between OW and PW results in better prediction performance.

Observation 7: For temporal domain case A, at 60 Km/h, longer OW could lead to worse prediction (for a given PW).

Observation 8: For temporal domain case A, at 60 Km/h, increasing the length of PW results in a minor increase in prediction error (~0.4 dB), however, it results in less measurement overhead.

Proposal 3: RAN2 to study the impact of prediction performance requirement on the length of prediction window at different UE speeds in temporal domain case A.

### 2.3 Inter-cell inter-frequency FR1 to FR1 frequency domain

To generate data, we consider a sampling period of 40 ms for L1 beam-level RSRPs. Then, non-sliding L1 filtering is applied to obtain L1 filtered beam-level RSRPs. With L1 filtering window length that is equal to 5, the sampling period of non-sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 200 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

In Case 3, we consider predicting L3 cell-level RSRPs directly according to Sub-case 3 where:

- Sub-case 3 (Direct Approach): L1 filtered beam-level RSRPs from OW time instances are input to AIML model to predict L3 cell-level RSRP at PW time instances directly.

Simulation parameters of Case 3 are listed in Table A.1 [2]-[4]. Evaluation results are shown in Table 8.

Table 8: Evaluation results for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameters | Sub-case 3, 4 GHz 2 GHz | Sub-case 3, 2 GHz 4 GHz |
| --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 |
| UE speed | 30 Km/h | 30 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A | N/A |
| Measurement reduction rate | N/A | N/A |
| Observation window (OW) | 1000 ms | 1000 ms |
| Prediction window (PW) | 1000 ms | 1000 ms |
| Any other parameters | N/A | N/A |
| Number of training examples | 14700 | 14581 |
| Number of validation examples | 2100 | 2100 |
| Number of testing examples | 4200 | 4200 |
| AIML model input | L1 filtered beam-level RSRPs from OW time instances | L1 filtered beam-level RSRPs from OW time instances |
| AIML model output | L3 Cell-level RSRPs of PW time instances | L3 Cell-level RSRPs of PW time instances |
| AIML model type | ResNet | ResNet |
| AIML model number of parameters | 1.84 M | 1.84 M |
| AIML-based Average L3 cell-level RSRP difference (dB) | 0.8609 dB | 0.9234 dB |

We notice that indirect L3 cell-level RSRP prediction via an AIML model that predicts L1 beam-level RSRPs (i.e., Sub-case 1) achieves a better performance than direct prediction (i.e., Sub-case 3). This is because the direct AIML model is required to predict a filtered L3 cell-level measurement given partial previous history (i.e., information from only 3 previous time instances). The performance of the direct model may be enhanced by providing either L1 beam-level RSRP input from more previous instances or previous measured and/or predicted L3 cell-level measurements.

We also notice that the AIML model achieve a lower prediction error as the UE speed becomes lower. In addition, AIML-based prediction outperforms baseline (sample-and-hold), in which L1-filtered beam-level RSRPs from latest measurement time instance before prediction are maintained as predicted measurements, by achieving less average prediction error.

Observation 9: Using measurements from a higher frequency (e.g., 4 GHz) as input, to predict measurements at a lower frequency (e.g., 2 GHz) results in a slightly better performance than the other way.

# 3. Conclusion

In this contribution, initial simulation results for the RRM measurement prediction use case were discussed and the following observations and proposals were made:

Observation 1: For temporal domain case B, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 2: For temporal domain case B, the indirect prediction approach outperforms the direct prediction approach when given the same input (i.e., only L1 beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 3: For temporal domain case B, AIML model prediction accuracy is higher than baseline (sample-and-hold)

Observation 4: For temporal domain case A, the indirect prediction approach outperforms the direct prediction approach when given the same input (i.e., only L1 filtered beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 5: For temporal domain case A, AIML model prediction accuracy is higher than baseline (sample-and-hold)

Observation 6: For temporal domain case A, using shorter OW at 60 Km/h for a fixed ratio between OW and PW results in better prediction performance.

Observation 7: For temporal domain case A, at 60 Km/h, longer OW could lead to worse prediction (for a given PW).

Observation 8: For temporal domain case A, at 60 Km/h, increasing the length of PW results in a minor increase in prediction error (~0.4 dB), however, it results in less measurement overhead.

Observation 9: Using measurements from a higher frequency (e.g., 4 GHz) as input, to predict measurements at a lower frequency (e.g., 2 GHz) results in a slightly better performance than the other way.

Proposal 1: RAN2 to study the impact of how the measured RSRPs and predicted RSRPs are combined in cell level measurement derivation.

Proposal 2: RAN2 to study the impact of different input window lengths in temporal domain case B.

Proposal 3: Study the impact of prediction performance requirement on the length of prediction window at different UE speeds in temporal domain case A

# 4. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	R2-2404955: Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO)

[3]	R2-2406095: Summary of [POST126][031][AIMob] Simulations

[4]	R2-2497849: Summary of [POST127][030][AIMob] RRM Simulation Assumptions

[5]	TR 38.901: Study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901 version 16.1.0 Release 16)

# Appendix

### A.1 Simulation assumptions of Intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m |
| Channel model | According to TR 38.901 [5] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 30 Km/h, 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [5] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 4 Tx Beams (4 Azimuth angles x 1 Elevation angle) 1 Rx Beam (1 Azimuth angle x 1 Elevation angle) |
| L1-Filtering: Method | Non-sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 1 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |

### A.2 Simulation assumptions of Intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 – 30 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 200 m |
| Channel model | According to TR 38.901 [5] |
| System BW | 80 MHz |
| Sub-carrier Spacing | 120 kHz |
| UE Speed | 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (4,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | 2 panels (left, right) |
| BS Tx Power | 40 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [5] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 16 Tx Beams (8 Azimuth angles x 2 Elevation angles) 4 Rx Beams (4 Azimuth angles x 1 Elevation angle) |
| L1-Filtering: Method | Sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 3 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |

### A.3 Simulation assumptions of Inter-cell inter-frequency FR1 to FR1 frequency domain

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz FR1 – 2 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m |
| Channel model | According to TR 38.901 [5] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 30 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [5] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 4 Tx Beams (4 Azimuth angles x 1 Elevation angle) 1 Rx Beam (1 Azimuth angle x 1 Elevation angle) |
| L1-Filtering: Method | Non-sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 1 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |