3GPP TSG RAN WG2 #127bis		 											                        R2-2408927

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.2.4

Source:	ETRI

Title:	Further discussions on A-IoT random access

Document for:	Discussion and Decision

Introduction

This contribution discusses details on random access procedure for A-IoT, including 3-step/2-step CBRA and CFRA.

Discussions

Further discussion 3-step CBRA

Regarding the 3-step CBRA, RAN2 had reached the following agreements in RAN2#127 meeting [1].

For 3-step CBRA Support fixed random ID size is 16 bit. The ID is randomly generated. Failure/success indication of D2R will be studied. FFS if it would be implicit or explicit and for which use case it is needed. FFS whether it is applied only to some cases.

Further discussions are required to evaluate the necessity of failure/success indication and to explore suitable mechanisms for indicating these situations.

Failure/success indication of msg1

In the 3-step CBRA, the success of msg1 transmission can be determined by whether msg2 is received. Specifically, if the device does not receive msg2, which contains the random number sent in msg1, it recognizes that msg1 transmission has failed. The failure of msg1 transmission can occur due to collisions among multiple devices or transmission errors on the D2R link.

Observation 1: The success of msg1 transmission can be determined by the reception of msg2, which contains the same random number.

The device requires a mechanism to determine whether msg1 has been successfully received after its transmission. In the 3-step CBRA, multiple devices can transmit msg1 during the same access occasion, leading to potential collisions, which prevents the reader from identifying the specific device that transmitted msg1. As a result, the reader is unable to transmit the success or failure of msg1 reception to the device by a message.

Observation 2: The success or failure of msg1 reception cannot be delivered by a message.

To determine whether msg1 was successfully transmitted, the device waits for msg2 for a certain period after sending msg1. If msg2 is not received within that time, or if the content of msg2 does not contain the random number sent by the device, it can be concluded that msg1 transmission has failed. Therefore, the success or failure of msg1 transmission should be determined using a timer mechanism.

Proposal 1: The determination of msg1 transmission success should be based on a timer mechanism.

Failure/success indication of msg3

In the Inventory only case, since there is no data transmission following msg3, there is no additional procedure to verify the success of the Device ID transmission in msg3. For Inventory + Command, the success of msg3 transmission can be determined by the receipt of additional data for the command. In other words, if a command message is received after msg3 is transmitted, it can be assumed that msg3 was successfully received. In the command only case, random access is not required, so there is no need to determine the success of msg3 transmission.

Thus, as mentioned earlier, the indication of the success or failure of msg3 transmission can differ depending on the use case. Therefore, RAN2 needs to discuss whether to apply different mechanism for indicating the success or failure of D2R transmission based on the use case or to adopt a consistent approach across all scenarios.

Observation 3: The success or failure of msg3 transmission can be evaluated differently depending on the use case.

Proposal 2: RAN2 to discuss whether to apply different methods for indicating the success or failure of D2R transmission based on each use case or to adopt a consistent approach.

In the Inventory only use case, as there are no subsequent message procedures, the following approaches can be considered to evaluate the success of msg3 transmission:

Explicit mechanism with response(Response message for msg3 reception): This approach involves defining an AS-level message to respond to msg3 reception. It may include sending an acknowledgment (ACK) when msg3 is successfully received or a negative acknowledgment (NACK) when msg3 is not received. In this method, an ACK is sent upon successful receipt of msg3, while a NACK is sent if the reader fails to receive msg3.

Explicit mechanism with retransmission(Retransmission of msg2): If the reader does not receive msg3 within a certain time after sending msg2, it can retransmit msg2. Upon receiving the retransmitted msg2, the device will resend msg3.

Implicit mechanism(Reinitiating the inventory service): If msg3 is not received within a specified period after the transmission of msg2, the reader can request the device to reinitiate the inventory service via a paging message. In this case, the device will retransmit msg1 in response to the reader's request. If the device does not receive a request to reinitiate the inventory service, it can assume that msg3 was successfully received by the reader.

As mentioned earlier, the method for indicating the success or failure of msg3 transmission to the device can either apply different approaches depending on the use case or define a new AS-layer message that would use a consistent method regardless of the use case. However, since A-IoT services generally have low time constraints and are mostly best-effort in nature, it may be more practical to verify the success of msg3 transmission by re-executing the reader’s procedure, rather than defining a specific response message. Therefore, it is possible to determine the success of msg3 transmission using an implicit mechanism rather than an explicitly defined message.

Proposal 3: RAN2 to discuss explicit and implicit mechanisms for the failure/success indication of D2R in the 3-step CBRA process.

Further discussion 2-step CBRA

Regarding the 2-step CBRA, RAN2 had reached the following agreements in RAN2#127 meeting [1].

for 2step CBRA, RAN2 design will support msg2. Whether it is needed it is up to the reader. FFS when it is needed. For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2. FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

Further discussions are needed to assess the necessity of msg2 transmission and to determine the types of optimizations required for devices supporting 2-step CBRA."

Necessity of msg2 in 2-step CBRA

Different approaches can be considered depending on the use case.

Inventory only: Since there is no further data transmission after msg1, the success of the Device ID transmission in msg1 can be confirmed through msg2.

Inventory + Command: In this case, the success of msg1 transmission can be determined by using the additional data received for the command. In other words, if the device receives a command after transmitting msg1, it can assume that msg1 was successfully transmitted.

Command only: Since the reader is already aware of the device’s presence, random access is not required.

In the 2-step CBRA, the necessity of msg2 may vary depending on the use case. Therefore, different mechanisms can be applied based on the use case, or a new message could be defined to accommodate these variations.

Observation 4: The method for determining the success or failure of msg1 transmission can vary depending on the use case in 2-step CBRA.

Additionally, it is necessary to discuss whether different mechanisms are applied for contention resolution and for indicating the success or failure of msg1 transmission, since the device ID and random number are transmitted together.

Proposal 4: RAN2 to discuss whether to apply different mechanisms are applied for contention resolution and for indicating the success or failure of msg1 transmission in 2-step CBRA.

Optimization for devices which supports 2-step CBRA

The 2-step CBRA involves transmitting both the random number and the device ID in msg1 simultaneously. In particular, unlike the 3-step CBRA, the device’s ID is transmitted in MSG1 during the 2-step CBRA, requiring a specific access occasion configuration. Therefore, the device should be informed of this configuration via a paging message.

Proposal 5: The reader should provide the 2-step CBRA configuration to the device via a paging message.

Further discussion on CFRA

Regarding the CFRA, RAN2 had reached the following agreements in RAN2#127 meeting [1].

In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes. FFS if reader assigns the AS ID for scheduling purposes

Further discussions are required to introduce AS ID for scheduling purpose.

In CFRA, msg1 should be transmitted without collisions between devices, so the reader designates which device is performing the access on a specific access occasion, meaning the reader should already know the device's identifier.

The identifier to designate the device can either be an ID provided by the upper layer or one configured by the AS layer. When using the upper-layer device ID, it may be longer and raise security concerns, but the advantage is that only a single ID needs to be managed for the device. On the other hand, using the AS-layer device ID requires multiple IDs for the device, but it offers enhanced security due to temporary assignment and a shorter ID length. The AS layer ID can also be used as a scheduling identifier for control information and data transmission during a specific time period. As a result, the reader can assign a unique AS layer device ID, which can serve as the device identifier for A-IoT scheduling.

Proposal 6: The AS ID is necessary as a scheduling identifier for both control information and data transmission.

There are two potential mechanisms for assigning the AS ID:

By reader assignment(option1): During the initial access(reception of msg1 in inventory), the reader assigns the device's AS ID and transmits it to the device through msg2.

Random number generation of device(option 2): The device generates a random number during the initial access. If the device receives msg2 containing the same random number that was transmitted in msg1, that number is used as the AS ID.

In option 1, additional information for the AS ID should be transmitted to the device. In contrast, option 2 uses the random number transmitted by the device in msg1, so no extra AS ID transmission is required in msg2. Considering the size of the transmitted content, option 2 is more advantageous.

Proposal 7: The AS ID can be set by the reader or generated using the device's random number.

Conclusions

Based on the discussion in the contribution, we have the following observation and proposals:

Observation 1: The success of msg1 transmission can be determined by the reception of msg2, which contains the same random number.

Observation 2: The success or failure of msg1 reception cannot be delivered by a message.

Observation 3: The success or failure of msg3 transmission can be evaluated differently depending on the use case.

Observation 4: The method for determining the success or failure of msg1 transmission can vary depending on the use case in 2-step CBRA.

Proposal 1: The determination of msg1 transmission success should be based on a timer mechanism.

Proposal 2: RAN2 to discuss whether to apply different methods for indicating the success or failure of D2R transmission based on each use case or to adopt a consistent approach.

Proposal 3: RAN2 to discuss explicit and implicit mechanisms for the failure/success indication of D2R in the 3-step CBRA process.

Proposal 4: RAN2 to discuss whether to apply different mechanisms are applied for contention resolution and for indicating the success or failure of msg1 transmission in 2-step CBRA.

Proposal 5: The reader should provide the 2-step CBRA configuration to the device via a paging message.

Proposal 6: The AS ID is necessary as a scheduling identifier for both control information and data transmission.

Proposal 7: The AS ID can be set by the reader or generated using the device's random number.

References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #127”, August, 2024.