3GPP TSG-RAN WG2 Meeting #127bis											R2-2408931

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.1.2.2

Source: 	Sharp

Title:  	On LCM for UE-sided Beam Management Use Case

Document for: 	Discussion

# Introduction

In RAN2#127, following agreements were made [1]:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In this document, we discuss signaling methods to configure the UE's conditions for reporting applicable functionalities. We also discuss about the functionality activation and signaling mechanisms to manage functionality behaviours, such as (de) activation, switching, and multiple configurations.

# Discussion on Applicable Functionality Reporting

It was agreed during the RAN2 #125bis and #126 meetings that supported functionalities (whether or not they have available models) are to be reported in the UE capability. Meanwhile, the existing UE Assistance Information (UAI) procedure will be utilized for applicable functionality reporting in the case of proactive reporting scenario.

During the email discussion [2], the applicable functionality and related inference configuration were discussed in the context of proactive and reactive reporting respectively.

From the discussions, it appears that most companies are inclined towards adopting a unified signalling approach for both proactive and reactive reporting. This preference indicates a consensus on the benefits of having a consistent framework for reporting applicable functionalities, regardless of whether the reporting is initiated by the UE without explicit requests from the network (proactive) or in response to specific network configurations (reactive).

Observation 1: There is a consensus towards a unified signaling approach for both proactive and reactive reporting of applicable functionalities.

RAN2 further has made following agreements and signalling procedure (as shown in the figure below) on applicable functionality reporting for beam management UE-sided model:

Figure 1: Signaling flow for applicable functionality reporting

We propose to introduce the following aspects for the agreed signalling flow:

Step 3 as shown in Fig. 1, only outlines the configuration elements, without addressing when, how, or under what conditions the functionalities should be reported by the UE. These aspects—timing, triggering conditions, and the mechanism for functionality reporting—are critical for ensuring that the network can receive and act on reports effectively, especially in scenarios where AI/ML functionalities may dynamically change.

Observation 2: Step 3 only outlines the configuration elements, without addressing when, how, or under what conditions the applicable functionalities should be reported by the UE

In legacy, non-AI/ML-based networks, feature configuration and activation are typically performed simultaneously. If the UE is unable to comply with the provided configuration, it encounters radio link issues that may affect its connectivity or performance.

However, AI/ML-based functionalities introduce a new dynamic: the network may need to frequently update or provide new inference configurations as applicable functionalities change. These changes can be triggered by modifications in network-side additional conditions, UE-side additional conditions, or when a specific AI/ML model becomes (un)available or due to the change in behavior and performance of functionality(s) itself.

Given the dynamic nature of the applicable functionalities and UE environment, the network should proactively/pre-configure configure the UE to report any changes in the applicability of the configured functionalities.

Proposal 1: RAN2 to discuss signaling to configure the conditions for the UE to report applicable functionalities for e.g., in step 3, Fig. 1 or in inference configuration.

Figure 2: Signaling flow for applicable functionality reporting

Figure 2 illustrates an example of signaling that may be used to configure the conditions under which the UE reports functionality. This signaling process enables the network to define specific parameters or triggers, ensuring that the UE reports relevant functionalities based on dynamic changes or predefined criteria. The configuration may include various conditions such as network-side or UE-side parameters, model availability, or other context-specific factors that impact the UE's reporting of applicable functionalities.

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3a, 3b” (can also be combined): Following configurations are provided from NW to UE:

  - The necessary configurations for the UE to report applicable functionalities via UAI OtherConfig reporting.

  - Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional.

  - Events/Conditions (e.g., model availability, thresholds for signal quality or latency, Functionality related KPI thresholds) that trigger a given functionality to be deemed applicable/inapplicable.

- Step 3c:  The UE evaluates the applicability of the AI/ML functionality based on several factors, which may include the associated ID and inference configurations provided by the network, as well as UE-specific internal parameters and the availability of the relevant model. This process ensures that the AI/ML functionality is applied appropriately, considering both network-provided inputs and the UE's internal conditions, to determine whether the functionality is ready to be activated or requires adaptation. In case any further adaptation to the functionality is needed, the network may provide an updated configuration in step 5, Fig. 2 respectively.

- Step 4: UE reports applicable functionality in the following scenarios:

    - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI, for e.g. based on the events or conditions as configured in step 3 or step 4.

    - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

In RAN2, there is an FFS and ongoing discussions regarding whether the UE can consider functionality configurations, such as inference configuration, in its decision-making process to determine applicable functionality(s). Additionally, it remains an open issue how the UE determines applicable functionality in cases where the network-side additional conditions or associated IDs are not provided in Step 3. This raises questions about how the UE autonomously evaluates functionality applicability based on its internal parameters, model availability, and other local conditions in the absence of network-side additional conditions or associated IDs.

Proposal 2: The network can provide conditions to the UE to determine applicability of a functionality for e.g., in inference configuration in step 3, Fig. 1.

The network configured conditions would enable the UE to notify the network when functionality changes occur, allowing network to provide timely updates or new inference configurations. The network-configured conditions would enable the UE to determine the applicability of a functionality, even in cases where, for example, NW-side additional conditions or associated IDs are not provided in Step 3.  By proactively addressing these changes, the network can minimize the risk of failures and improve overall system performance in highly dynamic environments. For example, for the Beam Management use-case, CSI configuration could determine when a specific functionality/inference configuration should be applied. When a significant variation in channel quality (e.g., CQI threshold crossed) is detected through CSI, the network could activate or update a specific functionality/inference configuration that adapts to these new conditions.

Observation 3: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

Furthermore, to avoid unnecessary signaling overhead, the network may provide multiple functionality configurations, such as inference configurations for a given supported or applicable functionality. This allows the network to select the most appropriate configuration based on current conditions and the UE's capabilities.

Proposal 3: The network should provide multiple functionality configurations, such as inference configurations for a given functionality.

Depending on the UE capabilities and requirements, the network may flexibly configure one or more of the configured functionalities to be activated simultaneously. The required configurations may be provided by the network, for e.g. in a single RRC message in step 3a in combination with step 3b, or with multiple instances of steps 3a and 3b. The network may determine the appropriate functionalities to be activated at different time instances and provide the necessary configurations to the UE, but to be activated simultaneously.

Proposal 4: If additional conditions including UE-internal conditions allows, Multiple functionalities may be activated simultaneously, each of which may consist of one or more functionality configurations.

# Discussion on Functionality Activation

In RAN2#127, the mechanism to (de) activate functionality was discussed and there were open points and FFS. The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  As per the agreements in RAN2 127, it is still FFS on the initial activation state of the functionality. Furthermore, it is also FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3 [1].

Proposal 5: Initial state of the functionality may be explicitly indicated by the network to the UE.

The initial configurations provided by the network may not provide all the necessary information for the UE to make the functionality applicable. Further, the UE may require some time to evaluate whether the internal conditions allow a certain functionality to be reported as applicable. Therefore, it may be considered that the initial state of the functionality is deactivated, especially when it is not explicitly indicated by the network.

Proposal 6: If initial state of the functionality is not indicated by the network to the UE, the functionality can be considered as de-activated.

In this context, the final applicability of a functionality may be determined either by the UE or the network, depending on whether the UE has sufficient information to evaluate the functionality's applicability. If the UE has adequate information, such as inference configurations or additional conditions, it may autonomously determine the applicability. Otherwise, the network may take responsibility for making this determination. In this context, the final applicability of a functionality may be determined either by the UE or the network, depending on whether the UE has sufficient information to evaluate the functionality's applicability

While the framework for activating applicable functionalities has been outlined, key aspects concerning the initial activation states and their reliance on the inference configurations provided in Steps 3 and 5 in Fig. 1, still necessitate further study and discussion within RAN2. Additionally, the potential for multiple inference configurations to be assigned to a single functionality, as well as the possibility of simultaneously activating multiple functionalities, are critical factors that must be incorporated to enhance efficiency and ensure optimal performance.

Proposal 7: The network may provide a default or preferred configuration in functionality configuration for e.g., inference configuration to activate the functionality.

However, using the inference configuration message (e.g., RRC message) to implicitly activate multiple applicable functionalities simultaneously may lead to substantial signaling overhead. This concern is particularly relevant given that several functionalities may be applicable at the same time. Alternatively, in time-critical scenarios such as inference and real-time monitoring, employing L1/L2 signaling, which is characterized by low latency, can be considered a more efficient and effective approach. Therefore, in addition to leveraging inference configuration for activating AI/ML-enabled applicable functionalities, we think that L1/L2 signaling could also be an effective method for indicating functionality (de) activation

Proposal 8: L1/L2 signaling can be utilized to manage functionality behavior (s) e.g., (de) activation, switching, configurations etc.

# Conclusions

Based on the discussion above, we have the following proposals:

Observation 1: There is a consensus towards a unified signaling approach for both proactive and reactive reporting of applicable functionalities.

Observation 2: Step 3 only outlines the configuration elements, without addressing when, how, or under what conditions the applicable functionalities should be reported by the UE

Observation 3: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

Proposal 1: RAN2 to discuss signaling to configure the conditions for the UE to report applicable functionalities for e.g., in step 3 or in inference configuration.

Proposal 2: The network can provide conditions to the UE to determine applicability of a functionality for e.g., in inference configuration if NW-side additional conditions are not provided in step 3.

Proposal 3: The network should provide multiple functionality configurations, such as inference configurations for a given functionality

Proposal 4: If additional conditions including UE-internal conditions allows, Multiple functionalities may be activated simultaneously, each of which may consist of one or more functionality configurations.

Proposal 5: Initial state of the functionality may be explicitly indicated by the network to the UE.

Proposal 6: If initial state of the functionality is not indicated by the network to the UE, the functionality can be considered as de-activated.

Proposal 7: The network may provide a default or preferred configuration in functionality configuration for e.g., inference configuration to activate the functionality.

Proposal 8: L1/L2 signaling can be utilized to manage functionality behavior (s) e.g., (de) activation, switching, configurations etc.

# Reference

[1] Chair notes, RAN2 127 3GPP TSG-RAN WG2 Meeting #127	Maastricht, Netherlands, 2024

[2] R2-2406381 Post meeting email discussion [POST126][032] [AI/ML PHY] LCM