3GPP TSG-RAN WG2 Meeting#127bis                           	R2-2408934

Hefei, China, Oct 14-18, 2024

Agenda item:	8.2.2

Source: 	Nordic Semiconductor ASA

Title: 	Message size reporting and segmentation

Document for:	Discussion and Decision

# Introduction

In recent RAN2 meetings several A-IoT UP functional details were discusses:

- Segmentation

  - Concerns on maximum/typical TBS sizes

  - Need for support and complexity

- In order delivery

- Retransmissions

  - How to ACK/NACK D2R messages

- Various reports from a device to a reader

  - Energy status

  - Message size

Based on the ongoing discussions a simple segmentation with retransmissions support could be possible, for example, using a lock-step protocol with a window size of one packet, especially for the D2R direction. Most of the discussed segmentation, retransmission and reporting topics are all interconnected, and need to be treated as a one package to avoid unnecessarily increase of the device implementation complexity and the need for additional resources in the device.

RAN2#127 had the following agreement:

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

In this paper we discuss the D2R message size reporting and potential optimizations to D2R message acknowledgements. The assumption in this paper is that D2R message segmentation is supported at a MAC layer.

# Discussion

## Message size reporting

When a device supports segmentation, it must have a mechanism to indicate a reader that there are more segments still pending. The indication could be as simple as a “more segments/no more segments” flag in every D2R message. It is not clear why a reader should know the unsegmented A-IoT D2R message size ahead of time in addition that there are more segments pending.

Whether there is a need for reporting the remaining message size in the last segment is also not clear. If the last segment is not large enough to fill the entire scheduled TBS that could be handled with MAC padding. This would imply the message sizes are always, for example, multiple of 8 or less bits.

There is no benefit for the reader to know the unsegmented D2R message size in finer granularity than that there are more segments pending.

The remaining message size reporting in the last segment can implicitly be handled by MAC padding.

Based on the above observations we propose that the “more segments/no more segments” indication flag is enough, and no further size report is required.

The “more segments/no more segments” indication flag is adequate as a simple message size reporting.

There is no need for a “remaining message size” reporting in the last segment.

## D2R message delivery acknowledgement

With the assumption that the device supports D2R message segmentation, the R2D trigger to schedule subsequent D2R segments would implicitly serve as an ACK/NACK for the previous D2R segment. However, there are use cases where the device does not need segmentation for the ongoing transaction or there is no need for an ACK/NACK for the D2R message delivery in general (the delivery of the last D2R message, segmented or not).

If the A-IoT use case does not require the device to receive an ACK for its D2R message, then the “more segments/no more segments” indication flag could have a dual role as “no ACK/NACK needed” indication to the reader.

The D2R message “more segments/no more segments” indication flag can have a dual role as “no ACK/NACK needed” indication to the reader.

# Conclusion

In this paper, we proposed an approach for message size reporting and a way to avoid unnecessary D2R message ACK/NACK. Based on the discussion in the paper, we respectfully ask RAN2 to discuss and consider the following proposals:

- The “more segments/no more segments” indication flag is adequate as a simple message size reporting.

There is no need for a “remaining message size” reporting in the last segment.

The D2R message “more segments/no more segments” indication flag can have a dual role as “no ACK/NACK needed” indication to the reader.

# References