3GPP TSG-RAN WG2 Meeting #127b	R2-2408936

Hefei, China, 14th – 18th October, 2024

Agenda Item:	7.9.4

Source:	Huawei, HiSilicon

Title:	Discussion on RLC channel Definition alignment across the specs

Document for: Discussion and Decision

# 1	Introduction

In the last meeting we had submitted an CR for 38.322 in [1] to add a note for L2 MP Relay using N3C indirect path. This had triggered a discussion on different RLC channel definition in the specs and the misalignment that exists in the specs. The RLC CR in [1] was postponed as companies had different understanding on the RLC channel definition and during the online discussion some companies proposed to remove the existing notes in section 4.2.1 of TS 38.322 that are highlighted below as a way forward to align the RLC channel definition in the specs.

### 4.2.1	RLC entities

The description in this clause is a model and does not specify or restrict implementations.

RRC is generally in control of the RLC configuration.

Functions of the RLC sub layer are performed by RLC entities. For an RLC entity configured at the gNB, there is a peer RLC entity configured at the UE and vice versa. In NR sidelink communication, in NR sidelink discovery, for an RLC entity configured at the transmitting UE, there is a peer RLC entity configured at each receiving UE.

An RLC entity receives/delivers RLC SDUs from/to upper layer and sends/receives RLC PDUs to/from its peer RLC entity via lower layers.

An RLC PDU can either be an RLC data PDU or an RLC control PDU. If an RLC entity receives RLC SDUs from upper layer, it receives them through a single RLC channel between RLC and upper layer, and after forming RLC data PDUs from the received RLC SDUs, the RLC entity submits the RLC data PDUs to lower layer through a single logical channel. If an RLC entity receives RLC data PDUs from lower layer, it receives them through a single logical channel, and after forming RLC SDUs from the received RLC data PDUs, the RLC entity delivers the RLC SDUs to upper layer through a single RLC channel between RLC and upper layer. If an RLC entity submits/receives RLC control PDUs to/from lower layer, it submits/receives them through the same logical channel it submits/receives the RLC data PDUs through.

NOTE 1: In case the upper layer is BAP as defined in TS 38.340 [7], an RLC channel refers to a Backhaul 		  RLC channel.

NOTE 2: In case the upper layer is SRAP as defined in TS 38.351 [9], an RLC channel refers to either a PC5 		  Relay RLC channel or a Uu Relay RLC channel.

In this paper we provide our views on the RLC channel definitions and suggest a way forward for aligning the RLC Channel definition across the specs.

# 2	Discussion

In our understanding the “RLC channel”  was a term that was initially introduced for the “internal link between upper layer and RLC entity" in R15. With the introduction of support of IAB and SL Relay features in the subsequent releases,  the  definition “RLC Channel” was extended to include  “BH RLC channel” and “PC5/Uu Relay RLC channel” as the external link between corresponding RLC entities.

This is quite clear if we take the example of IAB and look at the protocol stack in the figure below for IAB from Stage 2 specifications where the intention of BH RLC channel defined for IAB was to include internal channel between BAP and RLC layer as well as the external channel between IAB node and another node, which is more like an RLC bearer.

In our view when IAB experts extended the definition of RLC Channel and introduced this “BH RLC channel” concept in different specs, they attempted to have a broader definition to cover both internal interface and external interface. Note 1 in section 4.2.1 of TS 38.322 was introduced in v16.2.0 to reflect this and it indeed aligns with this intention and is also in line with the description IAB in TS 38.300.  When introducing SL relay functionality, we just adopted the same concept of IAB and introduced Note 2 in section 4.2.1 of TS 38.322 and we believe the same interpretation for SL Relay RLC Channels should be used as for the IAB.

We understand that there is a misalignment in the definition of the term “RLC Channel” and “XXX RLC Channel” in the specs, however, in our view deleting the notes in section 4.2.1 of TS 38.322 in an attempt to align the RLC Channel definitions will be incorrect and introduce further misalignment and confusion between Stage 2 and other specs. Furthermore, we in SL Relay session cannot decide on deleting the Note 1 without involving the IAB experts

Observation 1: When IAB experts extended the definition of RLC Channel and introduced this “BH RLC channel” concept in different specs, they attempted a broader definition to cover both internal interface and external interface. Note 1 in section 4.2.1 of TS 38.322 was introduced in v16.2.0 to reflect this and it indeed aligns with this intention and is also in line with the IAB description in TS 38.300

Observation 2: When introducing SL relay functionality, we just adopted the same concept of IAB and introduced Note 2 in section 4.2.1 of TS 38.322 in v17.2.0 and we believe the same interpretation for SL Relay RLC Channels should be used as for the IAB

Proposal 1: Considering observation 1 and 2, Notes 1 and 2 in section 4.2.1 of TS 38.322 should not be deleted and taken as the way forward for aligning the “RLC Channel” definition across the specs, specially knowing that Note 1 is related to IAB and is out side the scope of the SL Relay experts to modify it.

Our view is that instead of deleting the notes in TS 38.322 (RLC Specs), right approach would be that we generalize the definition of PC5 and Uu Relay RLC channel in TS 38.300 (Stage 2), TS 38.331 (RRC Specs) and TS38.351 (SRAP Specs) which are backward compatible and are more general for defining external link between two RLC entities.

PC5 and Uu Relay RLC channel definitions are proposed to be clarified in our accompanying CR for TS 38.300 in [2]. The changes are shown below.

PC5 Relay RLC channel: a channel which is used to transport RLC SDUs over PC5 between L2 U2N Remote UE and L2 U2N Relay UE for L2 UE-to-Network relay operation.

Uu Relay RLC channel: a channel which is used to transport RLC SDUs over Uu between L2 U2N Relay UE and gNB for L2 UE-to-Network relay operation.

Similar changes can be adopted for BH RLC channel and can be decided by the IAB experts.

Proposal 2: Clarify the definitions PC5 and Uu Relay RLC channel as described below in TS 38.300 as a starting point for aligning the RLC Channel definitions across the specs.

PC5 Relay RLC channel: a channel which is used to transport RLC SDUs over PC5 between L2 U2N Remote UE and L2 U2N Relay UE for L2 UE-to-Network relay operation.

Uu Relay RLC channel: a channel which is used to transport RLC SDUs over Uu between L2 U2N Relay UE and gNB for L2 UE-to-Network relay operation.

# 3	Conclusion

In this paper we provide our views on the RLC channel definitions and suggest a way forward for aligning the RLC Channel definition across the specs. We have the following observations and proposals

Observation 1: When IAB experts extended the definition of RLC Channel and introduced this “BH RLC channel” concept in different specs, they attempted a broader definition to cover both internal interface and external interface. Note 1 in section 4.2.1 of TS 38.322 was introduced in v16.2.0 to reflect this and it indeed aligns with this intention and is also in line with the IAB description in TS 38.300

Observation 2: When introducing SL relay functionality, we just adopted the same concept of IAB and introduced Note 2 in section 4.2.1 of TS 38.322 in v17.2.0 and we believe the same interpretation for SL Relay RLC Channels should be used as for the IAB

Proposal 1: Considering observation 1 and 2, Notes 1 and 2 in section 4.2.1 of TS 38.322 should not be deleted and taken as the way forward for aligning the “RLC Channel” definition across the specs, specially knowing that Note 1 is related to IAB and is outside the scope of the SL Relay experts to modify it.

Proposal 2: Clarify the definitions PC5 and Uu Relay RLC channel as described below in TS 38.300 as a starting point for aligning the RLC Channel definitions across the specs.

PC5 Relay RLC channel: a channel which is used to transport RLC SDUs over PC5 between L2 U2N Remote UE and L2 U2N Relay UE for L2 UE-to-Network relay operation.

Uu Relay RLC channel: a channel which is used to transport RLC SDUs over Uu between L2 U2N Relay UE and gNB for L2 UE-to-Network relay operation.

# 4	References

- R2-2407293 RLC correction for multi-path relay with N3C

- R2-2408663Clarification on definition of PC5 and Uu Relay RLC channel (R17 38.300 CR)