3GPP TSG-RAN WG2 Meeting#127bis                           	R2-2408938

Hefei, China, Oct 14-18, 2024

Agenda item:	8.2.4

Source: 	Nordic Semiconductor ASA

Title: 	AS scheduling ID and AS short ID considerations

Document for:	Discussion and Decision

# Introduction

RAN2#127 had the following agreement under A-IoT Random Access:

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

Furthermore, Random Access was extensively discussed in the post meeting report [1]. This paper discusses possible ways forward to align the use of the AS scheduling ID and the AS short ID for both CBRA and CFRA purposes.

# Discussion

## AS scheduling ID and Short ID considerations

The [1] report proposal 8 (part 1) concluded:

- RAN2 assumes this AS scheduling ID can be a short AS layer ID, rather than the upper layer device ID

Both IDs have same size and could use the same container in some A-IoT messages.

On the other hand, some companies had concerns that the AS short ID should be short lived to reduce the probability of randomly generated AS short ID collisions during multiple devices initiating CBRAs.

The [1] report proposal 8 (part 2) concluded:

- Option 2: reader assigns the AS scheduling ID, which is unique among the devices under service.

And finally, the [1] report proposal 8 (part 3) concluded:

- Option 2: reader assigns the AS scheduling ID. FFS via which R2D message.

- The AS scheduling ID could optionally be conveyed in relevant R2D messages (trigger or Msg2) for the possible subsequent R2D message. This would harmonize the protocol design and use the same conveyed ID for both CBRA and CFRA use cases.

- In the case of a CBRA procedure the Msg2 would convey the AS scheduling ID. The AS short ID in the Msg2 is used only for the contention resolution purposes and replaced with the newly assigned AS scheduling ID for the device use in subsequent message exchanges. Whether the AS short ID and the AS scheduling ID values have the same numerical value is up to reader implementation.

The randomized AS short ID should be replaced with the AS scheduling ID after the Msg2. Whether the value of the AS scheduling ID is the same or different to the AS short ID is up to a reader implementation.

- In the case of a CFRA procedure the initial trigger message would convey the AS scheduling ID along with the ID associated with device(s). The device proceeds directly to Msg3 transmission and the uses the AS scheduling ID with all the subsequent message exchanges.

The AS scheduling ID is conveyed in the initial trigger message for the CFRA procedure.

- Summarizing the above we propose the following:

The initial trigger and Msg2 messages are used to convey the AS scheduling ID.

The signalled AS scheduling ID is used in all message exchanges after the Msg2.

# Conclusion

In this paper, we discussed the uses of the AS scheduling ID and the AS short ID in both CBRA and CFRA procedures. We respectfully ask RAN2 to discuss and consider the following proposals:

- The initial trigger and Msg2 messages are used to convey the AS scheduling ID.

- The signalled AS scheduling ID is used in all message exchanges after the Msg2.

# References

# R2-2407985 Report of [POST127][033][AIoT] Random Access