3GPP TSG-RAN WG2 Meeting #127bis	R2-2408940

Hefei, China, 14th – 18th October 2024

Agenda Item:	7.2.1

Source:	Ericsson

Title:	Sidelink Positioning Postponed issues for RRC

Document for:	Discussion and Decision

# Introduction

The below SL positioning RRC items were postponed from previous RAN2 meeting (RAN2#127). In this paper we discuss those issues.

- R2-2406405	Miscellaneous Corrections for sidelink positioning	vivo	draftCR	Rel-18	38.331	18.2.0	F	FS_NR_pos_enh2

- Changes 2 and 6 are postponed

- R2-2407273	RRC correction on NR sidelink positioning	Philips International B.V.	CR	Rel-18	38.331	18.2.0	4940	-	F

- Changes 11 is postponed

- SL CBR ASN.1 issue:

We have below two ways to further consider as way forward:

- Do not do any change.

- BC with NCE

# R2-2406405 and R2-2407273

## 2.1	Change 2: If a dedicated SL positioning specific RRC Resume initiation procedure is needed or not.

This was discussed in RAN2#127 and also discussed as part of RIL review where it was agreed that even though the UE needs to initiate the procedure for SL positioning, the UE anyway has to first perform basic communication related procedure and thus a separate RRC Resume clause for SL positioning is not needed. The main discussion point was that there is no SL-PRS transmission only UE and hence separate clause is not essential.

SL Positioning can be considered as an additional service built upon the SL communication where SL communication procedure would be executed and SLPP session would be established before doing any RRC related; and hence it is unlikely that the UE would have to perform a resume procedure solely for the purpose of SL positioning.

There is no SL-PRS transmission only UE. For UEs involved in the SL positioning session, SL communication is still needed and therefore separate initiation procedure is not essential, however there is no harm in providing it. But as the bar for any correction is high and only essential correction is needed, we should not specify the new clause.

SL Positioning does not need separate clause for RRC Resume initiation.

## 2.2	Change 6: It proposes the below addition in text proposal

sl-BWP-PRS-PoolConfig

This field indicates the dedicated SL-PRS resource pool configurations for SL-PRS on the configured sidelink BWP. This field does not include sl-PRS-TxPoolExceptional. In this release, dedicated SL-PRS resource pool is not expected to be configured in the slots colliding with the slots (pre)configured for any other resource pool(s) or S-SSB resource(s) in other SL carriers.

However, the above change is a physical layer behaviour which is captured in RAN1 specification. Besides stage2 already captures below:

A sidelink resource pool which can be used for transmission of both, SL-PRS and SL data is referred to as SL-PRS shared resource pool. A sidelink resource pool which can be used for transmission of SL-PRS and cannot be used for transmission of SL data is referred to as SL-PRS dedicated resource pool.

Dedicated resource pool is defined in RAN2 stage2 TS 38.305 and RAN1 TS 38.214 and thus no further definition is needed in RRC.

SL-BWP-PRS-PoolConfig field description does not include dedicated SL-PRS Resource pool

## 2.3 Change 11: In clause 6.2.2, SidelinkUEInformationNR, in field description of sl-TxInterestedFreqList, remove “/SIB23”.

sl-TxInterestedFreqList Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication/positioning, for each destination. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12/SIB23, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12/SIB23, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12/SIB23 and so on.

As part of SUI, the UE indicates the interested frequencies for communication and/or positioning (for SL-PRS transmissions). It can indicate the Tx frequencies based upon the list, it receives from SIB12 and SIB23. SIB12 consist of shared resource pool which can be used for both communication and positioning (for SL-PRS transmission). SIB23 consist of dedicated resource pool for SL-PRS transmission.

There are two fields sl-freqInfoList and sl-PosfreqInfoList which exist in SIB12 and SIB23 correspondingly. In SUI, the UE may use sl-TxInterestedFreqList for interested sl-freqInfoList and sl-PosTxInterestedFreqList to indicate interested sl-PosfreqInfoList. It is clear that reference of SIB23 is not desired in field sl-TxInterestedFreqList.

However, a question that remains is how can UE indicate positioning frequency interest from the shared resource pool SIB12 configuration (sl-freqInfoList); Or the question can be does the NW need to be aware the purpose of why the UE is interested in this frequency. From NW perspective as long as it knows that it is interested could be adequate.

Change 11 suggests removing SIB23 from the sl-TxInterestedFreqList which is correct.

A question that remains is whether NW needs to know the purpose why UE is interested in that frequency configured for the shared resource pool by SIB12 configuration (sl-freqInfoList).

Since NW knows that the configured frequency has shared resource pool which can be used for positioning and communication, further distinction (indication from UE) may not be needed.

If the NW needs to know, then sl-PosTxInterestedFreqList field description need to be updated with additional mapping with frequencies in SIB12.

sl-PosTxInterestedFreqList Each entry of this field indicates the index of frequency on which the UE is interested to transmit SL-PRS. The value 1 corresponds to the frequency of first entry in sl-PosFreqInfoList broadcast in SIB23, the value 2 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

Remove SIB23 from the field description of sl-TxInterestedFreqList.

RAN2 to discuss the need to distinguish the purpose of interest (whether the interest is communication or positioning) while using SUI for frequency list configured from SIB12.

# Channel Busy Ratio (CBR) background

## CBR measurement

The Channel Busy Ratio (CBR) is an important metric used in wireless communications, particularly for vehicle-to-everything (V2X) and sidelink communications, and from R18 also for sidelink positioning. CBR is used to measure channel congestion levels, to enable decentralized congestion control mechanisms, and to help manage radio resources efficiently in sidelink communication/positioning.

[TS 38.215] SL PRS Channel Busy Ratio (SL PRS-CBR) measured in slot n is defined as the number of SL PRS resources in the dedicated SL PRS resource pool whose SL PRS RSSI measured by the UE exceed a (pre-)configured threshold provided by the higher layer parameter sl-ThreshS-RSSI-PRS-CBR sensed over a SL PRS-CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to higher layer parameter [sl-TimeWindowSize-PRS-CBR-positioning] divided by the total number of the configured SL PRS resources in the transmission pool over [n-a,n-1].

The calculation of SL PRS-CBR is limited within the slots for which the SL PRS-RSSI is measured. If the number of SL PRS-RSSI measurement slots within the SL PRS-CBR measurement window is below a (pre-)configured threshold, a (pre-)configured SL PRS-CBR value is used.

- UE measures CBR according to the above definition.

## CBR Ranges

The network configures a list of CBR ranges using the sl-CBR-RangeDedicatedSL-PRS-RP-List. The maximum number of CBR ranges for SL positioning is 8. Each entry SL-CBR-LevelsDedicatedSL-PRS-RP-r18 is associated to a specific priority and the mapping is defined by sl-PriorityThresholdDedicatedSL-PRS-RP-r18 and sl-CBR-ConfigIndexDedicatedSL-PRS-RP-r18. Each entry SL-CBR-LevelsDedicatedSL-PRS-RP-r18 includes multiple and maximum 16 CBR levels in ascending order, and the values of SL-CBR-LevelsDedicatedSL-PRS-RP-r18 /CBR levels are integers from 0 to 100, representing CBR percentages. For example, if the entry contains values [20, 50, 80], it defines the following CBR ranges: 0% to 20%, >20% to 50%, >50% to 80%.

Transmission Parameters
The sl-CBR-SL-PRS-TxConfigList defines various transmission parameter sets for SL-PRS. Each set includes:

- Maximum transmission power (sl-PRS-MaxTx-power)

- Maximum number of transmissions (sl-PRS-MaxNum-Transmissions)

- Channel occupancy ratio limit (sl-PRS-CR-Limit)

sl-CBR-SL-PRS-TxConfigList can define maximum 64 transmission parameter sets for SL-PRS.

Mapping CBR to Transmission Parameters

The sl-CBR-PriorityTxConfigDedicatedSL-PRS-RP-List maps CBR ranges and priority levels to specific transmission parameter sets. This mapping allows the system to adapt SL-PRS transmission based on channel congestion and SL-PRS priority.

Procedures

- The UE first measures the CBR for the sidelink resource pool

- The UE determines the priority of the SL-PRS transmission from upper layer.

- By the priority and according to configurations in sl-CBR-PriorityTxConfigDedicatedSL-PRS-RP-List-r18, UE determines sl-CBR-ConfigIndexDedicatedSL-PRS-RP-r18 and sl-PRS-TxConfigIndexList-r18, which are further used to determine entry of the CBR range configuration in sl-CBR-RangeConfigList-Dedicated-SL-PRS-RP and sl-CBR-SL-PRS-TxConfigList-r18.

- UE compares the measured CBR value and the CBR range to determine which set of SL-PRS transmission parameters to be used.

Agreement

In Scheme 2,

- For a dedicated resource pool for positioning,

  - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority:

    - Maximum SL PRS transmission power

    - Maximum Number of SL PRS (re-)transmissions

    - Discuss further the following four SL PRS transmission parameters:

      - Minimum Periodicity of SL PRS

      - Maximum Number of SL PRS resources in a slot

      - Maximum comb-size of a SL PRS resource in a slot

      - Maximum Number of OFDM symbols of a SL PRS resource in a slot

  - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool

    - Note: Similar to SL communication how to achieve the CR limit is left to UE implementation.

Agreement

In Scheme 2, with regards to the congestion control for SL PRS:

- SL-PRS congestion processing time: based on both SCS and UE capability, similar to legacy

- The maximum number of CBR ranges for SL positioning is 8

- Number of CBR levels is 16

- CBR measurement for SL PRS can be reported to gNB

FFS: Whether it is needed to be reported to LMF or another UE

For UE only SL Positioning operation, the preconfiguration determines how to evaluate whether the channel is busy and how much channel the UE can occupy. UE measures and calculates the CBR and with the received priority it maps to an index to identify the suitable SL-PRS transmission parameters.

Because of ASN.1 providing one less value for mapping, it may happen that the mapping is slightly less granular and few of the CBR computed value range/level and Priority may map to same SL-PRS transmission parameters.

ITS currently (TS 102 687 - V1.2.1 - Intelligent Transport Systems (ITS); Decentralized Congestion Control Mechanisms for Intelligent Transport Systems operating in the 5 GHz range; Access layer part (etsi.org) in Annex A ) provides only 5 states (i.e., 5 CBR ranges). Based on this, the CBR range that we have currently 7 should be ok.

ITS provides 5 CBR states.

RAN2 does not update the ASN.1 solution for CBR corrections and wait until SL resource pool extension is needed for any other purpose.

# 3	Conclusion

Observation 1	There is no SL-PRS transmission only UE. For UEs involved in the SL positioning session, SL communication is still needed and therefore separate initiation procedure is not essential, however there is no harm in providing it. But as the bar for any correction is high and only essential correction is needed, we should not specify the new clause.

Observation 2	Dedicated resource pool is defined in RAN2 stage2 TS 38.305 and RAN1 TS 38.214 and thus no further definition is needed in RRC.

Observation 3	Change 11 suggests removing SIB23 from the sl-TxInterestedFreqList which is correct.

Observation 4	A question that remains is whether NW needs to know the purpose why UE is interested in that frequency configured for the shared resource pool by SIB12 configuration (sl-freqInfoList).

Observation 5	Since NW knows that the configured frequency has shared resource pool which can be used for positioning and communication, further distinction (indication from UE) may not be needed.

Observation 6	For UE only SL Positioning operation, the preconfiguration determines how to evaluate whether the channel is busy and how much channel the UE can occupy. UE measures and calculates the CBR and with the received priority it maps to an index to identify the suitable SL-PRS transmission parameters.

Observation 7	Because of ASN.1 providing one less value for mapping, it may happen that the mapping is slightly less granular and few of the CBR computed value range/level and Priority may map to same SL-PRS transmission parameters.

Observation 8	ITS provides 5 CBR states.

Based on the discussion in the previous sections we propose the following:

Proposal 1	SL Positioning does not need separate clause for RRC Resume initiation.

Proposal 2	SL-BWP-PRS-PoolConfig field description does not include dedicated SL-PRS Resource pool

Proposal 3	Remove SIB23 from the field description of sl-TxInterestedFreqList.

Proposal 4	RAN2 to discuss the need to distinguish the purpose of interest (whether the interest is communication or positioning) while using SUI for frequency list configured from SIB12.

Proposal 5	RAN2 does not update the ASN.1 solution for CBR corrections and wait until SL resource pool extension is needed for any other purpose.

# 4	References

- [1] Annex A TS 102 687 - V1.2.1 - Intelligent Transport Systems (ITS); Decentralized Congestion Control Mechanisms for Intelligent Transport Systems operating in the 5 GHz range; Access layer part (etsi.org)