3GPP TSG-RAN WG2 Meeting #127bis	R2-2408945

Hefei, China, 14th – 18th of October 2024

Agenda item:	7.4.2

Source:	Nokia

Title:	Miscellaneous Rel-18 LTM Aspects and Corrections

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we are addressing several remaining issues for Release 18 LTM.

# 2	Discussion

## 2.1	On Early TA acquisition for inter-DU (RAN3 LS)

In RAN3#125 the LS was approved with the following questions RAN2 is supposed to answer [1]:

- Question 1: Are shared preamble resources from the RAN2 agreement, consisting of the combination of SSB index, PRACH mask index, and preamble index, necessary, or is the preamble index list sufficient? - Question 2: When using the RA-RNTI, is “SSB Positions In Burst” a necessary parameter to associate the TA value with a specific UE?

RAN2 is asked to respond to two questions related to random access configuration. Question 1 basically asks if preamble index is sufficient or whether more details are needed in F1 signalling.

Observation 1: RAN3 asks if F1 interface should convey the information related to just the preamble index or more details, such as SSB index, PRACH mask index, and preamble index are necessary.

Source DU obtains RACH Configuration which includes EarlyUL-SyncConfig IE. The details can be found in [2], section 9.3.1.328, while EarlyUL-SyncConfig is described in NR RRC [3]:

EarlyUL-SyncConfig-r18 ::= SEQUENCE { frequencyInfoUL-r18 FrequencyInfoUL, rach-ConfigGeneric-r18 RACH-ConfigGeneric, bwp-GenericParameters-r18 BWP, ssb-PerRACH-Occasion-r18 ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL, -- Need R prach-RootSequenceIndex-r18 CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) } OPTIONAL, -- Need R ltm-PRACH-SubcarrierSpacing-r18 SubcarrierSpacing OPTIONAL, -- Need L139 n-TimingAdvanceOffset-r18 ENUMERATED { n0, n25600, n39936, spare1 } OPTIONAL, -- Need R ... }

The aforementioned IE includes the SSB to RACH mapping (ssb-PerRACH-Occasion) and other essential parameters. The list of preamble indices and SSB to RACH occasion mapping is sufficient for the source DU to generate the PDCCH order. PRACH mask index is not needed either, as the source DU is able to realize when there are multiple RACH occasions for the selected SSB in PDCCH order. There will be a unique RA-RNTI per each RACH occasion, so the source DU can tell which UE is associated with the TA value it receives from the candidate DU. Thus, all of the necessary components are already available within the IEs in EarlyUL-SyncConfig.

Observation 2: EarlyUL-SyncConfig conveyed in RACH Configuration sent over F1AP contains all necessary RACH-related parameters required by the source DU to send the PDDCH order.

Thus, we suggest the following:

Proposal 1: RAN2 responds to Question 1 by stating no additional information is needed in RACH Configuration sent over F1AP.

Second question stated in RAN3 LS [1] concerned ssb-PositionsInBurst IE and whether this has to be signalled to associate the TA with a specific UE. ssb-PositionsInBurst is included in LTM-SSB-Config for any LTM-Candidate. This IE is used to indicate the number of SSBs transmitted by each cell. Together with ssb-PerRACH-Occasion it is necessary to associate the SSBs with their corresponding RACH occasion. Thus, RAN2 should positively respond to question 2: ssb-PositionInBurst should be signalled in EarlyUL-SyncConfig over F1AP.

Proposal 2: RAN2 responds to Question 2 by stating ssb-PositionInBurst should be signalled in EarlyUL-SyncConfig over F1AP.

## 2.2 	On the order of actions in Stage-2 LTM message sequence

As concluded in RAN4 and captured in [4], the requirements for PDCCH-ordered RACH apply only for the UEs that have provided L1- or L3-RSRP report (for FR1) or L1-RSRP (for FR2) [4]:

### 6.2.2C PDCCH ordered Random Access for LTM #### 6.2.2C.1 Introduction The requirements in this clause apply for a UE capable of [FG 45-5: RACH-based early TA acquisition] configured to trigger RACH procedure toward target LTM candidate cell before receiving cell switch command MAC-CE. The requirements in this clause apply if PDCCH order-based RACH on candidate cell is triggered based on L1-RSRP report or L3-RSRP beam-level report in FR1, or based on L1-RSRP report in FR2.

Observation 3: As per RAN4 specification, PDCCH-ordered RACH requirements apply only for the UE that has provided L1- or L3-RSRP report (for FR1) or L1-RSRP report (for FR2).

Similar concerns are applicable to TCI state activation. As per RAN4 and their [4], TCI state activation can be done only for a known TCI state (i.e. UE has recently reported L1-RSRP for the RS associated to the TCI state) or, in case of FR1, for an unknown TCI state if the UE has reported L3 measurement result with the associated SSB index of the TCI state.

Observation 4: As per RAN4 requirements, TCI state activation can be done only for the known TCI state or, in case of FR1, for unknown TCI state if the UE has reported L3 measurement result with the associated SSB index of the TCI state.

This implies Figure 9.2.3.5.2-1 in [5] and/or the corresponding description might require an update as these were drafted prior to the aforementioned RAN4 requirements have been made.

Observation 5: Figure 9.2.3.5.2-1 and the corresponding description in TS 38.300 were drawn prior to the RAN4 LTM requirements.

There are two options how to resolve the issue: either the figure can be updated or one of the steps in the associated description should be extended with the information on the required L1-RSRP or L3-RSRP reporting.

Proposal 3: RAN2 is asked to adopt the changes to TS 38.300 depicted in Annex A.

## 2.3 	On LTM recovery number of attempts

LTM recovery has been specified following the same principles as CHO recovery: upon detecting RLF/ LTM cell switch failure, the UE, if configured with LTM recovery, will select a suitable cell. If the suitable cell is an LTM candidate cell, the UE will attempt LTM recovery on this cell. This behaviour is specified in sections 5.3.7.3 and 5.3.5.8.3 of NR RRC specification [3].

However, unlike CHO, LTM can be subsequent (i.e. another LTM can be executed using the previously provided configuration). While with CHO, the configuration is deleted once CHO recovery is attempted, in LTM the configuration is maintained after successful LTM switch or LTM recovery (attempt or success).

Observation 6: In CHO Recovery the candidate target cell’s configurations are deleted upon the CHO recovery attempt is initiated. In case of LTM, the configurations are released only if the LTM recovery procedure has failed. Otherwise, configurations are kept by the UE.

Upon attempting LTM recovery for the first time (via the re-establishment procedure), considering the IF branch in section 5.3.7.3 of [3] (also shown below), the UE will execute LTM cell switch procedure as specified in section 5.3.5.18.6 [3]:

1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

If this is unsuccessful, the UE returns to the re-establishment procedure. The expected behaviour would be that the UE would now release the LTM configuration and perform a classic re-establishment since its LTM recovery attempt was already unsuccessful. However, based on the current specification, the LTM the configuration including the attemptLTM-Switch flag and the candidate list are maintained. The current text seems to suggest that the UE may be able to try LTM recovery for an unlimited number of times, which is not the intended behavior.

Observation 7: Due to the subsequent nature of LTM, based on the current specification, it seems the UE may be able to try LTM recovery multiple times while the intention was to support a single attempt.

Thus, we propose the following:

Proposal 4: RAN2 to confirm the multiple LTM recovery attempts is an issue and to discuss possible ways to fix it in specifications.

The simplest way to clarify the behaviour is to add a note stating that after a failed LTM recovery attempt the UE releases the LTM configurations, this is captured in Annex B. Another approach would be to add explicit procedural text, as shown in Annex C.

Proposal 5: To clarify the issue associated with the UE not releasing the LTM configurations after failed LTM recovery attempt, adopt either the TP presented in Annex B or the TP presented in Annex C.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: RAN3 asks if F1 interface should convey the information related to just the preamble index or more details, such as SSB index, PRACH mask index, and preamble index are necessary.

Observation 2: EarlyUL-SyncConfig conveyed in RACH Configuration sent over F1AP contains all necessary RACH-related parameters required by the source DU to send the PDDCH order.

Observation 3: As per RAN4 specification, PDCCH-ordered RACH requirements apply only for the UE that has provided L1- or L3-RSRP report (for FR1) or L1-RSRP report (for FR2).

Observation 4: As per RAN4 requirements, TCI state activation can be done only for the known TCI state or, in case of FR1, for unknown TCI state if the UE has reported L3 measurement result with the associated SSB index of the TCI state.

Observation 5: Figure 9.2.3.5.2-1 and the corresponding description in TS 38.300 were drawn prior to the RAN4 LTM requirements.

Observation 6: In CHO Recovery the candidate target cell’s configurations are deleted upon the CHO recovery attempt is initiated. In case of LTM, the configurations are released only if the LTM recovery procedure has failed. Otherwise, configurations are kept by the UE.

Observation 7: Due to the subsequent nature of LTM, based on the current specification, it seems the UE may be able to try LTM recovery multiple times while the intention was to support a single attempt.

Proposal 1: RAN2 responds to Question 1 by stating no additional information is needed in RACH Configuration sent over F1AP.

Proposal 2: RAN2 responds to Question 2 by stating ssb-PositionInBurst should be signalled in EarlyUL-SyncConfig over F1AP.

Proposal 3: RAN2 is asked to adopt the changes to TS 38.300 depicted in Annex A.

Proposal 4: RAN2 to confirm the multiple LTM recovery attempts is an issue and to discuss possible ways to fix it in specifications.

Proposal 5: To clarify the issue associated with the UE not releasing the LTM configurations after failed LTM recovery attempt, adopt either the TP presented in Annex B or the TP presented in Annex C.

# References

- R3-244793 LS on Early TA acquisition for the inter-DU scenario3GPP TSG-RAN WG3 Meeting #125 Maastricht, The Netherlands, 19 – 23 August 2024

- 3GPP TS 38.473 F1 application protocol (F1AP), V18.2.0 (2024-06)

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification, V18.3.0 (2024-09)

- 3GPP TS 38.133 NR; Requirements for support of radio resource management, V18.6.0 (2024-07)

- 3GPP TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2, V18.3.0 (2024-09)

# Annex A – TP for the order of actions in Stage-2 LTM message sequence

##### 9.2.3.5.2	C-Plane Handling

Cell switch command is conveyed in a MAC CE, which contains the necessary information to perform the LTM cell switch.

The overall procedure for LTM is shown in Figure 9.2.3.5.2-1 below. Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion. The general procedure over the air interface is applicable to SCG LTM. Further details of SCG LTM can be found in TS 37.340 [21].

Figure 9.2.3.5.2-1. Signalling procedure for LTM

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message to the gNB. The gNB decides to configure LTM and initiates LTM preparation.

2.	The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

3.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

4a.	The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command. The UE may activate and deactivate TCI states of LTM candidate cell(s), as triggered by the gNB and defined in [13], section 8.25.

4b.	The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB. When UE-based TA measurement is configured, UE acquires the TA value(s) of the candidate cell(s) by measurement. Upon L1-RSRP or L3-RSRP measurement report (as described in [13], section 6.2.2C, the UE performs early TA acquisition with the candidate cell(s) as requested by the network before receiving the cell switch command as specified in clause 9.2.6. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

5.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

6.	The gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

7.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

8.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

The steps 4-8 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 2.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to both intra-gNB-DU LTM and inter-gNB-DU LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].

# Annex B – Clarification on LTM configuration handling after LTM recovery failure

Ommited text not changed

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

2>	If the LTM cell switch procedure attempted based on attemptLTM-Switch fails:

3> release the ltm-Config IE;

# Annex C – Clarification on LTM configuration handling after LTM recovery failure

First change, ommited text not changed

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the relay (re)selection procedure, if ongoing;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:	It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

- If attemptLTMCounter is equal to zero; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2> 	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

2> Set attemptLTMCounter to one;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

1>	else:

2>	if UE is configured with attemptCondReconfig; or

2>	if UE is configured with attemptLTM-Switch; or

2> if  attemptLTMCounter is equal to one:

3>	reset MAC;

Remaining text not changed

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

Second change, ommited text not changed

2>	if the RRCReconfiguration is applied due to an LTM cell switch execution:

3>	submit the RRCReconfigurationComplete message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

3> reset attemptLTMCounter;

Remaining text not changed