3GPP TSG-RAN WG2 Meeting #127bis	R2-2408947

Hefei, China, 14 – 18 October 2024

Agenda item:	8.8.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining Issues for NTN over Regenerative Architecture

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this paper we present our view on the remaining aspects to resolve in order to support NTN over regenerative architecture.

# 2	Discussion

## 2.1	Satellite switching with re-synchronization

As part of Rel-18, RAN2 led the specification of the satellite switching with resynchronization feature. This feature, also known as “unchanged PCI” mobility, allows a UE to switch from one satellite to another one without carrying out an L3 handover (i.e., no dedicated RRC signalling is involved) under the conditions that gNB (which does not change) is on the ground and satellites will work as bent-pipe network nodes (i.e., transparent payload). Since the gNB does not change, the cell configuration can be maintained (i.e. SSB, SIB, paging and UE’s RRC configuration) after the satellite switching and no path switch is conducted at the AMF/UPF. The result is that RAN and CN require less signalling at the expense of a more complex NW implementation (i.e., the NW has less control over when UEs will be available in the target satellite and therefore resource allocation becomes more cumbersome).

Observation 1: Satellite switching with resynchronization was specified in Rel-18 to reduce radio signalling load.

In this contribution, we raise the following questions to analyse whether the satellite switching with resynchronization is yet feasible with a regenerative payload:

- In a regenerative payload, can the satellite switching feature provide similar benefits as in the transparent architecture?

- What would be the benefits (if any) of using the satellite switching instead of the existing RRC L3-level mechanisms for mobility?

- What changes (if any) are required to implement the satellite switching with resynchronization in a regenerative architecture?

As pointed in the first observation, one of the main benefits of the satellite switching with resync is the reduction in terms of signalling load under the assumption that the gNB does not change. In a regenerative architecture, given that the gNB is no longer on the ground, the single (physical) gNB assumption is also no longer valid, which means that the UE will frequently switch from a source gNB to a target gNB (i.e., this is called “inter-gNB mobility”) with the corresponding increase of signalling derived from, at least, the path switching at the AMF/UPF. Note that the latter is expected regardless of the mobility feature (e.g., during the CHO).

Observation 2: In a regenerative architecture, satellite switching under the assumption of single gNB is no longer valid since gNBs are on board of the satellites. The UE will frequently switch between gNBs, triggering inter-gNB mobility events and additional AMF/UPF signalling (e.g., for path switching).

In connection with the second question, as per the current specifications, connected mode mobility with a regenerative payload is already possible with basic mobility mechanisms such as the handover (HO) and the conditional HO (CHO).

Observation 3: Connected mobility with regenerative payloads is possible with existing mobility mechanisms, i.e. CHO and HO procedures.

Hereafter we analyse the differences between using the satellite switching with resync as compared to the existing mobility mechanism. The analysis uses Figure 1, which provides an overview of the regenerative architecture (including the interfaces connecting the different network components), as well as Figure 2 (see Appendix), which shows a possible implementation of the satellite switching with regenerative payload.

The first step for the satellite switching could start with the switching decision made by the Source gNB. In Step 2 and 3, Source and Target gNBs ‘agree’ on the satellite switch, which may be enhanced with exchange the information beneficial for the switching. As shown in Figure 1, this communication takes place over the Xn interface (either via inter-satellite link or routed through the NTN GW). Once the Source gNB is ready, it broadcasts the target satellite assistance information as part of SIB19 (same procedure as in Rel-18). Note that the preparation of the Target gNB could be done in advance and per cell instead of per UE to avoid a signalling storm.

Figure 1 NTN architecture with regenerative payload

On the UE side, the procedure may remain unchanged as compared with the transparent payload. The UE acquires SIB19 before the switching and initiates the resynchronization towards the target satellite upon t-Service (see Step 5 and 6). In this case, the switching occurs between gNBs. However, nothing prevents both gNBs to use the same cell configuration (i.e., virtually there could be only one cell).  This may require further orchestration and coordination between RAN nodes and OAM entities. Note that part of this coordination is anyway required for the L3 HO/CHO procedure (i.e., signalling exchange between source gNB and target gNB), while the UE benefits from skipping the reception of the HO command.

Observation 4: The source and target gNBs can use the same cell configuration.

Observation 5: The coordination between source and target gNBs may be similar as compared to the conventional L3 mobility, while UE and NW can benefit from a reduction of the signalling load by skipping the HO command.

An important aspect to consider is the security. In order to ensure data integrity and confidentiality, inter-gNB mobility procedures involve a change of security keys. After receiving the HO command from the source gNB, the UE updates its security context with new security keys derived based on the information received from the target gNB. As part of this process, the UE resets its PDCP entity, flushes data buffers, and establishes a new security context with the target gNB.

Given that these procedure is critical to avoid security risks, RAN2 should consider involving RAN3 and SA3 to analyse the implications and feasibility regarding the security aspects and the expected UE behaviour in case of pursuing the satellite switching with resync with a regenerative payload. We assume change of the gNBs (which will be an inherent part of satellite switching with resynchronization in regenerative architecture) cannot occur without updating the security key.

Observation 6: Specifying the satellite switching with resync with a regenerative payload requires involvement of RAN3 and SA3 to evaluate the security implications.

Considering that the security aspects are not compromised, once the UE has completed the satellite switching and the NW has acknowledged it, the satellite switching completion can be notified to the Source gNB by the target (Step 7). To finish the switching procedure, the user’s data must be re-routed. As in Step 9 of Figure 2, Target gNB and AMF/UPF convey a path switch so Source gNB can stop forwarding user’s data to the target. Finally, Target gNB sends a UE context release to the Source gNB.

As one may observe, the satellite switching with regenerative payload may re-use large part of the HO procedure (very similar signalling load). The identified benefit may be that the UE can avoid RRC Reconfiguration (and therefore the NW can also skip the transmission of this message). However, this may come at the expense of increased NW complexity in comparison to the HO procedure and enabling this feature may require specification changes to the security context management.

Observation 7: For satellite switching with regenerative payload, Rel-19 UE may benefit from similar gains as in transparent architecture (if security concerns are solved), while NW complexity may increase compared with regular L3 procedures (to be checked with RAN3 and SA3 for security issues).

Thus, as we do not see clear benefits compared to baseline L3 mobility, we propose the following:

Proposal 1: RAN2 does not pursue the work on satellite switching with resync in regenerative payload before RAN3 and SA3 resolve major issues related to this scenario.

Proposal 2: If RAN3 and SA3 found the satellite switching with resync to be feasible, RAN2 to discuss the potential benefits with respect to legacy mobility procedures. This is also subject to additional approval at the RAN Plenary level.

## 2.2	RACH-less handover

In Rel-18, RAN2 worked on the specification of the RACH-less HO for NTN (among other scenarios) where the focus was on intra-satellite mobility. The following text in TS 38.300 (Stage-2) describes the RACH-less HO, which restricts the mobility procedure to intra-gNB scenarios:

#### 9.2.3.6 RACH-less handover During intra-gNB HO procedure, RACH-less handover can be configured for a UE. The RACH-less handover procedure applies the following functionality: - The UE uses the same timing advance value at the target cell as in the source cell or timing advance value of 0. - The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell. - The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell. - The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

Observation 8: Stage-2 description is limited to intra-gNB HO procedure.

This restriction stems from the limited time left in RAN2 and RAN3 at that point in time, and the RAN1 feedback on the feasibility of the RACH-less HO procedure. In R2-2300020, RAN1 provided the response (quoted below) on whether RACH-less HO can be supported in intra/inter satellite and with/without feeder link switch:

RAN1 response For scenario (1), from RAN1 perspective the RACH-less handover is possible, assuming the following notes can be satisfied, when UE UL transmission synchronization can be maintained by applying pre-compensation using the assistance information, e.g., epoch time, ephemeris, common TA, of the target cell. For scenario (2)-(4), from RAN1 perspective the RACH-less handover may be possible, assuming the following notes can be satisfied, when UE UL transmission synchronization can be maintained by applying pre-compensation using the assistance information, e.g., epoch time, ephemeris, common TA, of the target cell. Note 1: RAN1 assumes that the RAN4 UL synchronization requirement specified in Table 7.1C.2-1 of TS38.133 applies to the first UL transmission in the target cell. Note 2: gNB is expected to provide valid assistance information of the target cell to UE. Note 3: gNB is expected to ensure the UE can perform the UL transmission while respecting common TA and UE processing time.

In their response, RAN1 stated that the intra-satellite HO without gNB switch is possible, while using a vague wording for the other cases (i.e., stating those “may be possible”).

Observation 9: According to RAN1, inter-satellite RACH-less HO with gNB switch may be possible.

In Rel-19, some companies have been proposing specification changes to accommodate the regenerative payload with the RACH-less HO procedure. As a result, during RAN2#127 meeting the following note was captured:

RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

In our understanding, this note is not sufficient, and it does not align with current Stage 2 description. RAN2 should revise the current Stage 2 description if other WGs (i.e. RAN3) confirm inter-gNB scenario is feasible for RACH-less HO in NTN. We consider that a first step to check the feasibility of this procedure with a regenerative payload, RAN2 should also ask RAN3 whether specification changes are required:

Proposal 3: RAN2 to revise current Stage 2 description and ask RAN3 for feedback on the feasibility of the NTN RACH-less HO with a regenerative payload.

The note also captured that “no further Uu enhancement is needed”, however, we think that some inherent aspects of a regenerative deployment could be discussed. One of the challenges of the procedure is the acquisition of the UL grant for the first UL transmission in the target node. The UE may acquire the UL grant via a configured grant included in the HO command or dynamically transmitted in the target cell. As indicated in Figure 3, for the latter the UE accesses the target cell and monitors the PDCCH channel waiting for the target cell to transmit an UL assignment. If the timing of when the UE will access the cell is unclear for the target cell this may lead to waste of resources. If the target cell transmits the UL grant too early (UE is not ready yet to acquire it), the cell may have to transmit the UL assignment for longer time wasting NW resources and increasing the risk of the HO failure. If the target cell transmits the UL grant too late, that may result in the UE monitoring the PDCCH for longer than required.

Observation 10: Target cell may underestimate/overestimate the appropriate time to transmit the UL assignment over PDCCH, which may lead to a waste of resources and increased power consumption at the UE’s side.

Figure 3 RACH-less HO with dynamic grant

In order to transmit the UL grant timely, the target cell has to estimate when the most appropriate time is. In Rel-18, only the transparent architecture was considered. In that case, assuming that serving and target cells belong to the same gNB located on the ground, the gNB can know when the source cell will stop serving the area and when the UE will access the cell. In Rel-19, with the specification of the regenerative payload, some issues may arise. Using the RACH-less HO procedure in a regenerative architecture means that serving gNB and target gNB are on board of separate satellites. In this case, the target gNB has no means to know when the UE may attempt cell access and its only time reference is when the HO Request ACK message was transmitted.

Observation 11: In regenerative architecture, target gNB has no means to know when the source gNB will stop serving the area and when the UE will access the cell.

One potential approach to reduce this uncertainty is to combine RACH-less HO with time-based CHO procedure. According to TS 38.423, as part of the HO Request, the source NG-RAN node can send to the target NG-RAN node the T1-T2 parameters (part of the condEventT1). In this case, the target gNB can make a better estimation given that it knows the time window when the UE will execute the HO (i.e., between T1 and T2).

Observation 12: In case of combining RACH-less HO and time-based CHO, the target gNB can better estimate when to transmit the UL assignment since it is aware of the T1-T2 time window.

However, we cannot assume that this is always possible (i.e., according to the specifications a RACH-less access may be combined with other mobility procedures such as the Release 16 CHO or baseline Release 15 HO), and therefore, RAN2 should provide means for all the configurations supported in NTN. For this reason, we propose the following:

Observation 13: The RACH less handover feature works for regenerative architecture, while few optimisations are possible, like informing the target gNB when the UE is supposed to be present in the target cell.

Proposal 4: RAN2 to discuss means to reduce target gNB’s uncertainty during RACH-less handover for regenerative architecture.

# 3	Conclusion

This document has made the following observations:

Observation 1: Satellite switching with resynchronization was specified in Rel-18 to reduce radio signalling load.

Observation 2: In a regenerative architecture, satellite switching under the assumption of single gNB is no longer valid since gNBs are on board of the satellites. The UE will frequently switch between gNBs, triggering inter-gNB mobility events and additional AMF/UPF signalling (e.g., for path switching).

Observation 3: Connected mobility with regenerative payloads is possible with existing mobility mechanisms, i.e. CHO and HO procedures.

Observation 4: The source and target gNBs can use the same cell configuration.

Observation 5: The coordination between source and target gNBs may be similar as compared to the conventional L3 mobility, while UE and NW can benefit from a reduction of the signalling load by skipping the HO command.

Observation 6: Specifying the satellite switching with resync with a regenerative payload requires involvement of RAN3 and SA3 to evaluate the security implications.

Observation 7: For satellite switching with regenerative payload, Rel-19 UE may benefit from similar gains as in transparent architecture (if security concerns are solved), while NW complexity may increase compared with regular L3 procedures (to be checked with RAN3 and SA3 for security issues).

Observation 8: Stage-2 description is limited to intra-gNB HO procedure.

Observation 9: According to RAN1, inter-satellite RACH-less HO with gNB switch may be possible.

Observation 10: Target cell may underestimate/overestimate the appropriate time to transmit the UL assignment over PDCCH, which may lead to a waste of resources and increased power consumption at the UE’s side.

Observation 11: In regenerative architecture, target gNB has no means to know when the source gNB will stop serving the area and when the UE will access the cell.

Observation 12: In case of combining RACH-less HO and time-based CHO, the target gNB can better estimate when to transmit the UL assignment since it is aware of the T1-T2 time window.

Observation 13: The RACH less handover feature works for regenerative architecture, while few optimisations are possible, like informing the target gNB when the UE is supposed to be present in the target cell.

And proposed the following:

Proposal 1: RAN2 does not pursue the work on satellite switching with resync in regenerative payload before RAN3 and SA3 resolve major issues related to this scenario.

Proposal 2: If RAN3 and SA3 found the satellite switching with resync to be feasible, RAN2 to discuss the potential benefits with respect to legacy mobility procedures. This is also subject to additional approval at the RAN Plenary level.

Proposal 3: RAN2 to revise current Stage 2 description and ask RAN3 for feedback on the feasibility of the NTN RACH-less HO with a regenerative payload.

Proposal 4: RAN2 to discuss means to reduce target gNB’s uncertainty during RACH-less handover for regenerative architecture.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- 3GPP TS 38.331 NR RRC, v18.2.0, July 2024

# Appendix

## A.1	Satellite switching with re-synchronization

Figure 2 Satellite switching with resynchronization in a regenerative architecture.