3GPP TSG-RAN WG2#127bis	R2-2408952

Hefei, China, 14-18 October 2024

Agenda Item:	8.2.2

Source: 	MediaTek Inc.

Title:  	Ambient IoT data transmission after initial access

Document for:	Discussion, decision

# 1	Introduction

This document discusses some issues related to AIoT data transmission outside the initial access procedure.

Data transmission as part of (or immediately following) the initial access procedure seems to be fairly well understood after RAN2#126: If the reader has R2D data, it may page the device and can then transfer data immediately after the initial paging/access exchange, while if the device has D2R data, it waits to be paged and can then deliver its data once the access procedure has completed.

We understand that “data-after-access” could be a regularly occurring scenario in many reasonable AIoT operations.  For instance, a firmware download would often require multiple write commands (or some form of data segmentation); retrieval of high-volume data like a period of sensor readings or a log might involve multiple read commands.  As it seems possible that the available transport block sizes will be small for some cases, we anticipate that it should be considered a normal scenario that the access procedure and immediately following messages (“Msg4”/“Msg5”) cannot carry all the data that need to be transferred.  Repeated operations like re-inventorying are also conceivable.

# 2	Discussion

## 2.1	R2D direction

If the reader has (or acquires) more R2D data for delivery shortly after a DT data transfer operation, it could of course re-page the device, the access procedure could be repeated, and the additional data could be delivered by the same procedure as the original data transfer.  However, this approach seems wasteful of bandwidth if the reader can directly address the device.  We will explore below the conditions for such addressing, but as a general principle, it seems desirable to support some form of “subsequent data transfer” between a reader and a device that have already communicated.

Proposal 1: Support transfer of data at least in the R2D direction after the completion of the initial access procedure and associated data transfer.

Proposal 1 implies that the device must remember a suitable identity, so that it can be subsequently addressed for data delivery.  It is clearly impractical (and probably insecure) to address the device always by its long permanent identifier, so these transmissions should be addressed by a shorter AS identity.  As discussed at previous meetings, we consider that this identity should be assigned by the reader (e.g., during the access procedure) for persistent use in the access stratum, rather than just using a random number drawn by the device during access, but even if the random number is used, the device needs to retain it and use it as a general-purpose AS identifier.  We understand this identifier to be the same as the “short temporary ID” discussed in SA2 ([1], section 6.13.2).

It is also necessary for timing to be maintained between the device and the reader (for any D2R transmission, including, e.g., an acknowledgement responding to R2D data).  This issue is under discussion in RAN1.  Our current understanding is that there may be a need for timing maintenance in the D2R direction, but the device will always be able to receive an R2D transmission provided it has enough energy.  Thus, we foresee no special requirement for timing availability for R2D transmissions, and we assume for the following proposal that there is no timing condition, but the general issue of timing will need to be revisited as RAN1 discussion progresses.

Proposal 2: Data can be transferred between the reader and device in the R2D direction as long as the device’s AS identifier (random number or value assigned by the reader) remains valid.  Additional conditions on timing may be added if required by RAN1 decisions.

## 2.2	D2R direction

Data transfer in the D2R direction requires some discussion, since only the DO-DTT model is supported in this release.  If a device has transferred some D2R data, still has a valid identifier and timing, and acquires more data to deliver to the reader, it is not clear that it should be allowed to transmit it without first receiving an inquiry from the reader (e.g., a new read command).  However, it seems reasonable that multiple segments of D2R data in response to a single request can be supported, as shown in figure 1.  This model could be necessary if the amount of data that can be requested in one read command exceeds the currently supportable D2R transport block size, which seems plausible especially in poor radio conditions where the device may not be able to send a large transport block.

(The read command is not shown explicitly in the figure; it could be in the paging message, in Msg2, or sent separately before step 4—though this last option raises the question of how the reader knows that contention resolution has finished and the command can be sent.)

Figure 1: Multiple D2R data segments (with acknowledgements)

Figure 1 shows each data segment requiring an acknowledgement.  Depending on the structure of the data transmissions, it might be possible for each data segment to have a “more data” indicator and the subsequent acknowledgement to indicate resources for the following transmission, but these details can be further discussed.

There seems to be no technical obstacle to allowing transmission in the D2R direction of new data provided the device has an AS identifier, timing, and available radio resources.  However, we consider this outside of the spirit of the SID (it goes beyond the DO-DTT model), and we do not propose to enable it as part of this study.

Proposal 3: Data can be transferred in the D2R direction after the completion of the access procedure when:

- the device’s AS identifier (random number or value assigned by the reader) remains valid; and

- timing is maintained; and

- radio resources in the D2R direction are available for the device’s use; and

- the concerned data were previously requested by the reader.

The exact conditions for data transmission will need to be revisited when the definition of timing and the procedures for radio resource allocation in the D2R direction are more clearly established, both with input from RAN1 required.

## 2.3	Other issues: acknowledgements, timing, and paging

The stop-and-wait model shown in figure 1 is an intentionally simple approach to data transfer.  The device should know when transmission N has been received, so that it knows when it can transmit transmission N+1.  (This operation is not directly related to segmentation in the MAC layer, which is under discussion already with RAN1 and SA2; a “transmission” could be a single upper-layer message or a shorter segment, depending upon the outcome of the work on segmentation.)

Proposal 4: The reader can acknowledge receipt of a data transmission in the MAC layer using a stop-and-wait procedure.  FFS if the R2D ack transmission also contains information to control transmission of a subsequent D2R data segment.

If data transfer in the D2R direction is needed, and the device’s timing has expired, a new access procedure is probably needed to re-establish timing.  (As noted above, this condition seems not to apply to the R2D direction.)  The expedient way to trigger this procedure would be a new paging message, transmitted when the reader has either R2D data for transmission or a trigger for retrieving D2R data.  (There is a potential corner case in which the device is transmitting segments of data in response to a read command and timing expires before all segments are transmitted; if this scenario can actually occur, the reader needs to detect it and recover.  The details can be discussed if it becomes clear that timing can expire under these circumstances.)

Proposal 5: If data transfer in the D2R direction is required while the device’s timing is expired, the reader pages the device to re-establish timing and trigger data delivery.  If timing expires during transmission of a series of D2R data segments in response to a read command, the reader is responsible for re-establishing contact with the device; details are FFS.

Note that expired timing need not mean an expired AS identifier.  If the reader needs to page a device for which the AS identifier is still valid, it should be able to page based on the AS identifier, to minimise the need to transmit the long permanent identifier over the air (both for data-size and potential security reasons).

Proposal 6: The reader can page the device by AS identifier if the AS identifier is valid.

It may be too early to define when the AS identifier/short temporary ID is considered valid, and some SA2 input seems needed.  If there is value in coordinating short identifiers between different readers, there would be impact to several layers, but if the AS identifier is used only between a device and a single reader, there could be use cases resulting in a device changing IDs frequently (e.g., in a warehouse or on a factory floor with multiple readers).  We thus tend to think that there is a benefit in being able to coordinate AS identifiers over time and among readers, but this is not entirely in RAN2 scope to do.  We suggest that the question of AS identifier validity be revisited as SA2 progress their work on device identifiers.

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: Support transfer of data at least in the R2D direction after the completion of the initial access procedure and associated data transfer.

Proposal 2: Data can be transferred between the reader and device in the R2D direction as long as the device’s AS identifier (random number or value assigned by the reader) remains valid.

Proposal 3: Data can be transferred in the D2R direction after the completion of the access procedure when:

- the device’s AS identifier (random number or value assigned by the reader) remains valid; and

- timing is maintained; and

- radio resources in the D2R direction are available for the device’s use; and

- the concerned data were previously requested by the reader.

Proposal 4: The reader can acknowledge receipt of a data transmission in the MAC layer using a stop-and-wait procedure.  FFS if the R2D ack transmission also contains information to control transmission of a subsequent D2R data segment.

Proposal 5: If data transfer in the D2R direction is required while the device’s timing is expired, the reader pages the device to re-establish timing and trigger data delivery.  If timing expires during transmission of a series of D2R data segments in response to a read command, the reader is responsible for re-establishing contact with the device; details are FFS.

Proposal 6: The reader can page the device by AS identifier if the AS identifier is valid.

# 4	References

[1] TR 23.700-13