3GPP TSG-RAN WG2#127bis	R2-2408953

Hefei, China, 14-18 October 2024

Agenda Item:	8.2.5

Source: 	MediaTek Inc.

Title:  	Ambient IoT topology 2 and out-of-coverage operation

Document for:	Discussion, decision

# 1	Introduction

This document discusses the topic of AIoT operation when a UE reader is out of Uu coverage in topology 2, including initial proposals on resource allocation mechanisms for the AIoT interface in such circumstances.  There is some overlap with the RAN1 topic of “control of intermediate UE”; however, we understand that this topic has not progressed substantially in RAN1, and it may be beneficial for RAN2 to spark some initial activity.

# 2	Discussion

## 2.1	Coverage scenarios and topology 2 vs. topology 4

It is difficult to guarantee that a UE-based reader never moves out of coverage while operating.  Even if the reader faithfully maintains itself in the nominal service area of a base station, radio fluctuations and obstructions can happen, resulting in temporary out-of-coverage situations.  This is a different scenario from topology 4, which envisions a reader and device communicating without the supervision of a network ([1]).

Observation 1: A UE reader can be expected to lose Uu coverage in some circumstances.

From a practical point of view, a reader in such a temporary situation should not simply abort an operation that it would be able to complete.  As long as the reader is aware of what radio resources it can use on the AIoT interface, there is no functional obstacle to short-term store-and-forward operation—delivering a DT transmission to a device even if Uu coverage has been lost since the data were received from the controller, or receiving a DO-DTT transmission from the device and waiting to forward it until the reader is back in Uu coverage.

Observation 2: Operations on the AIoT interface can reasonably be expected to complete even if the reader loses Uu coverage.

It should be noted that RFID has no network coverage requirement; RFID tags can be read anywhere, and a restriction to operate only with a cellular network already limits the applicable use cases for (the first release of) ambient IoT as compared to RFID.  While this restriction seems to be a concession to practical standardization timeline requirements, it is important to avoid having the constraints of the first release interfere with later functionality.

Proposal 1: Any AIoT design recommended as a result of the Rel-19 SI should allow the reader to go temporarily (e.g., on the order of the NAS inactivity timer) out of coverage without breaking AIoT application functionality.

Proposal 2: Any AIoT design recommended as a result of the Rel-19 SI should be extendable to a future release supporting topology 4.

## 2.2	Resource allocation issues

Among the principal functions that a UE reader could be expected to need from the base station is management of radio resources on the AIoT interface.  As noted in the previous section, this management is important for OOC operation, since the reader needs to know what resources it can use on the AIoT interface during the OOC period.

Even in coverage, constant “phone home” requests from the reader to the gNB for a new AIoT “grant” for every transmission, like sidelink mode 1, are wasteful of Uu resources and increase the latency of AIoT transmissions.  Moreover, the gNB cannot be expected to have detailed insight into where resource conflicts can occur; the short range of AIoT communication means that spatially separated readers can reuse the same radio resources, but the gNB has only a general knowledge of which reader is where (e.g., TRP level), so it is somewhat pointless to offload radio resource allocation to the gNB at the transmission-by-transmission level.

Observation 3: Frequent communication between the reader and the base station for resource grants on the AIoT interface is inefficient as well as being not compatible with temporary OOC operation.

Accordingly, there should be a mechanism for short-term persistent resource allocation, i.e., a grant of resources from the gNB to the UE reader that can be used for some period of time without being explicitly renewed for every transmission.

Proposal 3: The UE reader does not need to request from the gNB an individual grant of radio resources for each transmission on the AIoT interface.

The details of such a mechanism can be discussed.  We see two basic alternatives: (1) a resource-pool scheme in which the gNB configures the reader with a set of radio resources that it can use freely for some time, and (2) a configured-grant scheme in which the gNB configures resources for several transmissions in a single assignment.

In a resource-pool approach, the reader sends one request to the gNB on the Uu interface, containing some information about the expected transmissions, and the gNB answers with a pool of radio resources having a limited validity time.  For topology 2, the validity time would need to be quite short (e.g., on the order of the NAS inactivity timer), to prevent the problem of a reader wandering away and using previously assigned resources in a place where they would cause interference.

In a configured-grant approach, the reader similarly sends one request to the gNB on the Uu interface, but the gNB responds with a specific grant of radio resources on the AIoT interface, e.g., a periodic assignment covering a fixed time and/or a fixed number of transmissions.

Proposal 4: RAN2 further discusses the details of possible approaches to a short-term persistent assignment from the gNB to the UE reader of radio resources on the AIoT interface.

In the context of the coverage concern described above, a short-term persistent mechanism also means that the reader is not dependent on continuous coverage.  An interruption in coverage while the assigned resources are valid does not interfere with device/reader communication, and data can be received from or forwarded to the network when the reader comes back in coverage.  If the lifetime of the resources expires while the reader is still out of coverage, of course, it has no way to obtain more AIoT radio resources and has to stop operation—this characteristic distinguishes the temporary interruption of coverage from topology 4.

# 3	Conclusion

This document made the following observations:

Observation 1: A UE reader can be expected to lose Uu coverage in some circumstances.

Observation 2: Operations on the AIoT interface can reasonably be expected to complete even if the reader loses Uu coverage.

Observation 3: Frequent communication between the reader and the base station for resource grants on the AIoT interface is inefficient as well as being not compatible with temporary OOC operation.

The following proposals are given:

Proposal 1: Any AIoT design recommended as a result of the Rel-19 SI should allow the reader to go temporarily (e.g., on the order of the NAS inactivity timer) out of coverage without breaking AIoT application functionality.

Proposal 2: Any AIoT design recommended as a result of the Rel-19 SI should be extendable to a future release supporting topology 4.

Proposal 3: The UE reader does not need to request from the gNB an individual grant of radio resources for each transmission on the AIoT interface.

Proposal 4: RAN2 further discusses the details of possible approaches to a short-term persistent assignment from the gNB to the UE reader of radio resources on the AIoT interface.

# 4	References

[1] TR 38.848