3GPP TSG RAN WG2 Meeting #127bis		R2-2408954

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.3

Source:	Sharp

Title:	Duplicate A-IoT paging message transmission and indication

Document for:	Discussion and Decision

- Introduction

In RAN2 #127, it was agreed that a reader can send multiple ambient IoT(A-IoT) paging messages for the same request from the core network.

Agreements

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we discuss the triggering conditions for a dupplicate paging message transmission, or paging retransmission, and the required information to differentiate a supplicate paging from a new paging message.

- New or duplicate indication in a paging message

A paging message is the first message from a reader to initiate any AIoT initial access or data transmission. Different paging functions may be defined for unicast, broadcast, group cast and multicast, etc. At least a 2-step and a 3-step random access procedures should be supported for “inventory only” and “inventory and command” use cases, as discussed in RAN2.

A reader can send multiple ambient IoT(A-IoT) paging messages for the same request from the core network, i.e. the same paging process. Therefore, some information is needed to indicate the duplicate paging message by the reader, so that the A-IoT devices can detect whether a paging messaage is new paging or a duplicate message of an ongoing paging. Depending on the type of paging, an A-IoT device should be able to determine whether a new random access procedure should be performed or not.

To identify a paging message, as a minimum, a paging ID can be included in a paging PRDCH Msg0. The paging ID can be increased by one for each new paging process and cyclically reused based on the ID length.

If there are multiple AIoT readers, it is possible more than one reader may select the same paging ID at the same time and cause paging ID collision. If the paging ID is big enough, the collision probability can be negligible. To avoid the ambiguity of paging from different readers, a reader ID may be included in the paging PRDCH Msg0 together with the paging ID. The combination of paging ID and reader ID may define a unique paging process. An AIoT device can differentiate a different paging process from different readers as well based on the reader ID.

To differentiate an initial paging message from a duplicate paging message, a paging re-transmission counter may be included in the paging message for a given paging ID. The paging re-transmission counter is reset to 0 for each new paging process with a new paging ID, and increased by 1 for each paging message re-transmission for the same paging ID. Alternatively, new and duplicate paging can be indicated by 1 bit, e.g. 0 for initial transmission and 1 for retransmission.

An AIoT device may determine the paging message is a re-transmission of an existing paging process by the paging counter. If the counter is 0, the paging Msg0 is a new paging message, and if the counter is greater than 0, the paging message is a re-transmission of the same paging process.

Furthermore, the AIoT device may determine whether one or more paging messages is(are) not detected by observing if some paging counters are missed.

Proposal 1: A paging message should include a paging ID and a paging retransmission counter for new and duplicate paging indication.

- Triggering conditions for duplicate paging messages

For a paging that triggers contention based random access (CBRA), the AIoT device will perform contention access with slotted Aloha in a contention access region. Each contention slot can be defined as a random access occasion (RO). The reader listens to the channel for Msg1 reception in each RO, and may have three possible observations and reception status:

- Successfully receive and decode a Msg1 with a randomID and/or actual ID

- Detected energy with Msg 1 transmissions, but cannot decoded anything successfully. Thus implies a collision of Msg1 occurs, i.e. more than one AIoT devices transmit Msg1 in the same RO.

- Idle. No energy detected in the RO, i.e. no AIoT device selects this RO for Msg1 transmission.

An AIoT reader may need to send multiple paging messages for the same paging process. The reader may decide to retransmit Msg0 for a variety of reasons. For example, the reader may send multiple paging messages at different directions to cover more AIoT devices, or the reader may expect more AIoT responses.

In some cases, the reader is required to send a duplicate paging message if a collision is observed in the CBRA procedure of the previous paging message, especially:

- If the reader detects collisions in one or more ROs, a duplicate paging message will allow the AIoT devices involved in the collision to retransmit Msg1 for collision resolution.

- If the reader detects more than one AIoT devices reported the same random ID. A duplicate paging message should be triggered for these devices to retransmit with new random IDs for collision resolution.

Thus, the status of each RO in the contention access region can be observed and obtained at the reader. And the reader can determine whether a duplicate paging message should be transmitted for the same paging process, or a new paging process should be initiated.

Proposal 2: A duplicate paging message should be triggered at least when

- Channel collision is detected in one or more ROs in the contention access region, or

- Same random ID is reported from more than one AIoT devices.

- Conclusions

In this contribution, we discuss the methods to indicate a new or a duplicate paging message, and the conditions that a duplicate paging message should be transmitted. We propose

Proposal 1: A paging message should include a paging ID and a paging retransmission counter for new and duplicate paging indication.

Proposal 2: A duplicate paging message should be triggered at least when

- Channel collision is detected in one or more ROs in the contention access region, or

- Same random ID is reported from more than one AIoT devices.

- References

- Draft Report of 3GPP TSG RAN WG2 meeting #127, v1