3GPP TSG-RAN WG2 Meeting #127bis	R2-2408960

Hefei, China, Oct 14th – 18th, 2024

# Agenda item:	8.6.4

# Source:	ETRI

# Title:	Discussion on conditional intra-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#105 meeting, objectives of WI LTM were revised as follow and it was agreed to start RAN WG work on the support of conditional intra-CU LTM [1]:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this paper, we discuss some issues to be considered to support conditional intra-CU LTM.

- Discussion

  - Stage2 signaling flow

Since a conditional LTM is limited to intra-CU LTM case according to the revised WID [1], the stage-2 signaling procedure for intra-CU LTM in Rel-18 can be taken as a baseline for conditional intra-CU LTM signaling procedure. The main difference between conditional LTM and intra-CU LTM is which node determines a cell switch to a target cell. A source cell determines the target cell based on the L1 measurement report from a UE in the intra-CU LTM, while the UE determines the target cell based on the measurement results and execution conditions configured by the source cell in the conditional LTM. The other procedures such as LTM preparation, early synchronization, LTM cell switch completion can be reused for the conditional LTM as well. The following figure illustrates the proposed stage-2 signaling procedure for the conditional intra-CU LTM.

Figure 1. Signaling procedure for conditional intra-CU LTM

As in the intra-CU LTM, the source gNB determines conditional LTM candidate preparation based on L3 measurement report from the UE. The source gNB prepares the candidate cell configurations for conditional LTM and sends RRCReconfiguration message to the UE, containing the configuration of candidate conditional LTM cells. But, unlikely as in the intra-CU LTM, the source cell provides the execution conditions for LTM cell switch along with the candidate cell configurations in this step so that the UE can determines when it switches to which cell.

Proposal 1: During conditional LTM preparation phase, the source gNB provides the execution conditions for each candidate cells along with the candidate cell configurations so that the UE can determine when it switches to which cell.

In the intra-CU LTM, two kinds of TA measurement methods are supported for UL early synchronization, PDCCH ordered TA measurement and UE based TA measurement. In PDCCH ordered TA measurement, the UE sends a preamble to the candidate cell and the candidate cell calculates the TA based on the received preamble. The calculated TA is sent to the source cell and subsequently managed by the source cell. When the source cell determines a cell switch, it sends the UE the cell switch command MAC CE, which includes the TA. However, the cell switch command MAC CE is not used in the conditional LTM because the cell switch is triggered by the evaluation result of the execution condition, not by the cell switch command MAC CE. If the UE can’t acquire the TA before the cell switch, it shall perform a normal random access during the cell switch, which will increase the interruption during LTM cell switch.

Since early TA acquisition is an effective method for reducing interruptions during cell switching, it should also be supported for conditional intra-CU LTM. In the conditional LTM scenario, the source cell is unaware of when the UE will initiate a cell switch. Therefore, the source cell should provide the UE with the TA in advance before the cell switch occurs. For instance, the source cell may trigger the UE to perform the early TA acquisition procedure after completing candidate cell configuration preparation and deliver the TA to the UE as soon as it is received from the candidate cell. In this case, the management of the TA follows the same approach as for inter-CU LTM. The source cell manages the validity of the TA and requests the UE to repeat the early TA acquisition procedure if the TA becomes invalid while the UE is still connected to the source cell. There are two possible options for providing the TA to the UE. The first is to define a new MAC CE for this purpose, and the second is to reuse the CSC MAC CE with an indicator which notifies that this MAC CE is for delivering TA, not for a cell switch.

Proposal 2: PDCCH ordered TA measurement is supported for conditional intra-CU LTM and how to provide the TA to the UE is FFS.

As an alternative method for early TA acquisition, RAR-based TA measurement can be considered. During RAN2#126, it was agreed not to support RAR-based TA acquisition for inter-CU LTM, but its use for conditional LTM is still FFS. RAR-based TA acquisition may help reduce data interruptions between the UE and the target cell during cell switching. However, a data transmission/reception interruption with the source cell occurs during the UE receives the RAR from a candidate cell. Additionally, the UE performs this process with multiple candidate cells, which could result in greater interruptions than those mitigated during the cell switch.

Proposal 3: RAR-based TA measurement is not supported for conditional intra-CU LTM.

UE based TA measurement is another early UL synchronization method supported for Rel-18 intra-CU LTM. Since there is no difference in the network architecture and the scope of cell switching between conditional intra-CU LTM and Rel-18 intra-CU LTM, we see no reason not to support UE based TA measurement in the conditional intra-CU LTM as well.

Proposal 4: UE based TA measurement is supported for conditional intra-CU LTM.

After UE sends an RRCReconfigurationComplete message to the source gNB, it starts evaluating the conditional LTM execution conditions for the candidate cells. If at least one conditional LTM candidate cell satisfies the corresponding execution conditions, the UE detaches from the source cell, applies the stored corresponding configuration for that selected candidate cell and synchronizes to that candidate cell. The UE performs random access procedure towards the target cell if UE does not have valid TA of the target cell.

The UE completes the conditional LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell as in the intra-CU LTM. For RACH-based LTM cell switch, the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less conditional LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data

Proposal 5: the LTM cell switch procedure for conditional intra-CU LTM follows the same procedure as defined for intra-CU LTM.

Proposal 6: Signaling procedure in Figure 1 is take as the starting point for the discussion of conditional intra-CU LTM signaling procedure.

  - Mixture of conditional LTM and intra/inter-CU LTM

Conditional handover is introduced to reduce the possibility of RLF during handover and can be configured with a normal L3 handover. For example, a source cell can send a HO command to a UE after configuring the UE with candidate cell configurations for CHO. The UE will follow the L3 handover procedure if it receives HO command before any CHO execution condition is satisfied. However, if a CHO execution condition is satisfied before the HO command is received from the source cell, the UE will proceed with the CHO procedure.

Similar to the L3 conditional handover, conditional intra-CU LTM can be configured to reduce the probability of RLF during cell switching when intra-CU LTM is active. As described in section 2.1, there is no significant difference between conditional LTM and intra-CU LTM. In both cases, multiple cells within the same CU are prepared for potential cell switching, enabling subsequent switches among these prepared candidate cells without the need for RRC messaging. Since the switch is restricted to intra-CU, a cell switch in one LTM type does not affect the candidate cell configurations of the other LTM type. Consequently, the UE does not need to release the conditional LTM cell configuration after an intra-CU LTM and should also retain the conditional LTM cell configuration following a conditional LTM for subsequent LTM, unless explicitly released by the network.

Proposal 7: conditional LTM and intra-CU LTM can be configured simultaneously.

Proposal 8: a mixture of subsequent conditional LTM and intra-CU LTM after a conditional or intra-CU LTM cell switch should be supported.

Proposal 9: The UE keeps its conditional LTM cell configuration after a conditional LTM or intra-CU LTM for subsequent conditional LTM unless there are explicitly released by a network.

In the RAN2#125-bis meeting, it was agreed that the UE can be configured with a mixture of intra-CU and inter-CU candidate cells. If the previous proposals for supporting a mixture of conditional LTM and intra-CU LTM are agreed, the possibility of supporting a mixture of conditional LTM and inter-CU LTM should also be considered. There are several considerations when dealing with this combination. If an inter-CU LTM cell switch occurs while both conditional LTM and inter-CU LTM are configured, the target cell in a different CU becomes the new source cell. Since conditional LTM is restricted to cells within the same CU, the current conditional LTM configurations become irrelevant if the CU for the conditional LTM cells differs from the CU of the new source cell. Several options can be possible for handling conditional LTM cell configurations in this scenario:

- Option 1: Disallow a mixture of conditional LTM and inter-CU LTM.
In this option, conditional LTM can be configured only with Rel-18 intra-CU LTM. If the candidate cell configurations include an inter-CU cell as a candidate for LTM, conditional LTM cannot be configured simultaneously.

- Option 2-1: Allow a mixture of conditional LTM and inter-CU LTM, but release the conditional LTM cell configurations.
Here, the UE releases its conditional LTM cell configurations if both conditional LTM and inter-CU LTM are configured and an inter-CU LTM cell switch occurs.

- Option 2-2: Allow a mixture of conditional LTM and inter-CU LTM, but suspend the conditional LTM cell configurations.
In this case, the UE suspends its conditional LTM cell configurations after an inter-CU LTM cell switch occurs, and the configurations remain suspended until it receives any requests from the network.

Proposal 10: RAN2 is kindly asked to discuss a mixture of conditional LTM and inter-CU LTM and how to handle conditional LTM cell configurations if an inter-CU cell switch occurs.

- Conclusions

In this paper, we discuss conditional intra-CU LTM related issue. Following are the proposals made in this paper:

Proposal 1: During conditional LTM preparation phase, the source gNB provides the execution conditions for each candidate cells along with the candidate cell configurations so that the UE can determine when it switches to which cell.

Proposal 2: PDCCH ordered TA measurement is supported for conditional intra-CU LTM and how to provide the TA to the UE is FFS.

Proposal 3: RAR-based TA measurement is not supported for conditional intra-CU LTM.

Proposal 4: UE based TA measurement is supported for conditional intra-CU LTM.

Proposal 5: the LTM cell switch procedure for conditional intra-CU LTM follows the same procedure as defined for intra-CU LTM.

Proposal 6: Signaling procedure in Figure 1 is take as the starting point for the discussion of conditional intra-CU LTM signaling procedure.

Proposal 7: conditional LTM and intra-CU LTM can be configured simultaneously.

Proposal 8: a mixture of subsequent conditional LTM and intra-CU LTM after a conditional or intra-CU LTM cell switch should be supported.

Proposal 9: The UE keeps its conditional LTM cell configuration after a conditional LTM or intra-CU LTM for subsequent conditional LTM unless there are explicitly released by a network.

Proposal 10: RAN2 is kindly asked to discuss a mixture of conditional LTM and inter-CU LTM and how to handle conditional LTM cell configurations if an inter-CU cell switch occurs.

- References

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4; Apple, China Telecom;

- Chair notes RAN2#127;