3GPP TSG-RAN WG2 #127bis	R2-2408962

Hefei, China, Oct 14th– 18th, 2024

Agenda Item:	8.5.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions for NES

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. The core part objective based on the revised WID is:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this paper, we address the third objective given above. In RAN2#125bis and RAN2#127 meetings it was only possible to discuss the adaptation of paging occasions within the context of this third objective. For the RACH adaptation in spatial domain, it was agreed, during RAN2#126, that RAN2 will count on RAN1 evaluations and the conclusion. For reference regarding the progress of the discussion in RAN1 on the third objective, please see the list of RAN1 agreements in the Appendix.

# 2	Discussion

In this section, we consider the following aspects:

- adaptation of SSB signals,

- adaptation of PRACH occasions and related signaling, and

- adaptation of paging.

### 2.1 Adaptation of SSB signals

Currently, SSBs are specified as always-on transmissions whose periodicity can be changed semi-statically. A legacy NR UE may assume a default SSB periodicity of 20ms during initial cell search. Even though the frequency of most broadcast transmissions can be minimized (e.g., with a periodicity of 160ms or larger), SSB transmissions for UE initial cell search are required to be provided every 20ms regardless of whether there is light/no traffic in the cell. This leads to energy consumption in the network, which can be minimized by adapting the SSB periodicity or skipping SSB transmissions on certain occasions.

In TR 38.864 [2], the evaluation results for adaptation of SSB periodicity were captured. The results show that significant energy savings can be achieved on the network side with such adaptation. For example, Figure 1 shows how much energy saving can be achieved only by changing the periodicity of SSB transmissions from 20ms to 40ms. Note that the setup assumes that the other transmissions/receptions are minimized (e.g., SIB1/PRACH is 160ms) to show the full potential of such SSB adaptation.

Figure 1: NW energy saving (NES) when SSB periodicity is increased from 20ms to 40ms.

- Extending the periodicity of SSB transmissions in a cell can provide significant energy savings for the network.

In Rel-19, we think that UEs in RRC connected mode supporting the feature can be informed dynamically about the change in the periodicity of SSB transmissions. SSBs can, for example, be provided sparsely as a baseline, and upon need, such as prior to/during handover, positioning services (E-CID) or alike, additional SSBs can be provided on-demand. For UEs in RRC idle/inactive mode, having a longer SSB periodicity e.g., > 20ms as a baseline, and activating additional SSBs upon need can also be considered.

Support additional SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of baseline SSB transmissions in RRC connected mode.

For UEs in RRC idle/inactive mode, a non-initial-access cell may have SSB periodicity of 160ms, as captured in existing specifications:

SSB-MTC2-LP-r16 ::= SEQUENCE {

pci-List SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need R

periodicity ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

For example, if a capacity cell is overlapped by a coverage cell, the SSB transmission rate of the capacity cell may be every 160ms while the UE will be able to find the overlapping coverage cell for initial access as it is providing SSBs every 20ms. Then through neighbor cell information (SSB-MTC2-LP) provided by the coverage cell the UE may reselect to the low-rate SSB capacity cell.

Figure 2: Deployment according to legacy specifications in which a coverage cell providing initial access functionality provides SSB every 20ms, while the overlapped capacity cell (non-initial-access) provides SSBs every 160ms.

- In legacy specifications the periodicity of SSB can be 160ms for non-initial access cells.

However, in practice such low-rate SSB cells are not deployed. The main reason is that there may be UEs, for example UEs in poor coverage, in need of multiple SSBs (up to 3 SSBs) before such UEs can decode the paging message. This is visualized in Figure 3, in which case the UE needs to wake up more than 320ms ahead of the PO for processing the required number of SSBs. Note that PEI would not have helped in this scenario as the PEI occasion would still be 3 SSBs ahead and upon paging the UE would not be able to go back to deep sleep until the PO.

Figure 3: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO.

Instead, additional SSBs (in addition to a baseline SSB transmission rate) can be transmitted on demand prior to the POs for Rel-19 UEs in RRC Idle/Inactive. Through such scheme, which is backwards compatible as legacy UEs can rely on the baseline SSB transmissions, it would then become practical to deploy such cells that are configured with low rate SSB transmissions. Hence, we think that adaptations to SSBs in capacity cells should be supported also in RRC Idle/inactive.

- Capacity cells are typically not configured with SSB transmissions with low periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding.

Figure 4: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO if SSB adaptations are done. The UE may choose to wake up closer to PO when additional SSBs are provided.

- Adapting SSBs for UEs in Idle/Inactive mode such that additional SSBs are added on top of low-periodicity SSBs prior to POs makes deployment of such cells with low-rate SSB transmissions practical.

- If SSBs are transmitted with 20ms periodicity for UEs in Idle/Inactive mode, the energy savings from other techniques, regardless of RRC state, may become ineffective.

In RRC Idle/Inactive mode, on top of baseline SSB configuration, additional SSBs can (dynamically or semi-statically) be provided on-demand, e.g., additional SSBs are transmitted prior to PO.

Dynamic SSB adaptations on PCell in RRC Connected mode is beneficial for multiple scenarios such as for fast moving UEs, during handover, positioning, etc. Similar as agreed for objective 1 for OD-SSB provision on Scells via MAC-CE signalling, we think that a MAC-CE based mechanism could be used for PCell to adapt the SSB burst periodicity for connected mode. We think the values supported in current specifications (between 5ms and 160 ms) are sufficient for such adaptation. The 160ms periodicity ensures that a UE can operate in the network and gives the network the opportunity to enter deep sleep state. SSBs with 160ms could then be the baseline configuration. When needed, additional SSBs can be provided on top of the baseline configuration as often as up to every 5ms for optimal performance.

For connected mode, support dynamic adaptation of SSB in time domain based on MAC CE indication. The signaling should be common with Objective 1.

It shall be noted that one important scenario relevant for SSB-adaptations on PCell is for mobility, where additional SSBs from neighbour cells are transmitted when the UE approaches the cell border, or positioning, where additional SSBs from neighbour cells improve the measurement accuracy and reduce the measurement delay. When it comes to neighbour cells, a gNB can specify the neighbouring active (present) SSBs via the parameter ssb-ToMeasure and the associated rate/periodicity via the SSB Measurement Timing Configuration (SMTC), which defines the time window during which the UE measures the SSBs belonging to these neighbouring cells. SMTC of neighbour cells need to cover the additional SSBs for a UE to benefit from SSB adaptation on neighbour cells. That is, in addition to the two SSB configurations and the suggested signalling, the SMTC configuration for a UE needs to be dynamically adapted accordingly.

- Dynamic adaptation of SSB on neighbour cells need to be accompanied by corresponding dynamic adaptation of SMTC.

Introduce mechanism for dynamic SMTC adaptation.

### 2.2 Adaptation of PRACH occasions and the related signaling

Another important periodic/always-on activity for a cell that supports initial access is PRACH occasions and their periodicity. Similar to the discussion on SSB transmissions, the frequency of PRACH occasions can be changed dynamically rather than only semi-statically as specified today. These resources cannot be dynamically adjusted to accommodate changes in traffic load or save energy at the gNB. The PRACH reception on such cells must continue quite frequently even when the cell has no/light traffic. Dynamic adaptation of the PRACH occasions in the time domain, i.e., without using the system information update notification mechanism, allows energy saving on the network side while not compromising the initial access performance.

In TR 38.864 [2], the evaluation results for adaptation of the frequency of PRACH occasions were captured. The evaluations show that significant energy savings can be achieved by adapting the frequency of PRACH occasions. For example, Figure 5 shows how much energy savings can be achieved by changing the periodicity of PRACH occasions between 10ms and 40ms. Note that it is assumed in the setup that SSBs are broadcast every 20ms (as in legacy) and SIB1 is broadcast every 40ms.

Figure 5: 10ms PRACH occasion periodicity (left), versus another with 40ms PRACH occasion periodicity (right).

- Changing the periodicity of PRACH occasions in a cell can provide significant energy savings for the network.

In Rel-19, we think that UEs in RRC connected mode supporting the feature can be informed dynamically about the change in the periodicity of PRACH occasions. For UEs in RRC idle/inactive mode, changing the periodicity of PRACH occasions can also be considered. This can be realized by introduction of a second/additional configuration for PRACH occasions for Rel-19 UEs and dynamic signalling, such as DCI, can be used to activate these additional PRACH occasions per need basis. This would be backwards compatible as legacy UEs can continue to use such carrier when legacy PRACH configuration is provided as part of system information broadcast.

Support activation of additional PRACH occasions dynamically for UEs in RRC idle/inactive and connected modes

When it comes to signalling, the dynamic adaptation of additional PRACH resources can be achieved without requiring extra wake-up for UEs. For idle/inactive UEs, this adaptation can be communicated during their paging occasions (e.g., in paging DCI) and included as part of SIB1 (e.g., in SIB1 DCI), as paging and SIB1 reading are already part of the UEs’ regular idle/inactive mode activities. Another option is to use PEI, though not all networks or UEs may support it. We believe that providing the indication in both paging occasions (PO) and SIB1 is advantageous, as it efficiently addresses both new UEs entering the cell and reading SIB1, as well as existing UEs camped on the cell and monitoring their paging occasions.

- UEs in Idle/Inactive entering a new cell read SIB1 upon (re-)selection, UEs in Idle/Inactive already camped on a cell monitor their paging occasions.

Support adaptation mechanism for additional PRACH resources for Rel-19 UE in Idle/inactive mode in paging occasions, and SIB1 occasions.

UEs in connected mode monitor a paging occasion at least once per modification period, see from 38.331, section 5.2.2.2.2:

“UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.”

If paging DCI supports an adaptation mechanism for additional PRACH resources for both idle and connected UEs, it simplifies dynamic control of RACH resources and improves signaling efficiency by using the same signaling for both scenarios. Furthermore, it does not require additional wakeup for UEs as they anyways receive this DCI.

- A unified signalling method for adapting additional PRACH resources for both idle and connected UEs simplifies the dynamic control and enhances signalling efficiency by utilizing the same signalling for both scenarios.

Support adaptation mechanism for additional PRACH resources for Rel-19 UE in connected mode in paging occasions.

### 2.3 Adaptation of paging

The adaptation of paging was discussed during RAN2#125bis and RAN2#127, and the following agreements were made:

Agreements on paging adaptation (RAN2#125-bis):

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Agreements on paging adaptation (RAN2#127):

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

The motivation for the existing framework of paging messages is to distribute the paging frames/occasions (PFs/POs) evenly across DRX cycles in idle/inactive modes. This is to balance the load not only from gNB perspective in a cell, but also from UE perspective to receive the paging message within a period intended based on the configured DRX cycle. From network energy saving perspective, such framework does not provide many opportunities for a gNB to sleep. It may become even more challenging to achieve energy savings when paging messages are broadcasted in all cells in a tracking area.

One of the aspects that was discussed during the RAN2#125bis meeting is the compatibility of legacy RRC_IDLE/RRC_INACTIVE UEs in the case of paging adaptations. The first question that was raised is whether a network should prevent the access of legacy UEs via barring in the case of a cell that uses paging adaptations. This, of course, depends on which solutions are chosen for supporting paging adaptations. Our opinion is that the legacy UEs should not be barred for the purpose of paging adaptations. Therefore, minimizing the impact on legacy UEs should be one of the important criteria that RAN2 takes into consideration when designing paging adaptation mechanisms.

Legacy UEs are not barred in a cell where PF/POs are adapted for Rel-19 UEs.

The second question that was raised is whether the Rel-19 UEs should be allowed to use legacy PFs/POs along with the new Rel-19 PFs/POs. Our opinion is that the network should have a flexibility to decide whether Rel-19 UEs can use only Rel-19 PFs/POs or both Rel-19 PFs/POs and legacy PFs/POs. For example, an operator may want to control the load on different POs/PFs depending on estimated population of legacy vs Rel-19 UEs. On one hand, in case of high legacy UE paging load, the network may want to configure the Rel-19 PFs/POs to use only Rel-19 PFs/POs such that the congestion on legacy paging resources is minimized. On the other hand, in the case of a low legacy UE paging load, the network may want to configure the Rel-19 PFs/POs to use both Rel-19 PFs/POs and legacy PFs/POs such that the utilization of legacy PFs/POs is maximized.

It should be possible for the network to configure Rel-19 UEs to use legacy PFs/POs, i.e., while some other Rel-19 UEs use Rel-19 PFs/POs or configure all Rel-19 UEs to use Rel-19 PFs/POs only.

In what follows, we discuss different paging adaptation mechanisms, and their pros and cons.

For the purpose of understanding how the PF and PO for paging are determined in the current specifications, we recall the following formulas from the TS 38.304 [3]:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N). 							(1)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns,										(2)

where

T: DRX cycle of the UE,

N: number of total paging frames in T,

Ns: number of paging occasions for a PF,

PF_offset: offset used for PF determination, and

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

- 5G-S-TMSI mod 4096

else:

- 5G-S-TMSI mod 1024.

According to the outcome of RAN2#125bis meeting, there are two different potential solutions that RAN2 down-selected for further study of the adaptation of paging occasions in time domain.

Solution a): One of the potential solutions is to bundle PFs, i.e., to configure legacy PFs sparsely and to define new PFs for Rel-19 UEs that can be configured consecutively to the legacy PFs. Figure 6 shows a comparison of legacy (Rel-18) PF/PO configuration to a potential Rel-19 PF/PO configuration. As illustrated in this figure, legacy UEs can be supported using the existing PF/PO configuration that is complemented by the Rel-19 PF/PO configuration.

Figure 6: Legacy PF/PO configuration example (upper figure) vs proposed Rel-19 PF/PO configuration (bundling PFs shown in the lower figure) providing more gNB sleep opportunities.

It is important to observe that the impact on UE reachability can be avoided by bundling the PFs/POs without affecting the length of a paging cycle. This way, the bundling of PFs/POs would affect only the distribution of the PFs/POs on the timeline, but reachability, i.e., paging cycle length, will not be affected. Furthermore, legacy UEs would be still able to use a cell with bundled PFs as illustrated in Figure 6, and the NW can configure Rel-19 UEs to use only Rel-19 PFs/POs or both Rel-19 and legacy PFs/POs, depending on estimated population of legacy vs Rel-19 UEs. Therefore, we think that it would be beneficial to bundle PFs/POs. Note, however, that we do not intend to introduce a dynamic adaptation mechanism due to its complexity during updates, especially if some POs would be deactivated. We think that semi-static configurations and updates via system information update notification, as in legacy, are good enough.

One of the questions that was discussed during RAN2#127 meeting and a subsequent email discussion is how bundling of PFs/POs can be achieved, e.g., how a Rel-19 UE can calculate the SFN of a PF that should be monitored for paging. Eight different modifications of the legacy formula (1) for calculating SFN were considered for calculating the SFN of the bundled PFs. None of these formulas, however, ensure that the bundled PFs are configured while taking into consideration legacy PFs, which has two issues.

- The paging formulas proposed so far in RAN2 do not consider legacy PFs.

First, if bundled Rel-19 PFs are not consecutive to legacy PFs, the possibility for NW energy savings may be jeopardized, e.g., if the legacy and Rel-19 configurations result in Rel-19 PF bundles that are frames apart from legacy PFs. In this case, the NW would wake up for the purpose of legacy PFs, have an opportunity for a short sleep, and again wake up for the purpose of Rel-19 PFs. This is clearly not optimal from the point of view of NW energy savings, and it can be easily solved by ensuring that the Rel-19 PF bundles are configured consecutively to the legacy PFs.

Second, if the Rel-19 PF bundles are configured independently from legacy PFs, it may happen that a Rel-19 PF overlaps with a legacy PF (i.e., the Rel-19 and legacy PFs may have the same SFN). In this case, Rel-19 UEs and legacy UEs would be assigned to the same PF, which in the case of high legacy UE paging load could negatively affect legacy UEs that are configured with sparse paging resources for the sake of NW energy savings. This problem can be easily solved by allowing the NW to decide whether Rel-19 UEs should use legacy PFs or not depending on a specific paging load situation.

In the following, we propose a formula for calculating the SFNs of Rel-19 PFs, which addresses the problems discussed above. A Rel-19 UE can calculate the SFN of the PF from a PF bundle according to the following formula:

SFN’= (SFN + PF_offset_bundle) mod T,									(3)

where

PF_offset_bundle = bundled_PF * (1 + floor (UE_ID div N) mod N’),

bundled_PF ∈ {0,1}: indicates whether the UE calculates the index of a PF using legacy formula    or a new formula,

SFN: is calculated using legacy formula (1), i.e., as if Rel-19 UE was assigned to one of legacy PFs,

SFN’: is the index of the PF that a Rel-19 UE should monitor for paging,

N: number of legacy PFs in T,

N’: is the number of new bundled PFs in a single bundle,

T: DRX cycle of the UE, and

UE_ID: If the UE operates in eDRX as specified in clause 7.4:

-5G-S-TMSI mod 4096

else:

-5G-S-TMSI mod 1024.

Note: The total number of new bundled PFs in T is N*N’, and the total number of PFs (i.e., legacy and Rel-19) in T is N + N*N’.

The SFN of a PF that belongs to a Rel-19 PF bundle is calculated according to the following formula: SFN’= (SFN + PF_offset_bundle) mod T, where

- PF_offset_bundle = bundled_PF*(1 + floor (UE_ID div N) mod N’),

- bundled_PF ∈ {0,1}: indicates whether the UE calculates the index of a PF using legacy formula or a new formula,

- SFN: is calculated using legacy formula (1), i.e., as if Rel-19 UE was assigned to one of legacy PFs,

- SFN’: is the index of the PF that a Rel-19 UE should monitor for paging,

- N: number of legacy PFs in T,

- N’: is the number of new bundled PFs in a single bundle,

- T: DRX cycle of the UE, and

- UE_ID: If the UE operates in eDRX as specified in clause 7.4:

  - 5G-S-TMSI mod 4096

- else:

  - 5G-S-TMSI mod 1024.

- Note: The total number of new bundled PFs in T is N*N’, and the total number of PFs (i.e., legacy and Rel-19) in T is N + N*N’.

In the Appendix we present an example of how formula (3) can be used for implementing the bunding of PFs.

One of the questions that was discussed during RAN2#127 and the post email discussion concerns the gaps between bundled PFs. We think that the resolution of the gaps should be on a slot rather than a frame level, as the latter is unfavourable from the NW energy saving perspective. In our opinion, the main motivation of having gaps is to make sure that features such as PEI can co-exist with paging adaptations (i.e., PEI functionality would be broken if POs are separated less than when PEI is supposed to be transmitted prior to its associated PO as mentioned earlier), and this can be achieved by allowing gaps on a slot level. Consequently, we think that there is no need to introduce an explicit formula for configuring gaps between the PFs in a Rel-19 PF bundle, and the gaps can be simply configured, e.g., via the existing firstPDCCH-MonitoringOccasionOfPO parameter.

- The gaps between the PFs in a Rel-19 PF bundle that are on a frame level are not favourable from NW energy saving perspective.

- The gaps between the PFs in a Rel-19 PF bundle that would allow for a coexistence of paging adaptations and PEI can be simply configured, e.g., via the existing firstPDCCH-MonitoringOccasionOfPO parameter.

The resolution of gaps between the PFs in a Rel-19 PF bundle are configurable on a slot level.

Solution b): Another solution considered by RAN2 is to decrease the number of PFs by extending the set of possible values for N in the above highlighted formulas from the TS 38.304, and to compensate the decrease in the number of PFs by increasing Ns (POs per PF). In what follows we discuss the remaining unclarities related to this solution.

First, we observe that decreasing the number of PFs by extending the set of possible values for N would not be beneficial in terms of NW energy savings unless the legacy UEs are barred in the cell. This is because the cell supporting legacy UEs would still need to wake up for the sake of legacy PFs/POs that are configured according to the legacy values of parameter N.

Second, assuming that Solution b) is implemented in a way that supports legacy UEs, modifying the value range for parameters N and Ns does not address the question of how legacy and Rel-19 UEs are associated with POs. It is important to observe that without modifying the legacy formula (2) for calculating the PO index (i.e., if Rel-19 UEs use the legacy formula for calculating the PO indices) it may happen that Rel-19 and legacy UEs have the same PO index. Consequently, Rel-19 UEs and legacy UEs may be assigned to the same PO, which in case of high legacy paging load, could negatively affect legacy UEs that are configured with sparse paging resources for the sake of NW energy savings. Therefore, in order to allow for the needed flexibility to which POs to assign Rel-19 UEs, Solution b) would require a modification of the legacy formula (2) for calculating PO indices for Rel-19 UEs.

- If Solution b) supports legacy UEs through sparse legacy paging resources and Rel-19 UEs use the legacy formula for calculating a PO index, Rel-19 and legacy UEs may be assigned to the same PO, which in case of high legacy paging load could negatively affect legacy UEs.

Finally, concerning the impact of Solution b) on RRC signalling, our observation is that Solution b) can lead to excessive signalling.

For example, a typical configuration based on legacy parameters is T = 128, N = T/2 and Ns = 2. If Solution b) is implemented by configuring PFs as sparsely as legacy parameters allow (i.e., N = T/16 = 8), the value of Ns would need to be set to 16 to provide the same paging capacity as in legacy.

In another example, if a new value of nAndPagingFrameOffset is introduced to support N = 1 (i.e., nAndPagingFrameOffset = 128), the value of Ns would need to be set to 128 to provide the same paging capacity as in legacy. Consequently, since each PO within a PF is associated with one firstPDCCH-MonitoringOccasionOfPO parameter, Solution b) would lead to an excessive number of firstPDCCH-MonitoringOccasionOfPO parameters (i.e., 16 and 128 in the examples above, respectively, compared to the legacy case in which the maximum value of Ns is 4).

A possible solution to address the problem of excessive signalling is to introduce a repetitive pattern for POs within a PF. This solution, however, requires the definition of new parameters since there are no available tools to support repetitive PO patterns within a PF. On the contrary, repetitive patterns are already possible at the level of PFs (i.e., the distribution of the POs repeats in PFs), and hence in this context Solution a) would not necessarily have additional specification impact as it relies on the existing tools. Furthermore, the need to introduce the repetitive PO patterns within a PF to address the excessive signalling problem of Solution b) indirectly implies the need for defining new bundled PFs (i.e., Solution a)) since these repetitive patterns, although not called PFs, would mimic the PFs.

- Solution b) can lead to excessive signalling if Ns is set to a high value to provide sufficient paging resources since each of Ns POs within a PF would be associated with one firstPDCCH-MonitoringOccasionOfPO parameter.

- Solution a) does not suffer from the problem of excessive signalling since repetitive patterns are already possible at the level of PFs (i.e., the distribution of the POs repeats in PFs).

- Introducing a repetitive pattern for POs within a PF to address the problem of excessive signalling in case of Solution b) indirectly implies the need for defining new bundled PFs (i.e., Solution a)) as these new patterns can be interpreted as PFs.

Based on the observations made above, we propose the following:

Decreasing the number of PFs by extending the set of possible values for N is not supported as a part of paging adaptations.

Solution a) can be used in combination with a moderate increase of the Ns value ranges, i.e., a moderate increase in the maximum number of POs within a PF.

Finally, if a moderate increase of Ns values is used together with Solution a), as discussed above, there would be a need for a formula that would allow the NW to decide (i.e., depending on estimated population of legacy vs Rel-19 UEs) whether a Rel-19 UE should use a legacy PO or a Rel-19 PO. This needed flexibility can be achieved through the following modification of the legacy formula (2):

i_s’ = floor (UE_ID/N’) mod Ns’ + i_s_offset, 			(4)

where

i_s_offset∈ {0, 1, …, Ns}: The offset used to shift Rel-19 POs relative to legacy POs,

N’: is the number of new bundled PFs in a single bundle,

Ns’: The number of Rel-19 POs within a PF,

Ns: The number of legacy POs within a PF,

UE_ID: If the UE operates in eDRX as specified in clause 7.4:

-5G-S-TMSI mod 4096

else:

-5G-S-TMSI mod 1024.

Note: If 0 ≤ i_s_offset < Ns, the legacy and Rel-19 POs may overlap. Otherwise, if i_s_offset = Ns the legacy and Rel-19 POs do not overlap,

Note: The total number of POs (legacy and Rel-19) in a PF is max {Ns, Ns’} + i_s_offset.

If a moderate increase of Ns values is used together with Solution a), the following formula is used to have a flexible assignment of Rel-19 UEs to POs: i_s’ = floor (UE_ID/N’) mod Ns’ + i_s_offset, where

- i_s_offset∈ {0, 1, …, Ns}: The offset used to shift Rel-19 POs relative to legacy POs.

- N’: is the number of new bundled PFs in a single bundle, i.e., the total number of new bundled PFs in T is N*N’,

- Ns’: The number of Rel-19 POs within a PF.

- Ns: The number of legacy POs within a PF.

- UE_ID: If the UE operates in eDRX as specified in clause 7.4:

  - 5G-S-TMSI mod 4096

- else:

  - 5G-S-TMSI mod 1024.

- Note: If 0 ≤ i_s_offset < Ns, the legacy and Rel-19 POs may overlap. Otherwise, if i_s_offset = Ns the legacy and Rel-19 POs do not overlap,

- Note: The total number of POs (i.e., legacy and Rel-19) in a PF is max {Ns, Ns’} + i_s_offset.

In Example 2 in the Appendix we present an example of how formula (4) can be used for assigning Rel-19 UEs to the POs.

# 3	Conclusion

In this contribution we discuss the objective on the adaptation of common signal/channel transmissions. Based on the discussion in the previous sections we made the following observations:

Observation 1	Extending the periodicity of SSB transmissions in a cell can provide significant energy savings for the network.

Observation 2	In legacy specifications the periodicity of SSB can be 160ms for non-initial access cells.

Observation 3	Capacity cells are typically not configured with SSB transmissions with low periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding.

Observation 4	Adapting SSBs for UEs in Idle/Inactive mode such that additional SSBs are added on top of low-periodicity SSBs prior to POs makes deployment of such cells with low-rate SSB transmissions practical.

Observation 5	If SSBs are transmitted with 20ms periodicity for UEs in Idle/Inactive mode, the energy savings from other techniques, regardless of RRC state, may become ineffective.

Observation 6	Dynamic adaptation of SSB on neighbour cells need to be accompanied by corresponding dynamic adaptation of SMTC.

Observation 7	Changing the periodicity of PRACH occasions in a cell can provide significant energy savings for the network.

Observation 8	UEs in Idle/Inactive entering a new cell read SIB1 upon (re-)selection, UEs in Idle/Inactive already camped on a cell monitor their paging occasions.

Observation 9	A unified signalling method for adapting additional PRACH resources for both idle and connected UEs simplifies the dynamic control and enhances signalling efficiency by utilizing the same signalling for both scenarios.

Observation 10	The paging formulas proposed so far in RAN2 do not consider legacy PFs.

Observation 11	The gaps between the PFs in a Rel-19 PF bundle that are on a frame level are not favourable from NW energy saving perspective.

Observation 12	The gaps between the PFs in a Rel-19 PF bundle that would allow for a coexistence of paging adaptations and PEI can be simply configured, e.g., via the existing firstPDCCH-MonitoringOccasionOfPO parameter.

Observation 13	If Solution b) supports legacy UEs through sparse legacy paging resources and Rel-19 UEs use the legacy formula for calculating a PO index, Rel-19 and legacy UEs may be assigned to the same PO, which in case of high legacy paging load could negatively afect legacy UEs.

Observation 14	Solution b) can lead to excessive signalling if Ns is set to a high value to provide sufficient paging resources since each of Ns POs within a PF would be associated with one firstPDCCH-MonitoringOccasionOfPO parameter.

Observation 15	Solution a) does not suffer from the problem of excessive signalling since repetitive patterns are already possible at the level of PFs (i.e., the distribution of the POs repeats in PFs).

Observation 16	Introducing a repetitive pattern for POs within a PF to address the problem of excessive signalling in case of Solution b) indirectly implies the need for defining new bundled PFs (i.e., Solution a)) as these new patterns can be interpreted as PFs.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Support additional SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of baseline SSB transmissions in RRC connected mode.

Proposal 2	In RRC Idle/Inactive mode, on top of baseline SSB configuration, additional SSBs can (dynamically or semi-statically) be provided on-demand, e.g., additional SSBs are transmitted prior to PO.

Proposal 3	For connected mode, support dynamic adaptation of SSB in time domain based on MAC CE indication. The signaling should be common with Objective 1.

Proposal 4	Introduce mechanism for dynamic SMTC adaptation.

Proposal 5	Support activation of additional PRACH occasions dynamically for UEs in RRC idle/inactive and connected modes

Proposal 6	Support adaptation mechanism for additional PRACH resources for Rel-19 UE in Idle/inactive mode in paging occasions, and SIB1 occasions.

Proposal 7	Support adaptation mechanism for additional PRACH resources for Rel-19 UE in connected mode in paging occasions.

Proposal 8	Legacy UEs are not barred in a cell where PF/POs are adapted for Rel-19 UEs.

Proposal 9	It should be possible for the network to configure Rel-19 UEs to use legacy PFs/POs, i.e., while some other Rel-19 UEs use Rel-19 PFs/POs or configure all Rel-19 UEs to use Rel-19 PFs/POs only.

Proposal 10	The SFN of a PF that belongs to a Rel-19 PF bundle is calculated according to the following formula: SFN’= (SFN + PF_offset_bundle) mod T, where

	PF_offset_bundle = bundled_PF*(1 + floor (UE_ID div N) mod N’),

	bundled_PF ∈ {0,1}: indicates whether the UE calculates the index of a PF using legacy formula or a new formula,

	SFN: is calculated using legacy formula (1), i.e., as if Rel-19 UE was assigned to one of legacy PFs,

	SFN’: is the index of the PF that a Rel-19 UE should monitor for paging,

	N: number of legacy PFs in T,

	N’: is the number of new bundled PFs in a single bundle,

	T: DRX cycle of the UE, and

	UE_ID: If the UE operates in eDRX as specified in clause 7.4:

-	5G-S-TMSI mod 4096

else:

-	5G-S-TMSI mod 1024.

	Note: The total number of new bundled PFs in T is N*N’, and the total number of PFs (i.e., legacy and Rel-19) in T is N + N*N’.

Proposal 11	The resolution of gaps between the PFs in a Rel-19 PF bundle are configurable on a slot level.

Proposal 12	Decreasing the number of PFs by extending the set of possible values for N is not supported as a part of paging adaptations.

Proposal 13	Solution a) can be used in combination with a moderate increase of the Ns value ranges, i.e., a moderate increase in the maximum number of POs within a PF.

Proposal 14	If a moderate increase of Ns values is used together with Solution a), the following formula is used to have a flexible assignment of Rel-19 UEs to POs: i_s’ = floor (UE_ID/N’) mod Ns’ + i_s_offset, where

	i_s_offset∈ {0, 1, …, Ns}: The offset used to shift Rel-19 POs relative to legacy POs.

	N’: is the number of new bundled PFs in a single bundle, i.e., the total number of new bundled PFs in T is N*N’,

	Ns’: The number of Rel-19 POs within a PF.

	Ns: The number of legacy POs within a PF.

	UE_ID: If the UE operates in eDRX as specified in clause 7.4:

-	5G-S-TMSI mod 4096

else:

-	5G-S-TMSI mod 1024.

	Note: If 0 ≤ i_s_offset < Ns, the legacy and Rel-19 POs may overlap. Otherwise, if i_s_offset = Ns the legacy and Rel-19 POs do not overlap,

	Note: The total number of POs (i.e., legacy and Rel-19) in a PF is max {Ns, Ns’} + i_s_offset.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

# Appendix

### Examples

Example 1:

PF_offset = 2, T = 32, N = T/16 = 2, N’ = T/8 = 4, bundled_PF = 1,

UE_ID_legacy ∈ {0, 1, 2, 3, 4}, and

UE_ID_Rel_19 ∈ {5, 6, 7, 8, 9, 10, 11, 12, 13, 14},

Legacy UEs calculate the SFN of the PF according to the legacy formula given in equation (1). As a result, UEs with even UE_ID are associated with SFN = 30 and UEs with odd UE_ID are associated with SFN = 14, i.e.:

UE_ID_legacy = x ∈ {0, 2, 4}: (SFN + 2) mod 32 = (32 div 2)*(x mod 2), SFN = 30, and

UE_ID_legacy = x ∈ {1, 3}: (SFN + 2) mod 32 = (32 div 2)*(x mod 2), SFN = 14.

Rel-19 UEs calculate the SFN of the PF according to the new formula given in equation (3), i.e.:

UE_ID_Rel_19 = 5: SFN = 14, PF_offset_bundle  = 1 + floor(5 div 2) mod 4 = 3, SFN’ = (14 + 3) mod 32 = 17,

UE_ID_Rel_19 = 6: SFN = 30, PF_offset_bundle = 1 + floor(6 div 2) mod 4 = 4, SFN’ = (30 + 4) mod 32 = 2,

UE_ID_Rel_19 = 7: SFN = 14, PF_offset_bundle = 1 + floor(7 div 2) mod 4 = 4, SFN’ = (14 + 4) mod 32 = 18,

UE_ID_Rel_19 = 8: SFN = 30, PF_offset_bundle = 1 + floor(8 div 2) mod 4 = 1, SFN’ = (30 + 1) mod 32 = 31,

UE_ID_Rel_19 = 9: SFN = 14, PF_offset_bundle = 1 + floor(9 div 2) mod 4 = 1, SFN’ = (14 + 1) mod 32 = 15,

UE_ID_Rel_19 = 10: SFN = 30, PF_offset_bundle = 1 + floor(10 div 2) mod 4 = 2, SFN’= (30 + 2) mod 32 = 0,

UE_ID_Rel_19 = 11: SFN = 14, PF_offset_bundle = 1 + floor(11 div 2) mod 4 = 2, SFN’ = (14 + 2) mod 32 = 16,

UE_ID_Rel_19 = 12: SFN = 30, PF_offset_bundle = 1 + floor(12 div 2) mod 4 = 3, SFN’ = (30 + 3) mod 32 = 1,

UE_ID_Rel_19 = 13: SFN = 14, PF_offset_bundle = 1 + floor(13 div 2) mod 4 = 3, SFN’ = (14 + 3) mod 32 = 17, and

UE_ID_Rel_19 = 14: SFN = 30, PF_offset_bundle = 1 + floor(14 div 2) mod 4 = 4, SFN’ = (30 + 4) mod 32 = 2.

Figure 7: Location of legacy and Rel-19 bundled PFs and the assignment of the UEs to the PFs in Example 1.

The location of legacy and Rel-19 bundled PFs and the assignment of the UEs to the PFs are illustrated in Figure 7.

- legacy PF

UE_ID_Rel_19 = 10

UE_ID_Rel_19 = 12

UE_ID_Rel_19 = 5 and 13

UE_ID_legacy = 1 and 3

UE_ID_Rel_19 = 9

UE_ID_Rel_19 = 11

UE_ID_Rel_19 = 7

UE_ID_Rel_19 = 6 and 14

UE_ID_legacy = 0, 2 and 4

0

14

15

16

31

30

UE_ID_Rel_19 = 8

T = 32

1

2

17

18

- Rel-19 PF

Example 2:

PF_offset = 2, T = 32, N = T/16 = 2, Ns = 2, Ns’ = 6,

UE_ID_legacy ∈ {0, 1, 2, 3},

UE_ID_Rel_19 ∈ {10, 12, 15, 16, 18, 21}, and

First-PDCCH-monitoring-occasion-of-PO: 3 (for legacy PO with i_s = 0), 12 (for legacy PO with i_s = 1), 20 (for Rel-19 PO with i_s’ = 2), 28 (for Rel-19 PO with i_s’ = 3), 33 (for Rel-19 PO with i_s’ = 4), 41 (for Rel-19 PO with i_s’ = 5), 49 (for Rel-19 PO with i_s’ = 6), 54 (for Rel-19 PO with i_s’ = 7).

Both legacy and Rel-19 UEs calculate the SFN of the PF using legacy formula given in equation (1). As a result, UEs with even UE_ID are associated with SFN = 30 and UEs with odd UE_ID are associated with SFN = 14, i.e.:

UE_ID = x ∈ {0, 2, 10, 12, 16, 18}: (SFN + 2) mod 32 = (32 div 2)*(x mod 2), SFN = 30, and

UE_ID = x ∈ {1, 3, 15, 21}: (SFN + 2) mod 32 = (32 div 2)*(x mod 2), SFN = 14.

Legacy UEs calculate the index of the PO according to the legacy formula given in equation (2), i.e.:

UE_ID_legacy = 0: i_s = floor (0/2) mod 2 = 0, i_s = 0,

UE_ID_legacy = 1: i_s = floor (1/2) mod 2 = 0, i_s = 0,

UE_ID_legacy = 2: i_s = floor (2/2) mod 2 = 1, i_s = 1, and

UE_ID_legacy = 3: i_s = floor (3/2) mod 2 = 1, i_s = 1.

Rel-19 UEs calculate the index of the PO according to the new formula given in equation (4), i.e.:

UE_ID_Rel_19 = 10: i_s’ = floor (10/2) mod 6 + 2 = 7, i_s’ = 7,

UE_ID_Rel_19 = 12: i_s’ = floor (12/2) mod 6 + 2 = 2, i_s’ = 2,

UE_ID_Rel_19 = 15: i_s’ = floor (15/2) mod 6 + 2 = 3, i_s’ = 3,

UE_ID_Rel_19 = 16: i_s’ = floor (16/2) mod 6 + 2 = 4, i_s’ = 4,

UE_ID_Rel_19 = 18: i_s’ = floor (18/2) mod 6 + 2 = 5, i_s’ = 5, and

UE_ID_Rel_19 = 21: i_s’ = floor (21/2) mod 6 + 2 = 6, i_s’ = 6.

The location of legacy and Rel-19 POs and the assignment of the UEs to the PFs and POs are illustrated in Figure 8.

Figure 8 Location of legacy and Rel-19 POs and the assignment of the UEs to the POs in Example 2.

UE_ID_legacy = 1 and 3

UE_ID_Rel_19 = 15 and 21

UE_ID_legacy = 0 and 2

UE_ID_Rel_19 = 10, 12, 16 and 18

14

30

T = 32

…

…

…

…

…

…

…

…

12

3

20

28

33

41

49

54

UE_ID_legacy = 1

UE_ID_legacy = 3

UE_ID_Rel_19 = 15

UE_ID_Rel_19 = 21

- legacy PO

- Rel-19 PO

RAN1#116 Agreements

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

### RAN1#116bis Agreements

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2