3GPP TSG-RAN WG2 Meeting #127-bis					R2-2408963

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Discussion on LCM for UE-sided model for beam management

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the following remaining aspects to progress this topic.

- NW-side additional condition

- Performance monitoring

- Data collection for training

# NW-sided additional condition

Based on RAN1 discussion, it is understood that NW-side additional condition should be provided to ensure consistency between training and inference and one option agreed is using associated ID. That is, associated ID can be used for UE categorizing the measurements (as model inputs/labeling during training) and associated ID can also be used in the inference operation to identify proper models (i.e. applicable functionalities).

Observation 1: Associated ID is needs to be provided in both training and inference phase.

It means that NW-sided additional conditions would mean different configurations to affect model training e.g. size of set A / set B, composition of set A and set B, prediction window, gNB beam codebook, gNB antenna configuration, etc. Some of configurations would not be explicitly indicated because it is related to gNB implementation. In addition, it is FFS in RAN1 how to configure the associate ID i.e. how it is transmitted with respect to Set A/Set B resource configuration.

Observation 2: RAN1 is still discussing how to configure associate ID with respect to another configuration (e.g. Set A/Set B resource configuration).

RAN1 is discussing how to define associated ID and made following working assumption.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

We understand that RAN1 is discussing whether associated ID is cell specific or multiple-cell specific. Main consideration would be the granularity of categorizing the measurement. Cell specific would mean that UE categorize data per cell and per associated ID, which would require more complexity/burden in model training at UE side. In order to reduce complexity/burden, multiple cell specific ID is proposed, which can give further information to UE categorizing measurement results for similar properties.

Observation 3: RAN1 is still discussing whether associate ID is cell specific or multiple cells specific.

From signaling point of view, there could be two options to define associate ID to be applied for multiple cells.

- Area specific associated ID: associated ID(s) is provided with a certain area ID similar to system information area ID in NR. Different area ID can be used to differentiate the same associated ID. Therefore, the actual associated ID is the combination of associated ID and area ID. With this approach, the same associated ID can be applied across multiple cells with the same area ID. Allocation and mapping of area IDs to specific areas/cells should be left to implementation. At least, we can assume that area ID should be unique within a PLMN.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

- Globally unique associated IDs within PLMN: associated IDs are consistently unique as long as the cell belongs to the same PLMN. Allocation and mapping of associate IDs to specific NW sided additional conditions should be left to implementation.

The two options are quite similar from operation point of view i.e. requiring pre-allocation by operators to make it unique in PLMN. Area specific associated ID would require more specification efforts to update associated IDs with the received area ID. Nevertheless, both options are feasible from signaling point of view. Given that RAN1 is discussing the feasibility for multiple cells, it would be useful for RAN2 to indicate that it is feasible to support area/PLM specific associated ID from signaling point of view.

Proposal 1: RAN2 agree that it is feasible to support area/PLMN specific associated ID from signaling point of view.

# Performance monitoring

For beam management use case, as captured in TR 38.843, RAN1 is considering two types of performance monitoring, where type 1 is NW to control LCM operation of UE-side model and type 2 is UE to control the LCM operation.

Figure 1. LCM procedure for UE-side AI/ML model.

Contents of performance monitoring

Until now, there is no RAN1 agreement that is specific to performance monitoring other than RAN1 LS on Data Collection Requirements and Assumptions [1].

Based on the LS, we could consider two types of reporting.

- Actual prediction/measurement results: UE provides L1-RSRP and/or beam-ID(s) to gNB. The size can be variable depending on what measurement is provided and number of resources to report.

- Performance metric/event/management decision: in case of type 1/option 2, UE calculates the metric and provides to gNB. In this case, the amount data size is small (10s of bits). In case of Type2, it could be just UE’s decision outcome of model selection/activation/deactivation/switching/fallback.

Observation 4: For performance monitoring reporting, the level of data size can be different depending on whether UE report actual prediction/measurement results or performance metric/event/management decision.

Delay requirement of performance monitoring

As usual, three different signaling options (L1, MAC, RRC) are on the table in RAN1. From delay perspective, RAN1 concluded that the delay requirement of reporting for performance is near real time and provided that Near-real-time is e.g., several tens of msecs to a few seconds [1]. All signaling options are feasible to meet near-real-time requirement.

Observation 5: For performance monitoring reporting, all signaling options (L1, MAC, RRC signaling) are feasible to meet near-real-time requirement.

Signaling Flexibility

Another aspect to consider is signaling flexibility. Although L1/MAC CE can provide less delay, the format is not so flexible as RRC signaling. In beam management use case, multiple beams/L1-RSRP results may be included for multiple configured resources. If the number of configured resources to report is dynamic, more flexible signaling format would be required.

On the other hand, in case of performance metric/event/management decision, the less delay would be more beneficial to provide event situation/management decision immediately. In addition, performance metric/event/management decision requires small size of reporting.

Observation 6: For performance monitoring reporting, depending on the size and characteristics of reporting for performance monitoring, it may require more flexible format.

Since RAN1’s discussion on options for measurement results and performance metric, it would be early for RAN2 to identify which signaling option is more suitable.

Proposal 2: RAN2 wait for RAN1 inputs to identify signaling option for performance monitoring.

# Procedure for Data collection for training

# RAN1 status

In RAN1, the following has been agreed related to training.

For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Although it is a big step for UE-sided model LCM, it might be still not enough to design signaling framework for data collection for training or even if it is necessary to require a separate signaling design.

However, it might be also a good exercise to discuss high level procedure to check companies’ understanding and view.

# Data collection for training

The UE could collect data set for UE-side model training with various approaches.

- NW assisted data collection for training: In order to collect data for interested CSI-RS set A/B, the straightforward way is for NW to configure CSI-RS set.

- NW transparent data collection for training: in this case, UE vendor may train UE side model via offline training and collect data for training without dedicate network configuration. For example, the UE can collect measurement results while it is operated with legacy configuration (i.e. non-AI/ML feature). Alternatively, the UE vendor can coordinate with NW vendors to get specific network configuration information and collect data set. However, given that this can be achieved without NW support, there would be no spec impact.

Observation: for data collection for training, both NW assisted data collection and NW transparent data collection can be applicable.

It is reasonable for RAN2 to focus on NW assisted data collection for training because there is no spec impact with NW transparent data collection. However, we should keep in mind that NW assisted data collection is not mandatory to train UE-side model.

Proposal 3: RAN2 focus on NW assisted data collection for training but make it optional for UE.

# NW assisted data collection for training

In this section, NW assisted data collection for training is discussed in more detail.

Initiation of data collection procedure

It is noted that RAN1 assumes that NW will not explicitly indicate the purpose of configuration e.g. whether it is inference, performance monitoring or training. However, it is also considered that gNB would configure CSI-RS resource for training purpose i.e. without associating CSI-RS reporting.

However, it is not clear how or when NW provides a certain configuration for data collection for training yet. We could discuss all possible options from signaling point of view.

- Option 1: NW identify the need of training and required configuration via UE capability.

- Option 2: UE request the need of training and required configuration. As usual, in order for UE to request, NW should allow UE request.

- Option 3: NW identify the need of training and required configuration during applicable functionality configuration and/or performance monitoring.

Option 1 only might be tricky given that UE cannot update dynamically the status after NW provides configuration for data collection for training. On the other hands, option 2 and option 3 seem feasible to support from signaling point of view. However, we need to first understand to decide which options is most practical and useful because it is not so desirable to specify multiple options unless it is highly required.

One thing to discuss is which signaling would be involved. In option 1, it is obvious that it will rely on UE capability signaling. In option 2, given that configuration of data collection would require RRC signaling considering CSI-RS set A/B configuration, UE request signaling is likely based on RRC signaling. In option 3, given that it is implicitly identified, additional signaling may not be required unless UE need to provide additional information e.g. preferred configuration for data collection.

Proposal 4: RAN2 discuss the following potential signaling options to initiate for data collection for training procedure; 1) UE capability, 2) UE request and 3) implicit initiation.

NW configuration for data collection

Based on RAN1 discussion, UE should be aware of associated ID to ensure consistency between training and inference. In that sense, it is reasonable to assume that existing CSI-RS configuration can be reused to configure associated ID and CSI-RS set A/set B.  Although many details related to assistance ID are FFS, we could assume that existing RRC signaling (i.e. RRCReconfiguration) is used to provide configuration for data collection in high level.

Another aspect to discuss is whether configuration for data collection for training is separate from configuration for inference or for performance monitoring. In each procedure, associated ID and corresponding CSI-RS set A/set B need to be configured. Since the discussion in RAN1 is still progressing, it is premature to conclude this aspect.

Proposal 5: RAN2 assume that configuration for data collection for training is provided via RRC signaling.

Proposal 6: RAN2 wait for RAN1 input to decide structure of configuration for data collection for training.

UE response on NW configuration

From the signaling point of view, UE response may or may not be needed depending on initiation options above discussed. That is, in case of option 2, the gNB would provide configuration based on UE request. In this case, another response from UE is not necessary. On the other hand, in case of option 1 and 3, there might be cases where the UE cannot train models due to UE power/processing resource issue. In this case, UE response may be useful to indicate that the configuration is not necessary. At the same, we could also consider that NW relies on UE’s applicable functionality reporting to identify up-to-date information.

Proposal 7: RAN2 discuss the need of UE response if NW configures configuration for data collection for training without getting explicit UE request.

# Conclusion

We discussed the following three topics and provide the following proposals.

Topic1: NW-side additional condition

Observation 1: Associated ID is needs to be provided in both training and inference phase.

Observation 2: RAN1 is still discussing how to configure associate ID with respect to another configuration (e.g. Set A/Set B resource configuration).

Observation 3: RAN1 is still discussing whether associate ID is cell specific or multiple cells specific.

Proposal 1: RAN2 agree that it is feasible to support area/PLMN specific associated ID from signaling point of view.

Topic 2: Performance monitoring

Observation 4: For performance monitoring reporting, the level of data size can be different depending on whether UE report actual prediction/measurement results or performance metric/event/management decision.

Observation 5: For performance monitoring reporting, all signaling options (L1, MAC, RRC signaling) are feasible to meet near-real-time requirement.

Observation 6: For performance monitoring reporting, depending on the size and characteristics of reporting for performance monitoring, it may require more flexible format.

Proposal 2: RAN2 wait for RAN1 inputs to identify signaling option for performance monitoring.

Topic 3: Data collection for training

Proposal 3: RAN2 focus on NW assisted data collection for training but make it optional for UE.

Proposal 4: RAN2 discuss the following potential signaling options to initiate for data collection for training procedure; 1) UE capability, 2) UE request and 3) implicit initiation.

Proposal 5: RAN2 assume that configuration for data collection for training is provided via RRC signaling.

Proposal 6: RAN2 wait for RAN1 input to decide structure of configuration for data collection for training.

Proposal 7: RAN2 discuss the need of UE response if NW configures configuration for data collection for training without getting explicit UE request.

# Reference

- R2-2311720  	Reply LS on Data Collection Requirements and Assumptions (LS to RAN2)