3GPP TSG-RAN WG2 #127-bis                                             	    R2-2408965

Hefei, China, October 14th –18th, 2024

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	Measurement enhancements for LTM

Document for:	Discussion

# 1	Introduction

RAN2#127 agreed the following [1]:

- Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggSered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

RAN2#126 agreed the following [2]:

1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This paper discusses aspects related to measurement enhancements for LTM.

# 2	LTM event evaluation

## 2.1	Event dependencies

RAN2#127 agreed that any beam in the candidate RS configuration can be used for event evaluation. This implies that the UE may concurrently evaluate an event using multiple candidate beams. There are several scenarios to consider:

Case 1: Only one candidate beam meets the event condition.
This scenario is straightforward, as a single measurement report (MR) is triggered.

Case 2: More than one beam of the candidate cell meets the event condition around the same time (i.e., before issuing an MR based on any of the triggering beams).
In this case, triggering multiple measurement reports simultaneously is redundant, as all information can be included in a single MR.

Observation 1: For a given candidate cell, multiple candidate beams may meet an event trigger condition before an MR has been issued based on the triggered event.

Proposal 1: If multiple beams of the same candidate cell meet an event triggering condition and no MR has been successfully transmitted yet, only one MR should be triggered.

Case 3: One candidate beam triggers an MR, which is successfully transmitted. Shortly after, another candidate beam of the same candidate cell meets the event condition.
In this scenario, there are two options:

- Option 1: Multiple MRs may independently be triggered.

- Option 2: A new MR can only be triggered after a “sufficient” amount of time has passed since the previous MR was triggered. This requires introducing a prohibit timer that starts after an event condition is met, during which no new MRs can be triggered.

We observe that Option 1 is a special case of Option 2 by basically configuring a prohibit timer value of zero. Option 2 should be supported.

Observation 2a: For a given candidate cell, one candidate beam may meet an event trigger condition after another candidate beam has “recently” triggered a MR based on the same event.

Observation 2b: The following options are possible:

- Option 1: A new MR is independently triggered when the new candidate beam meets the event trigger condition.

- Option 2: A new MR is triggered only after the expiry of a prohibit timer, which starts once an MR has been triggered for the same candidate cell.

Observation 2c: Option 1 is a special case of Option 2 by setting the prohibit timer to zero.

Proposal 2a: Introduce a prohibit timer with the following behavior:

- The timer starts after a MR is triggered based on fulfilling an event condition for a candidate cell

- While the timer is running, no new MR can be triggered for the same candidate cell

RAN2 should further discuss the granularity of the prohibit timer.

Proposal 2b: RAN2 to discuss whether the granularity of configuration of a prohibit timer:

- Option (a): per event per candidate cell

- Option (b): per candidate cell

- Option (c): any or both Option (a) and Option (b) based on configuration

## 2.2	Multi-beam event evaluation

RAN2#127 agreed that a beam-level measurement result, as opposed to a cell-level measurement result, is used for LTM event evaluation. RAN2 should further clarify that a single-beam level measurement result is used for event evaluation. Using more than one beam for event evaluation is more complex. Additionally, multi-beam evaluation may not be necessary for reducing the overhead of measurement reporting, as this can be controlled using a prohibit timer as discussed above.

Observation 3: Defining event evaluation based on multiple beams is more complex and may not be needed, as MR overhead can be controlled with a prohibit timer.

Proposal 3: RAN2 confirms that any single beam in candidate RS configuration can be used for LTM event evaluation.

# 3	New triggers

## 3.1	Report on leave

If the UE does not send a measurement report after the leaving condition is met, measurement-related information in the network becomes outdated. Therefore, report-on-leave should be supported.

Observation 4: Without reporting measurements on leave, measurement results in the network may become outdated.

Proposal 4: UE may initiate a measurement report when the leaving condition is met.

## 3.2	Event-triggered periodic reporting

If the network receives a measurement report at both the entering and leaving conditions, additional reporting in the meantime does not provide significant value. Furthermore, it should be noted that L3 event evaluation is based on the cell level, whereas L1 event evaluation is based on the beam level. This means that while event-triggered periodic MRs based on L3 event evaluation may be useful, they are not needed for MRs based on L1 event evaluation since each candidate beam may trigger its own MR. Therefore, event-triggered periodic reporting can be deprioritized.

Proposal 5: Event-triggered periodic reporting is deprioritized.

# 4	MAC-CE-based measurement report

## 4.1	Configuration aspects

RAN#127 agreed that the event-triggered report configuration should be provided in the serving cell configuration. This agreement can be evaluated by considering the scenario where the UE is configured with CA. In this case, different UE behaviors can be considered:

- Option A: UE has one report configuration per serving cell group:

  - Option A1: UE may only send the MAC-CE report via the PCell.

  - Option A2: UE may send the MAC-CE report via any serving carrier.

- Option B: UE may have one report configuration per carrier, and UE issues MAC-CE reports on a carrier using the respective report configuration.

Observation 5a: For a UE configured with CA, the following options are possible:

- Option A: UE has one report configuration per serving cell group

- Option B: UE may have one report configuration per serving carrier, and UE sends a MAC-CE report on any carrier using the respective report configuration of that carrier.

Noting that it is the same serving gNB-DU that would receive the MAC-CE reports regardless of which carrier is used to carry the report, so one report configuration is sufficient. This implies Option A should be agreed upon. This further implies that the event-triggered report configuration does not need to be inside the serving cell configuration but rather may be within the CellGroupConfig (reverting the prior agreement).

Observation 5b: All MAC-CE reports issued on any carrier of the same serving cell group are received by the same gNB-DU, so multiple report configurations are not needed.

Proposal 6a: UE receives one event-triggered report configuration for the serving cell group.

Proposal 6b: Event-triggered report configuration is within CellGroupConfig but outside the serving cell configuration (reverting prior RAN2 agreement).

## 4.2	Report content

We propose the MAC CE report to include the following:

Proposal 7: MAC-CE-based report to include:

- Triggered event ID

- Config ID of candidate cell group, for event types LTM3/4/5, if not indicated by event ID.

- (List of) Triggering candidate beam ID(s)

- Measurement metric corresponding to triggering candidate beam ID

- Measurements of other beams of the triggering candidate cell.

- Serving PCell beam signal strength (if configured)

- FFS info about other candidate cells

- FFS format of measurements (e.g., exact vs. differential).

# Conclusion

This contribution discussed aspects related to measurement enhancements for LTM. The following observations and proposals have been made:

Event dependencies

Observation 1: For a given candidate cell, multiple candidate beams may meet an event trigger condition before an MR has been issued based on the triggered event.

Proposal 1: If multiple beams of the same candidate cell meet an event triggering condition and no MR has been successfully transmitted yet, only one MR should be triggered.

Observation 2a: For a given candidate cell, one candidate beam may meet an event trigger condition after another candidate beam has “recently” triggered a MR based on the same event.

Observation 2b: The following options are possible:

- Option 1: A new MR is independently triggered when the new candidate beam meets the event trigger condition.

- Option 2: A new MR is triggered only after the expiry of a prohibit timer, which starts once an MR has been triggered for the same candidate cell.

Observation 2c: Option 1 is a special case of Option 2 by setting the prohibit timer to zero.

Proposal 2a: Introduce a prohibit timer with the following behavior:

- The timer starts after a MR is triggered based on fulfilling an event condition for a candidate cell

- While the timer is running, no new MR can be triggered for the same candidate cell

Proposal 2b: RAN2 to discuss whether the granularity of configuration of a prohibit timer:

- Option (a): per event per candidate cell

- Option (b): per candidate cell

- Option (c): any or both Option (a) and Option (b) based on configuration

Multi-beam event evaluation

Observation 3: Defining event evaluation based on multiple beams is more complex and may not be needed, as MR overhead can be controlled with a prohibit timer.

Proposal 3: RAN2 confirms that any single beam in candidate RS configuration can be used for LTM event evaluation.

Report on leave

Observation 4: Without reporting measurements on leave, measurement results in the network may become outdated.

Proposal 4: UE may initiate a measurement report when the leaving condition is met.

Event-triggered periodic reporting

Proposal 5: Event-triggered periodic reporting is deprioritized.

Configuration aspects

Observation 5a: For a UE configured with CA, the following options are possible:

- Option A: UE has one report configuration per serving cell group

- Option B: UE may have one report configuration per serving carrier, and UE sends a MAC-CE report on any carrier using the respective report configuration of that carrier.

Observation 5b: All MAC-CE reports issued on any carrier of the same serving cell group are received by the same gNB-DU, so multiple report configurations are not needed.

Proposal 6a: UE receives one event-triggered report configuration for the serving cell group.

Proposal 6b: Event-triggered report configuration is within CellGroupConfig but outside the serving cell configuration (reverting prior RAN2 agreement).

Report content

Proposal 7: MAC-CE-based report to include:

- Triggered event ID

- Config ID of candidate cell group, for event types LTM3/4/5, if not indicated by event ID.

- (List of) Triggering candidate beam ID(s)

- Measurement metric corresponding to triggering candidate beam ID

- Measurements of other beams of the triggering candidate cell.

- Serving PCell beam signal strength (if configured)

- FFS info about other candidate cells

- FFS format of measurements (e.g., exact vs. differential).

# References

- Chairman Notes; TSG RAN WG2 Meeting #127

- Chairman Notes; TSG RAN WG2 Meeting #126