3GPP TSG-RAN WG2 Meeting #127bis	R2-2408974

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:		8.3.2

Source:				ETRI

Title:					Discussion on RRM measurement prediction

Document for:		Discussion and Decision

1. Introduction

In [POST127][030][AI mobility] RRM simulation assumptions email discussion, RAN2 discussed observation and prediction window for temporal domain case A and the rapporteur proposed a simple rule to follow. In this contribution, we discuss further observation and prediction window for temporal domain case A and propose to narrow down the combinations of observation and prediction window.

2. Discussion

2.1 Observation and prediction window

After [POST127][030][AI mobility] RRM simulation assumptions email discussion, the rapporteur made a proposal on simulation assumptions of observation and prediction window for temporal domain case A as follows [1]:

Proposal 4: For temporal domain case A, Company can report the observation window and prediction window with following limitation:

1, The ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

2, The window length should be multiple times of sampling period or measurement period of corresponding frequency range.

We understand the rapporteur’s intention which can leave flexibility to company’s implementation with simple rule to follow. However, it can result in too many options as some companies commented and therefore it may cause hardship in making consensus on the trends and capturing observations.

Observation 1: Too many options for observation and prediction window may cause hardship in making a consensus on the trends and capturing observations.

It can be helpful to consider use cases/scenarios to focus during the study to narrow down the combinations of observation and prediction window.

First, with regard to RLF/HO failure prediction, RAN2 agreed to focus on RLF detected upon T310 expiry in PCell. For RLF prediction, RAN2 study indirect RLF prediction and direct RLF prediction as follows [2]:

RLF can be predicted directly based on actual measurement result(s) e.g. SINR of PCell. In indirect RLF prediction, based on actual measurement result(s) e.g. SINR of PCell measurement result(s) is predicted at first, based on which RLF is derived. The RLF prediction yields probability of RLF happening within a time window or at time instance at least for direct RLF prediction.

In [POST127][029][AI mobility] RLF Simulation Assumption email discussion, the rapporteur proposed the definition of indirect RLF prediction as follows [3]:

Indirect RLF prediction: The L1 SINR results are predicted based on historical actual L1 SINR results of the serving cell by following intra-frequency temporal domain case A and then RLF event at one time instance is determined based on predicted L1 SINR and optional historical actual L1 SINR results without further AI/ML models.

The rapporteur commented on the prediction window for indirect RLF prediction as follows:

The assumed T310 is 1000ms in 36.839, which is proposed by [1][2][3]. If prediction window is equal or longer than T310, then only predicted L1 SINR will be used for derivation. But if prediction window is shorter than T310, then actual measured L1 SINR will be also involved in the assessment procedure.

Therefore, for indirect RLF prediction, it can be helpful to consider the prediction window equal or longer than T310 (e.g., 1000ms) to make a better decision and avoid RLF.

Observation 2: For indirect RLF prediction, it can be helpful to consider the prediction window equal or longer than T310 to make a better decision and avoid RLF.

Next, with regard to measurement event prediction, RAN2 will discuss use cases/scenarios to focus during the study in this meeting. Based on the Justification of the study item description [4], we propose to focus two use cases for measurement event prediction in our companion paper [5] with observation as below:

A “proactive” handover scheme using early measurement report based on measurement event predictions can decrease a handover failure or radio link failure and assure mobility robustness improvements.

A “short time of stay” handover avoidance based on measurement event predictions can reduce an unnecessary handover.

To enable a “proactive” handover scheme, early measurement report based on indirect measurement event prediction using predicted RRM measurement with prediction window equal or longer than TTT is the most straightforward and promising use case which is proposed in some contributions.

Observation 3: For indirect measurement event prediction, it can be helpful to consider the prediction window equal or longer than TTT to implement a proactive handover scheme for mobility robustness improvements.

For a “short time of stay” handover avoidance, indirect measurement event prediction using predicted RRM measurement with prediction window equal or longer than MTS can be helpful to skip a predicted unnecessary handover and make a better decision.

Observation 4: For indirect measurement event prediction, it can be helpful to consider the prediction window equal or longer than MTS for a “short time of stay” handover avoidance.

Proposal 1: RAN2 to consider use cases/scenarios to focus during the study to narrow down the combinations of observation and prediction window, for example:

- For indirect RLF prediction, the prediction window equal or longer than T310 (e.g., 1 sec in TR 36.839) to predict T310 expiry

- For indirect measurement event prediction, the prediction window equal or longer than TTT to implement a proactive handover scheme

- For indirect measurement event prediction, the prediction window equal or longer than MTS (e.g., 1 sec in TR 36.839) for a “short time of stay” handover avoidance.

3. Conclusion

Observation 1: Too many options for observation and prediction window may cause hardship in making a consensus on the trends and capturing observations.

Observation 2: For indirect RLF prediction, it can be helpful to consider the prediction window equal or longer than T310 to make a better decision and avoid RLF.

Observation 3: For indirect measurement event prediction, it can be helpful to consider the prediction window equal or longer than TTT to implement a proactive handover scheme for mobility robustness improvements.

Observation 4: For indirect measurement event prediction, it can be helpful to consider the prediction window equal or longer than MTS for a “short time of stay” handover avoidance.

Based on the discussion in Section 2, we propose the following:

Proposal 1: RAN2 to consider use cases/scenarios to focus during the study to narrow down the combinations of observation and prediction window, for example:

- For indirect RLF prediction, the prediction window equal or longer than T310 (e.g., 1 sec in TR 36.839) to predict T310 expiry

- For indirect measurement event prediction, the prediction window equal or longer than TTT to implement a proactive handover scheme

- For indirect measurement event prediction, the prediction window equal or longer than MTS (e.g., 1 sec in TR 36.839) for a “short time of stay” handover avoidance.

4. References

R2-2407849 Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions OPPO

R2-2406309 Text proposal of 38.744 OPPO

R2-24xxxxx Draft Summary of [POST127][029][AI mobility] RLF Simulation Assumption OPPO

RP-240082	Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR	OPPO

R2-2408978 Discussion on measurement event predictions ETRI