3GPP TSG RAN WG2 #127-bis			R2-2408980

Hefei, China, October 14th – 18th, 2024

Source: 	Fujitsu

Agena item:	8.8.5

Title:	Discussion on regenerative payload

Document for:	Discussion and decision

# Background

In RAN2#127 meeting, RAN2 made some agreements as follows [1].

Agreements on Regenerative Payload:

Common TA

- RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

- Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

RACH-less HO

- RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

SDT support

- We do not optimize UE power saving for SDT procedure in regenerative payload architecture.

On the other hand, a following issue was postponed in the last meeting.

SDT support

After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

In this contribution, we would like to discuss and provide our views on the Regenerative Payload from RAN2 perspective.

# Discussion

## RA-SDT

Since RAN2#126 meeting, discussion regarding the necessity of optimization for RA-SDT in regenerative payloads is ongoing.

In RAN2#127 meeting, RAN2 postponed the discussion about a following proposal [1]:

SDT support After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

This proposal suggests using the UE-gNB RTT as an offset after early contention resolution. This offset allows to stop the PDCCH monitoring for UE power saving. However, RAN2 consequently decided not to optimize UE power saving for SDT procedure in regenerative payload architecture.

In RA-SDT, if the UE applies the offset, the gNB has to wait for HARQ feedback from the UE before the DL transmission. According to the current specification, it is not always necessary for gNB to receive HARQ feedback before the subsequent DL transmission because whether to wait for HARQ feedback or not is up to the gNB implementation. In addition, as the downlink transmission timing becomes fixed by the offset, it impacts on the current gNB behaviours.

Observation 1: According to the current specification, it is not always necessary for gNB to receive HARQ feedback before the subsequent DL transmission.

Also in RAN2#115-e meeting, RAN2 discussed whether to introduce a new timer to control PDCCH monitoring for RA-SDT. RAN2 concluded no new timer (other than the SDT failure detection timer) is introduced to control the PDCCH monitoring during subsequent transmissions in RA-SDT in Rel-17.

Observation 2: In TN case, RAN2 concluded no new timer (other than the SDT failure detection timer) is introduced to control the PDCCH monitoring during subsequent transmissions in RA-SDT in Rel-17.

Taking the above observations into account, RAN2 shouldn’t consider UE-gNB RTT as an offset after early contention resolution.

Proposal 1: RAN2 shouldn’t consider UE-gNB RTT as an offset after early contention resolution.

## Time-based measurement

t-Service

In Rel-17 NTN, transparent payload case was only considered. In such case, the feeder link switch may cause the serving cell to change to a non-serving cell regardless of service link quality. To address this issue, time-based measurement was introduced to initiate the measurement on NTN UE before t-Service which is the time information on when the cell is going to stop serving the UE due to feeder link switch and service link switch, allowing for IDLE mode UE to have enough time to reselect the best cell.

Observation 3: Time-based measurement uses t-Service to initiate the measurement on NTN UE.

In Rel-19 NTN, satellites with regenerative payload are considered. Different from the transparent payload, feeder link switch may not affect the service link, because feeder link switch may not change the serving cell. In other words, the timing of the cell going to stop serving the UE is not the same as that of a feeder link switch.

Observation 4: In regenerative payload case, the timing of the cell going to stop serving the UE is not the same as that of a feeder link switch.

Meanwhile, in the current specification, the description of t-Service is defined as follows [2]:

t-Service

Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

Even in a case of a regenerative payload, service link switch will occur in earth-moving system, so time-based measurement is useful to indicate appropriate time for measurements, however, based on the above observations, t-Service may not be applied for feeder link switch case.

Therefore, applying time-based measurement to a regenerative payload requires modifying the current specification for t-service. For example, modifying the description of t-service as follows can handle feeder link switch without the serving cell change, enabling the time-based measurement to regenerative payload.

t-Service

Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field may applyies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

Proposal 2: In order to support time-based measurement in regenerative payload, RAN2 needs to clarify the t-Service may not be applied for feeder link switch case.

# Conclusion

In this contribution, the following proposals and observations were made:

RA-SDT

Observation 1: According to the current specification, it is not always necessary for gNB to receive HARQ feedback before the subsequent DL transmission.

Observation 2: In TN case, RAN2 concluded no new timer (other than the SDT failure detection timer) is introduced to control the PDCCH monitoring during subsequent transmissions in RA-SDT in Rel-17.

Proposal 1: RAN2 shouldn’t consider UE-gNB RTT as an offset after early contention resolution.

Time-based measurement

Observation 3: Time-based measurement uses t-Service to initiate the measurement on NTN UE.

Observation 4: In regenerative payload case, the timing of the cell going to stop serving the UE is not the same as that of a feeder link switch.

Proposal 2: In order to support time-based measurement in regenerative payload, RAN2 needs to clarify the t-Service may not be applied for feeder link switch case.

# References

- R2-2407573, Report from session on NR NTN and IoT NTN

- 3GPP TS38.331 V18.3.0, Radio Resource Control (RRC) protocol specification (Release 18)