3GPP TSG-RAN WG2 Meeting #127bis	R2- 2408981

Hefei, China, Oct 14th – 18th, 2024

Agenda Item		:	8.8.2

Source		:	LG Electronics Inc.

Title		:	Discussion on downlink coverage enhancement

Document for		:	Discussion and Decision

1. Introduction

During RAN2 #127 meeting, RAN2 made some progress with the following statements:

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In this contribution, we provide our views on DL coverage enhancements.

2. Discussion

Access Control

In the RAN2#127 meeting, it was agreed that the existing NTN bar bit can be used to bar pre-Rel19 NTN UEs from accessing a cell operating with downlink coverage enhancement (DL-CE). It is FFS about the behavior for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set. The access control to Rel-19 not supporting DL coverage enhancement should be same as that to pre-Rel19 NTN UEs, as they all cannot support DL-CE related procedure and signaling. That means that existing NTN bar bit is applicable and sufficient to bar Rel-19 NTN UEs not supporting DL coverage enhancements.

Proposal 1	Rel-19 NTN UEs that do not support DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs.

For Rel-19 UEs supporting DL-CE, a new bar bit should be introduced for NTN cell operating DL-CE to apply separate access control between Rel-19 UEs supporting DL-CE and other NTN UEs. If the new bar bit set to “notBarred” is not introduced, Rel-19 UEs supporting DL-CE would apply the existing NTN barring bit, thus if the existing NTN barring bit is set in a DL-CE cell, they are unnecessarily barred. In addition, the DL-CE cell may want to bar all UEs including UEs supporting DL-CE. For this case, the cell should be able to set the new bit to “barred”

Proposal 2	A new bar bit with code points {barred, notBarred} should be introduced to control access UEs supporting Rel19 NTN DL-CE.

SMTC impacts due to beam-hopping / larger SSB periodicity

In the RAN2#127, RAN2 decided to further consider the impact of beam hopping/SSB periodicity extension to SMTC configuration.

If the SSB periodicity is greater than 160ms, the SMTC periodicity should also be extended, as the maximum periodicity currently supported by SMTC is 160ms, as shown below

TS 38.331 v18.2.0 [1]. SMTC configuration:

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

<omitted>

-- TAG-SSB-MTC-STOP

-- ASN1STOP

Proposal 3	SMTC periodicity value range should be extended if SSB periodicity can be greater than 160ms.

The analysis on the impact of beam hopping to SMTC configuration requires the precise understanding on how the SSBs of a cell are transmitted via beam hopping. However, RAN1 did not discuss the details of beam hopping. Without a reasonable understating on how beam hopping is performed possibly with SSB periodicity extension, it is premature for RAN2 to discuss the impact of SMTC configuration.

Observation1 	No decision has been taken on the extension of the SSB periodicity and there is no progress on beam hopping in RAN1.

One contribution [2] proposed to extend NTN SMTC list or to define a new list to signal at least all neighbor SMTCs. For example, if there are six neighbor cells on intra-frequency and the transmission times of the SSBs are distributed over time, they assume that UE may need up to six SMTCs to measure neighbor cell SSBs on this frequency, but the current specification only supports a maximum of four SMTCs per frequency.

In our view, existing SMTC configuration can work without increasing the number of SMTCs for a frequency. In principle, if SMTC occurrences are aligned with the each SSB burst transmission timing of neighbor cells, UE can measure the SSBs. Such alignment is possible if SSB burst timing with beam hopping of neighbor cells are separated each other by a fixed time offset, and the SMTC periodicity is configured with a value same as the time offset. The figure1 shows the example. There are three neighbor cells, cell1, cell2, and cell3, each transmitting SSB burst with 320ms periodicity, and SSB burst of neighbor cells are separated by 20ms. Then, SMTC with 20ms periodicity can accommodate all the SSB bursts of different neighbor cells.

Figure 1

Observation 2	In case SSB burst timing with beam hopping of neighbor cells are separated each other by a fixed time offset, if SMTC periodicity is set to the as the time offset, the SMTC can accommodate the SSB burst transmissions of those neighbor cells.

Given the observation1 and 2, we think RAN2 cannot discuss the impact of beam-hopping/extension of SSB periodicity further until RAN1 makes progress and provide input to RAN2.

Proposal 4	RAN2 does not discuss the impact of beam-hopping on SMTC further until RAN1 provides necessary input on SSB periodicity extension and beam-hopping.

3. Conclusion

In this contribution, following statements are proposed:

Proposal 1	Rel-19 NTN UEs that do not support DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs.

Proposal 2	A new bar bit with code points {barred, notBarred} should be introduced to control access UEs supporting Rel19 NTN DL-CE.

Proposal 3	SMTC periodicity value range should be extended if SSB periodicity can be greater than 160ms.

Observation1 	No decision has been taken on the extension of the SSB periodicity and there is no progress on beam hopping in RAN1.

Observation 2	In case SSB burst timing with beam hopping of neighbor cells are separated each other by a fixed time offset, if SMTC periodicity is set to the as the time offset, the SMTC can accommodate the SSB burst transmissions of those neighbor cells.

Proposal 4	RAN2 does not discuss the impact of beam-hopping on SMTC further until RAN1 provides necessary input on SSB periodicity extension and beam-hopping.

4. References

[1] TS38.331	NR; Radio Resource Control (RRC); Protocol specification.

[2] R2-2407532, Downlink coverage enhancement SMTC impacts, Sequans Communications.