3GPP TSG-RAN WG2 Meeting #127bis			     R2-2408982

Hefei, China, October 14 – 18, 2024

Agenda Item:	8.7.5 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on further details of RLC enhancements for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses further details of unnecessary retransmission and proposes our views on timely RLC retransmissions.

Discussion

Avoiding unnecessary retransmission

In the RAN2#127 meeting, RAN2 made the following agreements for avoiding unnecessary retransmission.

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

As the baseline, RAN2 assume that Rx window advances first and then Tx window advances based on the STATUS report for both Tx-initiated and Rx-initiated approaches. One remaining issue for Tx window control is whether Tx-initiated approach can advance Tx window first based on obsolete SDUs. In our view, this approach may be useful only when the Tx window is stalled and no more new SDUs can be transmitted without advancing Tx window. However, we don’t think Tx window stalling occurs frequently and there is no problem to advance Tx window based on the STATUS report in most cases. Another drawback of this approach is that additional synchronization mechanism would be introduced to avoid unnecessary SDU discard at the Rx side since Rx side may discard received SDUs due to un-synchronization between Tx window and Rx window. Thus, it is sufficient only to support that Rx window advances first and then Tx window advances based on the STATUS report.

Proposal 1. For the Tx-initiated approach, advancing Tx window first is not supported, i.e., RAN2 only support that Rx window advances first and then Tx window advances based on the STATUS report.

Considering the following agreement for the Rx-initiated approach, the Rx side operation should be that

- Step 1: When the Rx side detects a missing RLC SDU, a timer is started;

- Step 2: When the timer expires, the Rx side abandons a RLC SDU causing the SN gap;

- Step 3: The Rx side moves the Rx window forward and transmits the STATUS report.

In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer.

Based on the above detailed Rx-initiated approach, the first discussion point is what the timer is in the step 1.

- Option 1: a new RLC timer is introduced;

- Option 2: t-reordering timer in PDCP is used at the RLC entity, i.e., RLC entity abandons a RLC SDU causing the SN gap after expiry of t-reordering timer in PDCP.

As per the current specification, when a missing RLC SDU is detected at the RLC entity, the PDCP entity would also detect a SN gap and then start the t-Reordering timer. After expiry of t-Reordering timer, the PDCP entity moves the receiving window forward and RLC SDU having SN lower than RX_DELIV in the PDCP entity would be discarded. Based on this observation, both options work well and only difference is dependency. In option 1, the RLC entity has its own timer to detect obsolete RLC SDUs independently and doesn’t need to check t-reordering timer and RX_DELIV in PDCP. In the option 2, however, there should be interaction between RLC and PDCP. For example, the PDCP needs to indicate to RLC entity after expiry of t-reordering timer and the RLC entity needs to check RX_DELIV to update RLC status variables. In our view, from complexity and specification impact point of view, the option 1 is clearer and better than the option 2.

Proposal 3. For the Rx-initiated approach, a new RLC timer is introduced to determine obsolete RLC SDUs.

In step 3 above, considering the below agreements, when the Rx side generates the STATUS report after advancing Rx window based on abandoned RLC SDUs, it is obvious that the abandoned RLC SDUs should be positively acknowledged in the STATUS report since if the abandoned RLC SDUs are not positively acknowledged in the STATUS report, the Tx side cannot synchronize the Tx window with the Rx window.

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances.

Proposal 4. For the Rx-initiated approach, the abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Timely RLC retransmissions

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

Before diving into each option in the above RAN2 agreements, it should be discussed first whether existing mechanisms can be reused for timely RLC retransmission without any enhancements as FFS indicated.

The opponent companies believe that if smaller values for existing STATUS report related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) are introduced and configured, the STATUS report can be transmitted frequently and there would be no more timely RLC retransmission problem. However, we think that this existing mechanism with smaller timer value would cause resource waste problem and early RLF declaration problem. Specifically, the current minimum value of t-Reassembly and t-StatusProhibit are zero. If zero is configured for both t-Reassembly and t-StatusProhibit, the receiving entity would not wait enough time to receive RLC SDU or RLC SDU segment which is still being transmitted or retransmitted at the lower layer and generate the STATUS report including pre-mature NACK. For example, as shown in the below figure, when the receiving entity detects a SN gap after receiving the RLC SDU5, the receiving entity triggers and transmits the STATUS report including NACK for RLC SDU3/4 to the transmitting entity since t-Reassembly and t-StatusProhibit are configured with zero. Then the receiving entity receives RLC SDU 3/4 successfully after transmitting the STATUS report, but the transmitting entity should retransmit the RLC SDU 3/4 unnecessarily and this resource waste cannot be avoided. We think that even though non-zero value, e.g., smaller value, is configured for these timer, same situation can occur and this resource waste problem may not be avoided due to aggressively frequent STATUS report transmission.

Observation 1. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) can generate the STATUS report with pre-mature NACK and this cause unnecessary resource waste problem.

Another problem is that a STATUS report with pre-mature NACK can be transmitted frequently and the pre-mature NACK in each STATUS report can retransmit the RLC SDU with incrementing the ReTx counter. This can reach maximum retransmission threshold more quickly than normal configuration. That is, the possibility of RLF declaration by maximum RLC retransmissions would be increased by a smaller value of RLC STATUS reporting related timers. It is obvious that early RLF by maximum RLC retransmissions is bad for timely RLC retransmission.

Observation 2. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) may trigger unnecessary RLF by maximum RLC retransmissions.

Based on the observation 4 and 5, existing mechanism with smaller timer values cannot resolve timely RLC retransmission problem and rather generates another side effects. Therefore, enhancements for timely RLC retransmission should be investigated in Rel-19.

Proposal 5. RAN2 confirm that existing mechanisms with smaller timer values are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19.

As the next step, which enhancements are needed to achieve timely RLC retransmissions for XR traffic?

The basic RLC AM retransmission mechanism is based on a STATUS report. When the transmitting entity receives the STATUS report, the transmitting entity can retransmit only RLC SDU which is indicated by NACK in the STATAU report. This means that the transmitting entity has to delay the retransmission of the RLC SDU until the STATUS report is received. In addition, even though the transmitting entity receives the STATUS report, if the received STATUS report does not include NACK for the RLC SDU in the transmitting window, the transmitting entity cannot retransmit the RLC SDU and should wait for the next STATUS report including NACK for the RLC SDU. For this reason, RAN2 should consider two approaches together for ensuring timely RLC retransmissions: one is how to get a STATUS report for timely retransmission and another is how to retransmit the RLC SDU without the STATUS report since the received STATUS report may not include NACK for the delay-critical RLC SDU timely.

For how to get a STATUS report for timely retransmission, the current STATUS report can be triggered by polling at the transmitting entity or a SN gap detection at the receiving entity. As per the current RLC specification, the transmitting entity can include a poll bit to request a STATUS report when the number of transmitted RLC PDUs or the amount of transmitted bytes reaches a certain threshold, but there is no condition to check delay status of a RLC SDU. In this situation, even though the delay-critical RLC SDU is transmitted, the transmitting entity cannot include a poll bit unless the current condition for polling is satisfied. The transmitting entity may not receive the STATUS report before expiry of discard timer for the delay-critical RLC SDU since there is no poll included in the last transmission and the receiving entity may not detect any missing RLC SDUs. As a result, the delay-critical RLC SDU may not have another retransmission opportunity before exceeding PDB and this becomes an out-dated RLC SDU unnecessarily. Thus, it would be important to receive the STATUS report after the delay-critical RLC SDU transmission to receive another retransmission opportunity and satisfy the PDB of the delay-critical RLC SDU. Thus, the transmitting entity should include a poll when the delay-critical RLC SDU is submitted to the lower layer for transmission.

Proposal 6. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer for transmission.

Another option to receive a STATUS report for timely retransmission is to enhance status report mechanism and followings can be considered for this enhancement.

- Option 1. STATUS report trigger based on bytes/number of PDUs received at the Rx entity;

- Option 2. Periodical status report;

- Option 3. Allow immediate reporting of the RLC status;

- Option 4. Indication in the STATUS report to triggers retransmission of all RLC SDUs having lower than the threshold and not yet indicated by ACK.

For option 1, considering that the transmitting entity already counts bytes and number of PDUs to include a poll for the STATUS report, even though the option 1 is introduced, the receiving entity anyway triggers a STATUS report by the poll bit from the transmitting entity and this duplicate triggering condition at the receiving entity would not be helpful to transmit a STATUS report for timely retransmission.

For option 2, the network can transmit the STATUS report for UL traffic periodically and can include a poll for DL traffic periodically by network implementation. This means that same result can be achieved by network implementation already.

For option 3, there are two proposals for this: one is to stop the t-StatusProhibit timer if it detects an SN gap and another is to ignore the condition (i.e., SN x < RX_Highest_Status or x >= RX_Next + AM_Window_Size) for triggering STATUS report. Regarding immediate STATUS report after detecting SN gap, this can be achieved by configuring smaller value of STATUS report related timers since the zero value is already possible to configure the t-StatusProhibit timer. However, as explained in the observation 4/5, this can cause pre-mature NACK in the STATUS report which can waste radio resource. Just immediate SATAUS report after detecting SN gap may not be helpful and this should be controlled by proper configuration based on the XR traffic characteristic. For ignoring the condition for triggering STATUS report (i.e., SN x < RX_Highest_Status or x >= RX_Next + AM_Window_Size), in our understanding, this condition is to ensure that the RLC Status report is transmitted after HARQ reordering and this would not be long. Thus, it is sceptical how much gain can be achieved from the option 3.

For option 4, according to the RLC specification, the STATUS report only includes status of a RLC SDU within RX_Next <= SN < RX_Highest_Status and ACK_SN in the STATUS report is indicated by RX_Highest_Status. In this condition, even if the transmitting entity receives the STATUS report, if the RLC SDU having SN higher than ACK_SN has remaining time lower than the threshold at the transmitting entity, there is no way to retransmit this delay-critical RLC SDU until the next STATUS report is received. However, considering the remaining time of the RLC SDU already lower than the threshold, the discard timer of the RLC SDU would expire and the RLC SDU is discarded unnecessarily before receiving next STATUS report. For instance, transmitting entity transmits RLC SDU 1 through RLC SDU 9 and then grey colored RLC SDUs (SDU 1 through 7) has remaining time lower than the threshold, but the receiving entity successfully receives only RLC SDU 2/4 and transmits the STATUS report including ACK_SN 5 and NACK 1/3. In this situation, even if RLC SDU 5/6/7 has remaining time lower than the threshold, the transmitting entity cannot retransmit those RLC SDUs and those can be discarded unnecessarily due to discard timer expiry.

To resolve this problem, we think that an indication can be included in the STATUS report to make the transmitting entity retransmit all RLC SDUs having remaining time lower than the threshold except already positively acknowledged RLC SDUs, i.e., can retransmit RLC SDUs which are not yet indicated by ACK and remaining time lower than the threshold.

Based on the explanation so far, we think that option 1/2/3 are not essential and may not be needed, but the option 4 can be considered to retransmit RLC SDU timely.

Proposal 7. A new indication is included in the STATUS report to make the transmitting entity retransmit all RLC SDUs having remaining time lower than the threshold and not yet indicated by ACK.

Even if enhancements to receive a STATUS report for timely retransmission is adopted, it should be also considered how to retransmit the RLC SDU without the STATUS PDU since RLC PDUs can be missed anytime and there is still possibility not to receive the STATUS report timely. For instance, if the PDU including a poll is missed, another poll bit can be included again after expiry of a poll retransmission timer or satisfying another triggering condition which would take an unacceptable latency for delay-critical RLC SDUs. In addition, even the PDU including a poll is successfully transmitted, the receiving entity cannot know exactly which RLC SDU is a delay-critical RLC SDU at the transmitting entity. The receiving entity cannot selectively include necessary NACK for the delay-critical RLC SDU and NACK in the STATUS report is determined only based on the identified missing SN from the receiving window. It would be hard to receive the STATUS report at the time point where the transmitting entity wants for retransmission of delay-critical RLC SDU. To overcome this limitation of RLC AM retransmission mechanism, we think that if remaining time of a RLC SDU in the transmitting window becomes below a threshold, e.g., the RLC SDU is indicated as a delay-critical RLC SDU by the PDCP, this delay-critical RLC SDU should be considered for retransmission without receiving NACK for this delay-critical RLC SDU. Furthermore, considering low remaining time of the delay-critical RLC SDU, one time retransmission of the delay-critical RLC SDU would not be sufficient since a retransmission of the delay-critical RLC SDU can be also failed and the transmitting entity may not receive another STATUS PDU before exceeding the PDB of the delay-critical RLC SDU. Therefore, multiple autonomous retransmissions can be considered for the delay-critical RLC SDU to satisfy the PDB.

For concerns on capacity loss due to blind autonomous retransmission, when remaining time of a RLC SDU becomes below a threshold, a delay status report at the MAC layer would be already triggered and transmitted depending on the configured value of the threshold. The network would give the UE both an uplink grant and the STATUS report together after receiving a DSR. In this situation, the transmitting entity can cancel the retransmission of the RLC SDU having lower than the threshold based on the positive acknowledgment in the STATUS report. If a network vendor has a concern to trigger retransmission of a RLC SDU by the UE itself without the network’s order, this autonomous retransmission can be controlled by a STATUS report with a new indication as proposed in the proposal 7 above. After receiving the DSR from the UE, the network can transmit a STATUS report with a new indication to make the UE retransmit RLC SDUs having remaining time lower than the threshold and not yet indicated by ACK. This means that capacity loss would be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

Observation 3. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, capacity loss may be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

There is another proposal for this issue to retransmit the RLC SDU based on HARQ feedback. However, this proposal requires huge impact to both RAN1 and RAN2 since explicit HARQ ACK/NACK for each uplink transmission should be newly defined. We think that it is unreasonable to introduce explicit HARQ ACK/NACK only for RLC retransmission determination and this should not be considered for this enhancement.

Proposal 8. If remaining time of a RLC SDU in the transmitting window becomes below a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Proposal 9. Multiple autonomous retransmissions of a delay-critical RLC SDU are supported.

Proposal 10. RLC retransmission based on HARQ NACK is not supported.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) can generate the STATUS report with pre-mature NACK and this cause unnecessary resource waste problem.

Observation 2. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) may trigger unnecessary RLF by maximum RLC retransmissions.

Observation 3. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, capacity loss may be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

Proposal 1. For the Tx-initiated approach, advancing Tx window first is not supported, i.e., RAN2 only support that Rx window advances first and then Tx window advances based on the STATUS report.

Proposal 2. No RLF is triggered by the obsolete SDU reaching maximum retransmission.

Proposal 3. For the Rx-initiated approach, a new RLC timer is introduced to determine obsolete RLC SDUs.

Proposal 4. For the Rx-initiated approach, the abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Proposal 5. RAN2 confirm that existing mechanisms with smaller timer values are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19.

Proposal 6. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer for transmission.

Proposal 7. A new indication is included in the STATUS report to make the transmitting entity retransmit all RLC SDUs having remaining time lower than the threshold and not yet indicated by ACK.

Proposal 8. If remaining time of a RLC SDU in the transmitting window becomes below a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Proposal 9. Multiple autonomous retransmissions of a delay-critical RLC SDU are supported.

Proposal 10. RLC retransmission based on HARQ NACK is not supported.