3GPP TSG-RAN WG2 Meeting #127bis	R2-2408986

Hefei, China, Oct. 14 – 18, 2024

Agenda item:	8.7.3

Source:	Samsung

Title:	Discussion on RRM measurement gaps/restrictions enhancements

Document for:	Discussion & Decision

Introduction

In the Rel-19 XR WID [1], it is specified that RAN1/2/4 will specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements, as excerpted below.

Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

Also, in the last RAN1 meeting, RAN1 has reached a working assumption [2] for enabling transmission/reception in gaps/restrictions, as excerpted below.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

In this paper, we discuss the essential aspects of the operations regarding transmission/reception in gaps/restrictions, taking into account the relevant RAN1 progress.

Discussion

RAN1 Progress Overview

RAN1 has discussed on the solutions for enabling Tx/Rx in gaps/restrictions, mainly for network-initiated solutions. The discussion [3] has started with, largely, three “high-level” alternatives, i.e.,

- Alt. 1: dynamic indication (DCI)-based solutions, i.e., skipping of gap(s)/restriction(s) can be signalled per gap/restriction occasion dynamically by DCI

- Alt. 2: semi-persistent solutions, i.e., skipping of gap(s)/restriction(s) can be activated persistently by corresponding activation command, until being deactivated by corresponding deactivation command

- Alt. 3: semi-static solutions, i.e., once configured, skipping of gap(s)/restriction(s) can be activated intermittently for a periodically configured time interval(s), or periodically configured skipping pattern(s),

among which, a solution belonging to Alt. 1 is selected as final working assumption, by considering the expected benefits brought by its flexibility [3] (compared to other Alts.), in terms of:

- application packet arrival uncertainty handling: in reality, traffic arrival is not perfectly periodic due to jitter and exact traffic arrival time is unknown

- application packet size uncertainty handling: the application packet size is not fixed and varies over time so that although the packet arrival is before the MG occasion, skipping the next MG may or may not be needed depending on the packet size

- scheduling uncertainty handling: although the packet arrival from or for a specific user collides with a MG, it does not mean that gNB decides to skip a MG since it may need to prioritize the other user which does not collide with any MG. Note that MG configuration is UE specific so not all users have the same MG occasions.

- retransmission uncertainty handling

Observation 1: Regarding enabling Tx/Rx in gaps/restrictions caused by RRM measurements, RAN1 has focused on discussing network-initiated gap/restriction skipping solutions.

Observation 2: For network-initiated gap/restriction skipping solutions, RAN1 has adopted DCI-based solution as working assumption, by considering the expected benefits brought by its flexibility, in handling uncertainties in terms of application packet arrival time/size/scheduling/retransmission.

UE-Initiated Solutions

Based on the Observations 1 and 2, a question worth discussing is, whether or not the UE-initiated solution is essential besides the network-initiated solution adopted as working assumption?

First, for downlink traffic, we believe network-initiated solution is enough to prioritize the delay-critical data transmission (if needed) over gaps/restrictions caused by RRM measurements, since network is aware of the whole situation.

Second, for uplink traffic, we should consider whether network has sufficient capability to handle the delay-critical data in terms of:

- if the uplink traffic is perfectly periodic or the jitter is not that intense in terms of its arrival time and traffic amount, network is able to properly make the decision on whether to skip a certain gap/restriction to prioritize the urgent uplink traffic.

- if the uplink traffic is not periodic or with intense jitter in terms of its arrival time and/or traffic amount, network is not able to make a decent decision on whether a gap/restriction should be skipped, if UE does not provide relevant assistant information, e.g., BSR/DSR/SR, in a timely manner. Since UE is aware of the whole situation, it is reasonable for UE to make decision on whether to skip gap(s)/restriction(s), in this case.

Consistently, if we consider the underlying rationale behind the working assumption made by RAN1 (c.f. Observation 2), we should introduce UE-initiated solution(s), to deal with the uncertainties of uplink traffic, in terms of its packet arrival time/size/scheduling/retransmission, when the uplink traffic is not periodic or with intense jitter.

Proposal 1: RAN2 to consider introducing UE-initiated solution(s) for skipping gaps/restrictions caused by RRM measurements.

In developing UE-initiated solution(s) for skipping gaps/restrictions, the target cases worth being addressed are:

- Case 1: an SR triggered by LCH with delay-critical data is delayed by overlapping gaps/restrictions. Before receiving the SR, network has no knowledge on the existence of the delay-critical uplink data, thus relying solely on network-initiated solution is not feasible in this case.

- Case 2: a CG PUSCH transmission multiplexed with DSR is delayed by overlapping gaps/restrictions. Note that if CG resource is available for accommodating DSR MAC CE plus its subheader, the corresponding SR for DSR will not be triggered/transmitted (based on SR for DSR triggering condition), and hence, network is not aware of the existence of the delay-critical data before receiving the CG PUSCH with DSR, thus relying solely on network-initiated solution is not feasible in this case.

- Case 3: a CG PUSCH transmission multiplexed with delay-critical data is delayed by overlapping gaps/restrictions. Note that, DSR/SR may not be transmitted by or before the CG PUSCH, if the CG PUSCH includes all the PDCP SDUs associated with all pending DSR(s) (based on current DSR inclusion rule), and hence, network is not aware of the existence of the delay-critical data before receiving the CG PUSCH with the delay-critical data.

It is better for RAN2 to first discuss the target cases to be addressed by the UE-initiated solution(s), before delving into the solution(s) itself.

Proposal 2: RAN2 to discuss the target case(s) to be addressed by UE-initiated gap/restriction skipping solution(s), considering:

- Case 1: an SR triggered by LCH with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Case 2: a CG PUSCH transmission multiplexed with DSR is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Case 3: a CG PUSCH transmission multiplexed with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: Regarding enabling Tx/Rx in gaps/restrictions caused by RRM measurements, RAN1 has focused on discussing network-initiated gap/restriction skipping solutions.

Observation 2: For network-initiated gap/restriction skipping solutions, RAN1 has adopted DCI-based solution as working assumption, by considering the expected benefits brought by its flexibility, in handling uncertainties in terms of application packet arrival time/size/scheduling/retransmission.

Proposal 1: RAN2 to consider introducing UE-initiated solution(s) for skipping gaps/restrictions caused by RRM measurements.

Proposal 2: RAN2 to discuss the target case(s) to be addressed by UE-initiated gap/restriction skipping solution(s), considering:

- Case 1: an SR triggered by LCH with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Case 2: a CG PUSCH transmission multiplexed with DSR is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Case 3: a CG PUSCH transmission multiplexed with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

References

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3.

[2] 3GPP TSG RAN WG1 #118, RAN1 Chair’s Notes.

[3] R1-2407289, Moderator summary #2 – Enabling TX/RX for XR during RRM measurements.