3GPP TSG-RAN WG2 Meeting #127bis	R2-2408987

Hefei, China, Oct. 14 – 18, 2024

Agenda item:		7.5.3

Source:				Samsung

Title:					Correction for delay-critical indication from PDCP to RLC

Document for:		Discussion & Decision

1 Introduction

In this paper, we address an essential correction for delay-critical indication from PDCP to RLC.

2 Discussion

The determination of delay-critical RLC SDU at RLC is totally based on the delay-critical indication from PDCP (see the highlighted part below).

TS 38.322 v18.1.0 3.1 Definitions Delay-critical RLC SDU: RLC SDU corresponding to a PDCP PDU indicated as delay-critical by PDCP (see TS 38.323 [4]).

It means that, the delay-critical indication for a PDCP Data PDU should be provided from PDCP to RLC in both:

- Case 1: A PDCP SDU becomes a delay-critical PDCP SDU, and the corresponding PDCP Data PDU has already been submitted to lower layers, corresponding to the case that the time instance a PDCP SDU becomes delay-critical is later than the time instance the corresponding PDCP Data PDU is submitted to RLC, and

- Case 2: A PDCP Data PDU containing a delay-critical PDCP SDU is being submitted to lower layers, corresponding to the case that the time instance a PDCP SDU becomes delay-critical is not later than the time instance the corresponding PDCP Data PDU is submitted to RLC.

However, current PDCP specification specifies the delay-critical indication only for Case 1, based on the highlighted part below.

TS 38.323 v18.3.0 5.15 Data volume calculation for delay status reporting For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. If a PDCP SDU becomes a delay-critical PDCP SDU, and if the corresponding PDCP Data PDU has already been submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to lower layers.

Hence, the delay-critical indication for Case 2 should be inclusively specified.

Proposal 1: Inclusively consider providing delay-critical indication from PDCP to RLC for the case when a PDCP Data PDU containing a delay-critical PDCP SDU is being submitted to lower layers. Adopt TP 1.

Text Proposal 1:

TS 38.323 v18.x.0 5.15 Data volume calculation for delay status reporting For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. If a PDCP SDU becomes a delay-critical PDCP SDU, and if the corresponding PDCP Data PDU has already been submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to lower layers. If a PDCP Data PDU containing a delay-critical PDCP SDU is submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to lower layers.

3 Conclusion

RAN2 is requested to discuss and agree the following proposal:

Proposal 1: Inclusively consider providing delay-critical indication from PDCP to RLC for the case when a PDCP Data PDU containing a delay-critical PDCP SDU is being submitted to lower layers. Adopt TP 1.