3GPP TSG-RAN WG2#127-bis	     					      R2-2408999

Hefei, China, October 14 – 18, 2024

Agenda item:	8.1.2.2

Source: 	Kyocera

Title: 	Considerations on LCM of AI/ML for NR air Interface

Document for:	Discussion

Introduction

In this contribution, we discuss the remaining issues regarding the five functionalities, the considerations on the differences between deactivation and fallback, as well as signalling considerations for activation, deactivation, switching, and fallback in the LCM of AI/ML for NR air interface.

Discussion

The Remaining issues for the five Functionalities

RAN2 Agreements

RAN2 decided to discuss different functionalities in offline discussions at the RAN2#126 meeting. In the 1st round of email discussion ([POST126][032][AIML PHY] LCM (Intel, Samsung)) [1], the rapporteur summarized the following conclusion based on the opinions from each company.

Conclusion - 2.1 Supported functionalities - Proposal 1: supported functionalities refer to functionalities that UE can indicate by using UE capability signalling. - 2.2 Configured functionalities - Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed. - 2.3 Applicable functionalities - Proposal 3: Applicable functionalities refer to functionalities that the UE is ready to apply for model inference. - Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities. - 2.4 Activated Functionalities - Proposal 5: Activated functionalities refer to functionalities already activated and performing inference. - 2.5 Available Functionalities - Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

In RAN2#127, consensus was reached on the following points [4]; however, some issues regarding specific functionalities remain, and these were discussed further.

Agreements on definitions 1 Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) 2 Applicable functionalities refer to functionalities that the UE is ready to apply for inference 3 Activated functionalities refer to functionalities already enabled for performing inference

Configured functionalities

The rapporteur reached the following proposal regarding Configured functionalities in the conclusion of the email discussion.

Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

In CA, the SCell is configured to the UE. When the SCell is applicable (i.e., the radio condition is good), the gNB activates the SCell of the UE via MAC CE. When the SCell is not applicable (i.e., the radio condition worsens), the gNB deactivates the SCell, but the gNB does not need to deconfigure the SCell from the UE.

Currently, we believe all functions are configured to the UE. In other words, no function can be activated without configuration. For example, even if the UE is capable of CA, it does not mean the CA function is configured. The CA (specifically, SCell) can be activated only after the CA is configured.

Based on existing operations, the activation of the model on the UE side can also occur only after receiving the configuration from the NW side. Without such a process, the NW side would not be able to manage the UE's operations. Since AI/ML operations in the use cases cannot be completed by the UE alone, management from the NW side is necessary. Therefore, we believe this principle should also be applied to AI/ML operations. Furthermore, our preference is for NW configuration to be applicable to both proactive reporting and reactive reporting.

RAN2 should agree that Configured functionalities are configured from the NW side to the UE.

RAN2 should agree that no AI/ML functionality can be activated without configuration as a Configured functionalities from NW side.

RAN2 should agree NW configuration is required for both proactive reporting and reactive reporting.

Direction of Signalling for the five Functionalities Conditions

In view of Proposal 1, it is also important to clarify the direction of signalling for the five different functionalities conditions in the UE-sided model in Table 1, which could be considered as baseline.

Table 1: Direction of Signalling for UE-Sided Model

| Functionalities Conditions to be achieved | Direction of Signalling |
| --- | --- |
| Supported Functionalities | UE → NW |
| Available Functionalities | UE → NW |
| Applicable Functionalities | UE → NW |
| Configured Functionalities | NW → UE |
| Activated Functionalities | NW → UE |

RAN2 should agree to use Table 1 as the baseline for the signalling directions of the five different functionalities conditions.

Applicable functionalities

The rapporteur reached the following conclusion in the email discussion.

Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

Furthermore, the rapporteur has provided the following summary and conclusions regarding this matter.

Summary from Q5 discussion: - Companies have diverged view on relationship between configured functionalities and applicable functionalities. - View 1: configured functionalities may or may not be applicable. - View 2: configured functionalities are always applicable because NW should configure functionalities within applicable functionalities. - View 3: no definition of configured functionalities. Therefore, no need to discuss.

In Proposal 2, it is stated that the activation of AI/ML functionalities can only be performed if they are configured functionalities. Therefore, we support View 2 above. In this case, we think that the NW can proceed to activate AI/ML when it is deemed configured.

RAN2 should agree that configured functionalities are always applicable.

Available Functionalities

The rapporteur reached the following conclusion in the email discussion.

Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

Regarding available functionalities, the rapporteur has also provided the following summary.

- Companies have quite diverged view on defining available functionalities separately from applicable functionalities. - Separate definition: - available functionalities are the super set of applicable functionalities e.g. due to NW sided additional conditions. - As a starting point, we could have two different definitions to discuss further details between UE and NW. - Not to define available functionalities - Applicability already imply that it has at least one available model. - Applicability is determined by UE i.e. applicability should not be defined from NW point of view only. - Need further discussion about specific cases and relationship between applicability and availability especially the case where there is no applicable functionality despite having available models.

First, we consider that Applicable functionalities are to be set as ‘applicable or not applicable’, and Available functionalities are to be set as ‘available or unavailable’. Furthermore, 'applicable/not applicable' and 'available/unavailable' should be used at different stages, since applicability relates to the situations or conditions under which a function can be applied, while availability depends on the current state or configuration.

We believe that it is possible to converge on each of these combinations. The reasons for this will be explained based on Table 2 below. This table shows the interactions between the applicability and the availability of functionalities, while it also explains why Available functionalities might be considered unnecessary.

Table 2: Applicable and Available Functionalities Results

| # | Applicability | Availability | Result |
| --- | --- | --- | --- |
| 1 | Applicable | Available | Included in Applicable functionalities |
| 2 | Applicable | Unavailable | Included in Applicable functionalities |
| 3 | Not applicable | Available ⇒ Unavailable | Converges to a state of unavailable |
| 4 | Not applicable | Unavailable | Whether to notify as not applicable functionalities is another discussion |

- 3. The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability.

- #1, #2:
According to the following agreements from RAN2#126, applicable functionalities already include available functionality. 

For #2, the following example illustrates a scenario whereby a functionality is applicable but unavailable.

  - Ex #1:
Applicable: The AI/ML model is trained and ready for inference.

Unavailable: The UE is short on computing resources, so it cannot execute this AI/ML model.

Outcome: The AI/ML model is 'applicable' because it is trained and ready to be used, but it is 'unavailable' due to the lack of UE computing resources.

- #3: We believe this case converges to the unavailable case. This can be better understood by considering the following example scenarios.

  - Ex #2:
Scenario: The computational resources (GPU or CPU) for the system are sufficient, and the AI/ML model can be executed.

Reason for Non-Applicability: The AI/ML model has not been trained on the current data or environment, making accurate predictions impossible. This means that while the functionality is not applicable in the current context, it could potentially be supported with the appropriate training on relevant data.

Outcome: Although the model can be executed, the results are unreliable, so it is considered practically unavailable.

- #4: Whether to consider this as not applicable functionalities is for a future discussion.

Based on #3, we have the following observation.

The not applicable but available case converges to being unavailable.

Based on the above, we think that Applicable functionalities are sufficient for the situations classified as applicable/not applicable and available/unavailable, and therefore, Available functionalities are unnecessary.

RAN2 should agree that Available functionalities are unnecessary, since it is included within Applicable functionalities.

Considerations on the Difference between Deactivation and Fallback

In RAN2, there have been several proposals regarding the differences between Deactivation and Fallback, and how these differences impact system performance and user experience. When considering these differences, we believe the key focus should be whether these approaches lead to significant impact on the system or user experience.

Based on our understanding, Deactivation typically refers to the complete shutdown of a specific function or service, rendering it entirely unusable; whereas, Fallback is considered an action that provides an alternative solution when a specific function is unavailable, and potentially returns to the original state once the issue is resolved.

Application of Deactivation to AI/ML Functions

When applying Deactivation to AI/ML functions, the following behaviours can be expected:

- Definition of Deactivation: Deactivation refers to the process of shutting down a specified AI/ML function. This process results in the functionality being rendered unusable. The reasons for this shutdown could include insufficient resources or an environment in which the AI/ML model is no longer applicable.

There are two possible system behaviours after Deactivation:

- No alternative operation, including legacy functionality, resulting in the complete unavailability of the function

Pros: The system remains simple, reducing the risk of unexpected behaviour or bugs.

Cons: The function becomes entirely unavailable, potentially reducing the user experience, particularly if the function is critical.

- Legacy functionality is activated as a backup

Pros: The user can continue to use the legacy function, maintaining usability and preserving the user experience. System performance is also partially retained.

Cons: The system becomes more complex, increasing the risk of bugs and maintenance issues, as well as potentially adding resource burdens.

We believe that deactivation should not automatically imply activation as a backup. The activation as a backup should be first configured by the NW. Recall that RAN2 agreed on the following regarding 'Activated functionality'. We believe that deactivated functionalities mean the opposite, which is 'Deactivated functionalities refer to functionalities that are already disabled for performing inference'.

Agreements on definitions 3 Activated functionalities refer to functionalities already enabled for performing inference

RAN2 should agree that ’Deactivated functionalities refer to functionalities already disabled for performing inference’.

The above proposal is for the functionalities. Therefore, regarding the process of 'AI/ML deactivation', it refers to simply disabling it for performing inference. That is, if the UE is not configured with any beam measurement from the NW, then the UE does not perform any beam management if the AI/ML BM functionality is deactivated.

RAN2 should agree that 'Deactivation refers to disabling for performing inference'.

Application of Fallback to AI/ML Functions

When applying Fallback to AI/ML functions, it generally involves switching to a backup function when the AI/ML function fails to perform as expected. The following options are available for transitioning:

- Legacy operation
Switching to a traditional method previously employed by the system, which offers reliable operation.

- Another AI/ML function
Switching to a different AI/ML functionality or model.

Currently, we are awaiting confirmation from RAN1 [5], but if the granularity is indeed at the sub-use case level, (2) switching to a different AI/ML functionality is not viable.

It is important to note that with Fallback, there is a need to start another procedure (e.g., switching to a corresponding legacy function or another AI/ML model), or there is a possibility of returning to the original AI/ML function once it recovers. This makes Deactivation a permanent shutdown, while Fallback involves a temporary switch. If P7 and/or P8, are agreeable, we propose the following:

RAN2 should agree that Deactivation and Fallback have different meanings.

RAN2 should agree that ‘Fallback refers to starting another procedure when a specific inference is unusable’.

Signaling Considerations for Activation, Deactivation, Switching, and Fallback

When considering the signaling for Activation, Deactivation, Fallback, Switching and Fallback in the context of AI/ML functions, the decision regarding which layer to use (DCI, MAC-CE, or RRC) is crucial. Each layer offers different trade-offs in terms of real-time responsiveness, control complexity, and flexibility.

Currently, the decision on which signalling (DCI, MAC-CE, or RRC) should be used for signaling these operations should depend on further analysis of the real-time responsiveness and control requirements of each function. Given the complexity of AI/ML functions and their interaction with legacy systems, it is essential that we wait for RAN1’s progress to provide guidance on the following:

- The real-time requirements for each function (Activation, Deactivation, Switching, and Fallback).

- The level of control information required for each operation.

- The potential impact of using different layers (L1, L2, or L3) on system performance and user experience.

RAN2 should agree to defer the decision on signaling (DCI, MAC-CE, or RRC) for Activation, Deactivation, Switching, and Fallback to await RAN1’s progress on real-time requirements and control complexity.

Conclusion

In this contribution, we discuss the issues in terms of LCM of AI/ML for NR air Interface. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should agree that Configured functionalities are configured from the NW side to the UE.

Proposal 2	RAN2 should agree that no AI/ML functionality can be activated without configuration as a Configured functionalities from NW side.

Proposal 3	RAN2 should agree NW configuration is required for both proactive reporting and reactive reporting.

Proposal 4	RAN2 should agree to use Table 1 as the baseline for the signalling directions of the five different functionalities conditions.

Proposal 5	RAN2 should agree that configured functionalities are always applicable.

Observation 1	The not applicable but available case converges to being unavailable.

Proposal 6	RAN2 should agree that Available functionalities are unnecessary, since it is included within Applicable functionalities.

Proposal 7	RAN2 should agree that ’Deactivated functionalities refer to functionalities already disabled for performing inference’.

Proposal 8	RAN2 should agree that 'Deactivation refers to disabling for performing inference'.

Proposal 9	RAN2 should agree that Deactivation and Fallback have different meanings.

Proposal 10	RAN2 should agree that ‘Fallback refers to starting another procedure when a specific inference is unusable’.

Proposal 11	RAN2 should agree to defer the decision on signaling (DCI, MAC-CE, or RRC) for Activation, Deactivation, Switching, and Fallback to await RAN1’s progress on real-time requirements and control complexity.

References

- [POST126][032][AIML PHY] LCM (Intel, Samsung)

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#125bis

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#126

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#127

- R2-2407848, “LS on applicable functionality reporting for beam management UE-sided model”, (Intel), RAN2#127