3GPP TSG-RAN WG2#127-bis		      					R2-2409000

Hefei, China, October 14 – 18, 2024				     Revision of R2-2407439

Agenda item:	8.6.2

Source: 	Kyocera

Title: 	Discussion on Inter-CU LTM

Document for:	Discussion

Introduction

RAN#105 approved the revised WID for NR mobility enhancements Phase 4 [1]. The objectives for inter-CU LTM support are stated in the revised WID, which included the three DC scenarios as follows [1].

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, the potential issues for supporting Inter-CU LTM in DC are discussed.

Discussion

Coexistence of Inter-MN MCG LTM and Intra-SN SCG LTM

According to the WID, coexistence of Intra-MN/Inter-MN MCG LTM and Inter-SN/Intra-SN SCG LTM scenarios as follows:

- Coexistence of Inter-SN SCG LTM and Intra-MN MCG LTM

  - Inter-SN SCG LTM with MN unchanged

- Coexistence of Inter-MN MCG LTM and Intra-SN SCG LTM

  - Inter-MN MCG LTM with SN unchanged

  - Inter-MN MCG LTM with SN released

In case of coexistence of Inter-MN MCG LTM and Intra-SN SCG LTM, “Inter-MN MCG LTM with SN released” has a potential issue in handling SCG LTM Config. The scenario of Inter-MN MCG LTM with SN released is depicted in Figure 1.

Figure 1 Inter-MN MCG LTM with SN released

According to the TS 38.331 [2] below, the UE maintains MCG and SCG LTM Configs in separate VarLTM-Configs and each LTM Config is configured from the MN and the SN respectively, i.e., it’s the problem that the MN cannot delete SCG LTM Config. Our understanding is shown in Figure 2.

The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE. In NR-DC, the UE may receive two independent ltm-Config: - an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and - an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, or, alternatively, embedded in an RRCReconfiguration message received via SRB1.

Before “Inter-MN MCG LTM with SN released”, the UE is configured with two LTM Configs as follows:

- MCG LTM Config: A mixture of Intra-MN cells and Inter-MN cells can be configured.

- SCG LTM Config: Only Intra-SN cells can be configured.

Figure 2 LTM Config stored in UE before Inter-MN MCG LTM with SN released

According to the TS38.300 [3] below, for the subsequent LTM, the UE maintains LTM Configs until the NW explicitly indicates to remove it. Therefore, the UE maintains SCG LTM Config after Inter-MN MCG LTM execution. Note that the SCG is released upon Inter-MN MCG LTM in this scenario.

However, it’s unclear if the target MCG will use a new DC with the same SN (i.e., the source SN for the Intra-SN LTM Config which the UE stores), i.e., the target MCG may not use the DC or may configure the UE with a new DC with the target SN different from the source SN. Since the SCG is released in the scenario of “Inter-MN MCG LTM with SN released”, the stored SCG LTM Config (for Intra-SN) is no longer necessary from the viewpoint of the target MCG.  Note that keeping DC with the same SN after Inter-MN MCG LTM execution is considered in “Inter-MN MCG LTM with SN unchanged” scenario.

Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion.

Since the SCG is released in the scenario of “Inter-MN MCG LTM with SN released”, the stored SCG LTM Config (for Intra-SN) is no longer necessary from the viewpoint of the target MCG.

According to the 38.300 [3] below, the UE should perform L1 measurement as long as LTM Config in the LTM preparation phase is applicable. Therefore, maintaining the unnecessary LTM Config increases the UE power consumption.

- The UE performs L1 measurements on the configured candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

If the UE has unnecessary SCG LTM Configurations, the UE needs to perform L1 measurements as long as LTM Config is applicable, which causes a problem that the power consumption of the UE increases.

To avoid this issue, the unnecessary SCG LTM Config should be removed upon execution of “Inter-MN MCG LTM with SN released”, which can be only done by the SN according to the current specification. However, this SN is already released when Inter-MN MCG LTM is executed. In the current specification, neither the UE nor the MN can remove SCG LTM Config. So, RAN2 should discuss how to remove the unnecessary SCG LTM Config, when no SN is configured to the UE.

The unnecessary SCG LTM Config should be removed upon execution of “Inter-MN MCG LTM with SN released”.

The SCG LTM Config can only be done by the SN according to current specification, but this SN cannot delete the SCG LTM Config because this SN was already released when Inter-MN MCG LTM was executed.

RAN2 should discuss how to remove SCG LTM Config after executing Inter-MN MCG LTM with SN released.

RAN2#127 agreed on Inter-MN MCG LTM for DC as follows [4]:

Agreements on inter-CU LTM Inter-CU MCG LTM: SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

There are two questions regarding this agreement, especially considering the SN Release case.

Question 1: What kind of “SCG configuration” is released in this agreement?

In our understanding, there are three types of SCG configurations, 1) The SCG configuration currently applied to the UE (mrdc-SecondaryCellGroupConfig IE in RRC Reconfiguration), 2) The SCG configuration stored by the UE in VarLTM-Config for MCG, and 3) The SCG configuration stored by the UE in VarLTM-Config for SCG.

Regarding Case 1) The SCG configuration currently applied to the UE (mrdc-SecondaryCellGroupConfig IE in RRC Reconfiguration), the “SCG configuration” is released if mrdc-SecondaryCellGroupConfig is set to the “Release”. However, LTM Config is not released by the MR-DC release procedure specified in section 5.3.5.10 which is performed when “(mrdc-SecondaryCellGroupConfig is set to release)” in section 5.3.5.3 in [2].

Regarding Case 2) The SCG configuration stored by the UE in VarLTM-Config for MCG, the “SCG configuration (mrdc-SecondaryCellGroupConfig)” set to Release is stored as part of MCG LTM Config by the UE (Upper part of Figure 3), i.e., this SCG configuration is included in the LTM Config for Inter-MN MCG LTM. The SCG configuration is applied to the UE upon Cell switch command to perform the Inter-MN MCG LTM; then, the same procedure as Case 1 is initiated.

Regarding Case 3) The SCG configuration stored by the UE in VarLTM-Config for SCG; in principle, only the SN configures the UE with the “SCG configuration” (Lower part of Figure 3), i.e., using ltm-CandidateToAddModList or ltm-CandidateToReleaseList [2]. This SCG configuration is for the Intra-SN SCG LTM and can only be released by the SN by ltm-CandidateToReleaseList. So, the MN cannot release SCG LTM Config.

SCG LTM Config, i.e., already stored in VarLTM-Config for SCG, is not released when mrdc-SecondaryCellGroupConfig, (i.e., currently applied SCG configuration before/upon the cell switch), is released by the MN.

SCG LTM Config can only be released by ltm-CandidateToReleaseList which is generated by the SN.

Figure 3 Two types of SCG Configuration

Question 2: What is the meaning of “how to handle SCG part up to NW implementation (e.g. release or reconfiguration)”.

In our understanding, the above agreement means a combination of the Secondary Node Release procedure and the Inter-CU MCG LTM procedure can be handled by NW implementation. According to the WID [1], however, it indicates to specify a new procedure(s) for the three DC scenarios, so RAN2 needs to consider a procedure(s) for each scenario, i.e., the inter-CU MCG LTM procedure involving the SN release.

RAN2 should clarify the agreement that “SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration)”.

Conclusion

In this contribution, the potential issues for supporting inter-CU LTM are discussed.  RAN2 is kindly asked to take into account the observations and proposal below:

Observation 1	Since the SCG is released in the scenario of “Inter-MN MCG LTM with SN released”, the stored SCG LTM Config (for Intra-SN) is no longer necessary from the viewpoint of the target MCG.

Observation 2	If the UE has unnecessary SCG LTM Configurations, the UE needs to perform L1 measurements as long as LTM Config is applicable, which causes a problem that the power consumption of the UE increases.

Observation 3	The unnecessary SCG LTM Config should be removed upon execution of “Inter-MN MCG LTM with SN released”.

Observation 4	The SCG LTM Config can only be done by the SN according to current specification, but this SN cannot delete the SCG LTM Config because this SN was already released when Inter-MN MCG LTM was executed.

Proposal 1	RAN2 should discuss how to remove SCG LTM Config after executing Inter-MN MCG LTM with SN released.

Observation 5	SCG LTM Config, i.e., already stored in VarLTM-Config for SCG, is not released when mrdc-SecondaryCellGroupConfig, (i.e., currently applied SCG configuration before/upon the cell switch), is released by the MN.

Observation 6	SCG LTM Config can only be released by ltm-CandidateToReleaseList which is generated by the SN.

Proposal 2	RAN2 should clarify the agreement that “SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration)”.

References

- RP-242356 “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom, RAN#105

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung), RAN2#127