3GPP TSG RAN2 Meeting #127bis                                                              R2-2409005

14th Oct. – 18th Oct. 2024, Hefei, China

Agenda item:	8.4.2

Source:	Samsung

Title:	Procedure and Configuration of LP-WUS in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1]. And during the previous meeting #127, with respect to procedure and configuration of LP-WUS in RRC idle/inactive mode agenda, RAN2 agreed as follows:

LP-WUS configuration Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. LP-WUS entry/exit condition Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

And RAN1 reached the following agreements regarding procedures and configurations for idle/inactive modes during meeting #118[2] :

Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. - LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS. Agreement At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams. Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. Agreement For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: - Option 1: X = 8 - Option 2: X = 16 - Option 3: X = 32 - Option 4: X = 64 - Option 5: X = 128 - Option 6: X = 256 For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option. Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives. - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded. Note: The PO mentioned above refers to legacy PO configured for the UE. Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Conclusion RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the findings from the LP-WUS/WUR study conducted during the Study Item (SI) phase in Release 18, it has been proposed that LP-WUS/WUR should be implemented as a power saving mechanism specifically designed for RRC_IDLE and RRC_INACTIVE mode devices. Furthermore, the concept of an "ultra-deep sleep state" has emerged as a potential power saving strategy for both RRC_IDLE and RRC_INACTIVE modes. By leveraging LP-WUS/WUR, a user equipment's (UE) Main Receiver (MR) can effectively transit into an ultra-deep sleep state, thereby achieving significant power savings. While in this ultra-deep sleep state, the UE can use its LP-WUS/WUR functionality to continuously monitor network signaling activities, waking up the MR when necessary.

- Configuration

The UE needs to recognize the resource on which the LP-WUS will be transmitted, regardless of whether continuous or duty cycle-based LP-WUS monitoring is used. It is essential that both the UE and gNB have a mutual understanding of this resource, and therefore, it should be communicated from the network to the UE.

For such configuration, RAN2 agreed to utilize a system information as the container. Considering a cell-specific monitoring occasion (MO), which closely resembles a PRACH configuration, we may regard SIB1 as a potential candidate for signaling LPWUS occasions (Los), as illustrated in Figure 1. It should be noted that SIB1 will be delivered through MR, whereas LP-WUS will be transmitted through LP-WUR.

Figure 1 LO configuration and LP-WUS monitoring

And we propose RAN2 to discuss and determine the configuration of LP-WUS occasions, as in SIB1.

Proposal 1. The LP-WUS related configuration for IDLE/INACTIVE state is provided via SIB1.

In line with the previous RAN1 agreement, 'The entry/exit thresholds can be individually set for various types of LR', and RAN2 agreement, ‘Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS’. As such, for UEs in Idle or Inactive mode, cells utilizing LP-WUS might necessitate separate configuration of entry and exit thresholds for each distinctive LR type. The sent entry and exit criteria must possess the ability to specify the associated LR.

Proposal 2. The configuration signaling of LP-WUS entry/exit conditions should be capable of specifying the applicable LR type.

- Entry/Exit condition

According to the previous RAN2 discussion, now we have the entry/exit condition(s) of LP-WUS monitoring configured in SIB (e.g., SIB1).

For the exit requirement from LP-WUS monitoring, in addition to the serving cell measurement result based on LR, the UE must terminate LP-WUS monitoring when either of the following conditions occur:

- Upon receiving an LP-WUS signal whose Subgroup ID matches that of the UE; or

- In the presence of MO traffic.

These conditions are necessary for exiting LP-WUS monitoring, although they might not necessarily belong to the 'exit condition' itself. This behavior could potentially be detailed through LP-WUS reception and subsequent UE actions, as well as MO traffic-originated actions.

Proposal 3. RAN2 discuss and agree the following additional termination criteria for LP-WUS monitoring:

- Upon receiving an LP-WUS signal whose Subgroup ID matches that of the UE; or

- In the presence of MO traffic.

If the entry condition is met, UE monitors LPWUS using LR. Even though UE switches to LR and monitors LPWUS, UE have to regularly wakeup the MR for inter frequency measurements wherein the UE measures the SSBs transmitted on neighboring frequency for cell (re-)selection.  Note that LR cannot be used neighbor cell measurements. Waking up MR and again switching to LR after each measurement leads to increased power consumption. Additionally, long time to wake up MR and switching to LR may lead to UE missing paging indication.

So, we propose that if criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LPWUS. If UE is already monitoring LPWUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

Proposal 4. If criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LPWUS. If UE is already monitoring LPWUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

- Subgrouping

To further enhance the energy saving gain, RAN2 agreed to take PEI subgrouping method as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping.

For UEs in RRC idle/inactive state, LP-WUS can either replace PEI (LP-WUS → PO) or coexist with PEI (LP-WUS → PEI → PO). Thus, in the above two different cases, subgrouping of LP-WUS can limit the monitoring of PF/PO, or PEI occasions, respectively.

- Scenario 1) When LP-WUS replaces PEI and subgroups are applied on LP-WUS, it reduces the number of PF/PO that UEs need to monitor.

- Scenario 2) When both LP-WUS and PEI exist and subgroups are applied on LP-WUS and PEI, it reduces the number of PEI occasions/ PF/PO that UEs need to monitor.

For Scenario 1, where UE monitors PO upon receiving LPWUS signal, UEs monitoring a PO can be divided into several LPWUS subgroups based on UE_ID.  LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +   
                  (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

For Scenario 2, where UE first monitors PEI upon receiving LPWUS signal and then monitors PO (if indicated by PEI).  LP WUS subgroup ID based on UE_ID can be determined independently from paging subgroup ID used for PEI as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +  
(LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Some companies have proposed to consider PEI subgrouping while determining the LPWUS subgroup ID. In our view this can not reduce false alarms or triggers to wakeup MR. It may reduce PDCCH monitoring in PF/PO after receiving PEI. However, when LP WUS is configured, power savings from reducing PDCCH monitoring in PF/PO after receiving PEI is not significant compared to power saving from MR deep sleep/LP WUS monitoring.  ’.

Proposal 5. LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

- LS from RAN1: UE not supporting LP-WUS on all the bands

RAN2 received the LS from RAN1[3] as follows:

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

So far it has been assumed that UE supports LP-WUS reception on all the bands supported by the MR i.e. no difference of each supported band between MR and LR from UE perspectives. Hence, before delving into the details on any issue and specification support, we should be very cautious what needs to be addressed in this release according to the RAN1 LS.

During RAN1 discussion, some companies mentioned that LR and MR (serving cell) can be operated under different bands i.e. operating LP-WUS with lower frequency band (e.g. 700MHz) and MR operating frequency band (e.g. 2.6GHz) for better coverage of LP-WUS/LP-SS. We see non-trivial issues to support it i.e. the feasibility to support the RRM relaxation and serving cell measurement offloading feature, how to define entry/exit conditions due to mismatch between LR and MR measurement etc. We think that supporting LR and MR on different carriers should be out of scope.

Proposal 6. From RAN2 perspective, supporting LR and MR on different carriers is out of scope, i.e. LR shall be always operated on the same carrier/band as serving cell.

Network may not support LP-WUS/LP-SS on all the bands supported by the network. Similarly, it is not desirable to mandate UE to support LP-WUS reception on all the bands supported by the UE. We do not see any critical issue for this.

Proposal 7. The supported bands on LR can be a subset of UE supported bands on MR.

Some specification efforts are required to support LP-WUR feature as subset of UE supported bands on MR. For example, separate UE capability per band granularity may be required to indicate LP-WUS feature. Without separate UE capability, network has a limited control to load balance IDLE/INACTIVE UEs via dedicated signalling i.e.

- For re-direction case, network has to blindly redirect LP-WUS reception capable UEs to perform cell selection on the particular band with no idea whether LP-WUS reception is supported by the UE there.

- For cell-reselection case, dedicated frequency priorities are to be provided such that LP-WUS reception capable UEs may support LP-WUS reception on (part of) frequencies when network supports LP-WUS there.

Observation 1. Per band UE capabilities may be required for UEs with limited LP-WUS supported bands.

Also, some enhancement on cell barring mechanism and cell reselection priorities in system information could be considered in order to load balance LP-WUS reception capable UEs and LP-WUS reception non-capable UEs properly.

Observation 2. RAN2 could enhance the cell barring mechanism and reselection priorities handling for UEs with limited LP-WUS supported bands.

.

- 3. Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. The LP-WUS related configuration for IDLE/INACTIVE state is provided via SIB1.

Proposal 2. The configuration signaling of LP-WUS entry/exit conditions should be capable of specifying the applicable LR type.

Proposal 3. RAN2 to discuss and agree the following additional termination criteria for LP-WUS monitoring:

- Upon receiving an LP-WUS signal whose Subgroup ID matches that of the UE; or

- In the presence of MO traffic.

Proposal 4. If criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LPWUS. If UE is already monitoring LPWUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

Proposal 5. LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Proposal 6. From RAN2 perspective, supporting LR and MR on different carriers is out of scope, i.e. LR shall be always operated on the same carrier/band as serving cell.

Proposal 7. The supported bands on LR can be a subset of UE supported bands on MR.

Observation 1. Per band UE capabilities may be required for UEs with limited LP-WUS supported bands.

Observation 2. RAN2 could enhance the cell barring mechanism and reselection priorities handling for UEs with limited LP-WUS supported bands.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] Draft_Minutes_report_RAN1#118_v030, Draft Report of 3GPP TSG RAN WG1 #118 v0.3.0, RAN1

[3] R2-2407921, LS on LP-WUS operation in IDLE/INACTIVE mode (R1-2407559; contact: Apple), Apple, RAN1