3GPP TSG RAN2 Meeting #127bis                                                              R2-2409007

14th Oct. – 18th Oct. 2024, Hefei, China

Agenda item:	8.4.4

Source:	Samsung

Title:	Procedures for LP-WUS in RRC Connected Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1][2].

During the previous meeting #127, with respect to Procedures for LP-WUS in RRC Connected Mode agenda, RAN2 agreed as follows:

Understanding of Option 1-2-1 and 1-2-2 For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

And, RAN1 made the below related summary at the meeting #118 w.r.t. various LP-WUS procedure options[3]:

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18. - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration). - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration. - LP-WUS triggers the start of a timer during which UE monitors PDCCH. - FFS the timer is existing timer and/or new timer. - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected. - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL. - No impact on RRM/RLM/BFD measurement requirements is assumed. - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not. - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE. Note: Above can be revisited considering RAN2 decisions. Agreement Select one of the following alternatives in RAN1#118bis: Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s)

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the discussions that have been carried out in RAN1 and RAN2, it has become apparent that the main and most significant use case for LP-WUS in the RRC Connected mode will revolve around  'PDCCH monitoring which is triggered by LP-WUS in conjunction with the C-DRX configuration'. In light of this use case, RAN1 is currently engaged in the thorough examination and evaluation of two options [3]:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE. Note: Above can be revisited considering RAN2 decisions.

For a quick summary, we capture the each options examples from RAN1’s discussion [4]:

Option 1-1: Replacing DCP following legacy configurations

Figure 1: LP-WUS monitor procedure in Option 1-1

Option 1-2: UE performs LP-WUS monitoring outside at least legacy C-DRX active time, and PDCCH monitoring may be additionally triggered

Figure 2: LP-WUS monitor procedure in Option 1-2 with ‘duty-cycled’ LP-WUS

Figure 3: LP-WUS monitor procedure in Option 1-2 with ‘continuous’ LP-WUS

Below Table from RAN1 discussion indicates when to start/ stop PDCCH monitoring w.r.t. different options[4]:

Table 1: LP-WUS operation methods in RRC_CONNECTED

| LP-WUS application | Details | Start PDCCH monitoring condition | Stop PDCCH monitoring condition |
| --- | --- | --- | --- |
| Direction 1: LP-WUS monitoring occasion is determined based on timer(s) related C-DRX | Option 1-1: similar to Rel-16 DCP, i.e. the LP-WUS monitoring occasion is located before drx-onDurationTimer. | If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started | It could follow legacy, i.e. at expiration of a C-DRX timer, e.g. C-DRX drx-OnDurationTimer or when the UE receives Rel-17 PDCCH skipping indication (if supported and configured). |
|  | Option 1-2: the LP-WUS monitoring occasion is located at any time outside DRX active time to indicate UE to enter into active time | If LP-WUS addressed to UE or UE's subgroup is detected |  |

Moreover, based on the discussions from RAN2#127, RAN2 progressed and established the precise PDCCH monitoring timing as follows:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the timer is started. FFS on the timer (e.g., newly defined timer or legacy timer)

The remaining question is whether the 'timer' mentioned in Option 1-2 refers to the conventional timer or a novel one. This timer is exclusively utilized for LP-WUS-capable UEs with Option 1-2, and it operates according to the following guidelines:

Observation 2. The timer used for LP-WUS-enabled UE with Option 1-2:

- Starts at a time offset after receiving the LP-WUS indication.

- If expired, the UE goes back to monitoring LP-WUS.

It remains unresolved whether this timer is newly configured or utilizes an existing traditional timer such as drx-onDurationTimer. It is noteworthy that if the abovementioned timer for Option 1-2 corresponds to drx-onDurationTimer and the LP-WUS periodicity is comparable to the DRX cycle, then Option 1-2 might be equivalent to Option 1-1.

Observation 3. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

In the meantime, while the timer is active, UE behavior needs to be specified so that connected mode communication can proceed, not solely through PDCCH monitoring but also by maintaining/extending the PDCCH monitoring. Therefore, we suggest:

Proposal 1. While the PDCCH monitoring timer is running (e.g., drx-onDurationTimer for Option 1-1 and FFS new/legacy timer for Option 1-2), UE behaves the same as in UE C-DRX on duration, e.g., applies drx-inactivityTimer for a successful PDCCH reception.

Furthermore, if Proposal 1 is accepted, we need to discuss the appropriate UE behavior after the drx-inactivityTimer expires in case of LP-WUS capable UE with Option 1-2.

Proposal 2. If Proposal 1 is agreed, RAN2 needs to discuss the UE behavior after the drx-inactivityTimer expiry, e.g, the UE goes back to LP-WUS monitoring.

- 3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the timer is started. FFS on the timer (e.g., newly defined timer or legacy timer)

Observation 2. The timer used for LP-WUS-enabled UE with Option 1-2:

- Starts at a time offset after receiving the LP-WUS indication.

- If expired, the UE goes back to monitoring LP-WUS.

Observation 3. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

Proposal 1. During the PDCCH monitoring timer is running (e.g., drx-onDurationTimer for Option 1-1 and FFS new/legacy timer for Option 1-2), UE behaves the same as in UE C-DRX on duration, e.g., apply drx-inactivityTimer for a successful PDCCH reception.

Proposal 2. When the LP-WUS capable UE is monitoring PDCCH with Option 1-2, and if the drx-inactivityTimer is expired, the UE goes back to LP-WUS monitoring if possible.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] 3GPP TR38.869 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on low-power wake up signal and receiver for NR (Release 18)

[3] Draft_Minutes_report_RAN1#118_v030, Draft Report of 3GPP TSG RAN WG1 #118 v0.3.0, RAN1

[4] R1-2407378 FL summary #2 on LP-WUS operation in CONNECTED mode, NTT DOCOMO, RAN1