3GPP TSG-RAN WG2 Meeting #127bis	R2-2409009

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.6.2

Source: 	Fujitsu

Title:	Discussion on fast LTM recovery support for Rel-19 LTM

Document for:	Discussion and Decision

- Introduction

This document discusses which cases fast LTM recovery mechanism can be applied for Rel-19 intra-CU or inter-CU LTM.

- Discussions

In RAN2#127 meeting, it was agreed as a baseline that a new Rel-19 ID would be introduced so that the UE could differentiate the target cell is intra-CU or inter-CU cell [1].

RLC and PDCP re-establishment in inter-CU LTM: 6. If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Observation 1: In RAN2#127 meeting, it was agreed as a baseline that a new Rel-19 ID would be introduced so that the UE could differentiate the target cell is intra-CU or inter-CU cell.

Also, in RAN2#127 meeting, it was mentioned this new Rel-19 ID could be used for determining whether fast LTM recovery would be performed or not.

In Rel-18 intra-CU LTM, fast LTM recovery is supported if all of the following conditions are met:

Condition 1:	attemptLTM-Switch is configured

Condition 2:	cell selection (in RRC re-establishment procedure) is triggered by detecting RLF of the MCG or failure of LTM cell switch for MCG indicated from lower layers

Condition 3:	if the selected cell is one of the LTM candidate cells associated with the MCG

For Rel-19 intra-CU or inter-CU LTM, in order to recover RRC connection after RLF and LTM cell switch, fast LTM recovery support is required.

Observation 2: In order to recover RRC connection after RLF and LTM cell switch, fast LTM recovery support is required for Rel-19.

However, in Rel-19 intra-CU or inter-CU LTM, the following cases will be caused.

Case 1: In Condition 2, the RRC re-establishment procedure is triggered by failure of inter-CU LTM cell switch for MCG indicated from lower layers. In other words, the Rel-19 ID of the target cell is different from the Rel-19 ID of the source cell, and the LTM cell switch is failed.

Case 2: In Condition 3, the selected cell is inter-CU candidate cell associated with the MCG, i.e., the Rel-19 ID of the selected candidate cell is different from the Rel-19 ID of the cell which RLF is detected or Rel-19 of the source cell.

For Case 1, security key updated is performed before LTM cell switch failure is detected. During Rel-18 discussion RAN2 confirmed that fast LTM recovery would not be possible after failure of handover with security key change. Therefore, it has been agreed that fast LTM recovery would not be supported after failure of legacy L3 handover (including CHO) and Mobility from NR which might perform security key change before the failure. For backward compatibility, we propose that fast LTM recovery is not supported for Case 1.

Proposal 1: For backword compatibility, fast LTM recovery is not supported after failure of inter-CU LTM cell switch for MCG.

For Case 2, if fast LTM recovery is supported, the UE needs to autonomously change security key to perform LTM cell switch. However, in the current SA3 discussion, it is unclear whether it is possible that the UE autonomously changes security key and SA3 conclusion will not be received before March 2025. Therefore, it may be too late to RAN2 decides whether fast LTM recovery is supported for Case 2 after receiving SA3 conclusion. RAN2 should discuss that support of fast LTM recovery for Case 2 will be decided before or after receiving SA3 conclusion.

Observation 3: In RRC re-establishment procedure, if the selected cell is inter-CU candidate cell associated with the MCG and fast LTM recovery is supported, the UE needs to autonomously change security key to perform LTM cell switch.

Observation 4: In the current SA3 discussion, it is unclear whether it is possible that the UE autonomously changes security key and SA3 conclusion will not be received before March 2025.

Proposal 2: Whether or not to support fast LTM recovery for the case the selected cell is inter-CU candidate cell associated with the MCG, RAN2 to down select:

1. Wait for SA3 conclusion, i.e., discuss and decide after March 2025

2. Decide not to support fast LTM recovery for this case, i.e., decide fast LTM recovery is supported only when the selected cell is intra-CU candidate cell associated with the MCG, without waiting for SA3 conclusion.

- Conclusion

We have the following observations and proposals:

Observation 1: In RAN2#127 meeting, it was agreed as a baseline that a new Rel-19 ID would be introduced so that the UE could differentiate the target cell is intra-CU or inter-CU cell.

Observation 2: In order to recover RRC connection after RLF and LTM cell switch, fast LTM recovery support is required for Rel-19.

Proposal 1: For backword compatibility, fast LTM recovery is not supported after failure of inter-CU LTM cell switch for MCG.

Observation 3: In RRC re-establishment procedure, if the selected cell is inter-CU candidate cell associated with the MCG and fast LTM recovery is supported, the UE needs to autonomously change security key to perform LTM cell switch.

Observation 4: In the current SA3 discussion, it is unclear whether it is possible that the UE autonomously changes security key and SA3 conclusion will not be received before March 2025.

Proposal 2: Whether or not to support fast LTM recovery for the case the selected cell is inter-CU candidate cell associated with the MCG, RAN2 to down select:

1. Wait for SA3 conclusion, i.e., discuss and decide after March 2025

2. Decide not to support fast LTM recovery for this case, i.e., decide fast LTM recovery is supported only when the selected cell is intra-CU candidate cell associated with the MCG, without waiting for SA3 conclusion.

References

[1] R2-2407571, “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung)