3GPP TSG-RAN WG2 Meeting #127bis	R2-2409014

# Hefei, China, 14 – 18 October 2024

Agenda item:	7.5.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on setting DRX SFN counter during handover

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

DRX_SFN_COUNTER initiation at HO was discussed in the previous meeting. It was agreed to come to the issue this meeting. There was also an tentative agreement on using the SFN of target cell as the reference SFN which was also left open in the previous meeting.

Agreements - We leave this up to network implementation, unless a solution that works in all scenarios (CHO/LTM/Normal HO/Reconfiguration etc) and does not require network to send an extra RRC reconfiguration message can be agreed at next meeting.

In this contribution, we further discuss the issue.

# 2	Discussion

According to current specification, the DRX_SFN_COUNTER is set upon reception of the RRC reconfiguration message (step 1 in figure 1) which is problematic for HO case since the application of the target cell configuration only happens later when the HO is executed. Esp. for CHO/LTM case, the UE would still operate on the source cell until the conditions are met, setting the counter already upon HO command reception would cause problem when the UE still operate at the source cell.

2> if DRX is (re-)configured by RRC: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1.

Figure 1 HO procedure with current spec

Observation 1: initiation of the SFN counter happens only upon HO command reception at the source cell, but no re-initiation upon HO execution or HO complete.

The proposal of using the target cell SFN as the reference would not help as based on current specification of setting the counter upon HO command reception, the target cell SFN is unknow to the UE yet at step 3 in the figure 1 and sync to target cell happens much later during HO execution.

Observation 2: using target cell SFN instead does not help as the initiation of the SFN counter time point is based on HO command reception at the source cell.

For the issue of asynchronized SFN between source and target, current RAN3 signalling already support SFN timing offset exchanged between the gNBs, and the target gNB could be able to take the SFN difference into account when setting the drx-TimeReferenceSFN value.

#### 9.2.2.75 SFN Offset This IE contains the time offset between an absolute time reference and the SFN0 start. The IE is calculated assuming that the SFN transmission started at the absolute time reference. The absolute time reference chosen is 1980-01-06 T00:00:19 International Atomic Time (TAI). IE/Group Name; Presence; Range; IE type and reference; Semantics description. SFN Time Offset; M; BIT STRING (SIZE(24)); Time offset in microseconds between the absolute time reference "1980-01-06 T00:00:19 International Atomic Time (TAI)” and the SFN0 start. The maximum usable value is (1024*10^4-1). Values higher than the maximum are discarded..

If it is assumed the HO procedure can be completed within 10s, it seems to be no issue with current procedure even based on the reception timing of RRC reconfiguration from the source gNB. However, in case of CHO/LTM, the drx-TimeReferenceSFN value sent a few SFN round ago would be absolete.

Observation 3: no issue with asynchronized system if HO is completed in 10s and if the target gNB takes the SFN time offset into account, but it might not always be guaranteed.

Observation 4: the initiation performed upon HO command reception does not work for CHO/LTM case.

Changing the timing for SFN counter initiation to when the RRC reconfiguration is applied was also brought up during the offline discussions. But it does not solve the problem of the reference SFN giving in the HO command becoming obsolete.

Observation 5: changing the DRX_SFN_COUNTER initiation when the RRC reconfiguration is applied does not help when referring to obsolete reference SFN received long ago for CHO/LTM.

For the other option brought up during the offline discussion on changing the timing for SFN counter initiation to when RRC reconfiguration complete, there was also concerns that it is problematic for normal reconfiguration case as there would be a period when the UE has applied the DRX operation without a valid SFN counter value initiated.

Observation 6: changing the DRX_SFN_COUNTER initiation when the RRC reconfiguration complete is sent is problematic for normal reconfiguration as it results in a period that the UE has applied the DRX configuration without a valid SFN counter value initiated.

The option of leaving it to NW implementation is not desirable either as it implies the NW would need to perform reconfiguration after HO complete which is not the typical NW implementation and it defeats the purpose of signalling overhead reduction with CHO and LTM. Besides, there is also an ambiguity period before the reconfiguration is done.

Observation 7: it is undesirable to leave it to NW implementation as it implies the NW needs to perform RRC reconfiguration after HO with additional overhead as well ambiguity period before reconfiguration is done.

One possible option brought up during the offline discussion was to reset the counter to 0 upon MAC reset after RRC complete message is sent HO. Since the timing for transmission of the RRC complete message to the target cell is deterministic and known to the target cell, it is enough to set it 0 without referring to the obsolete reference SFN. There is no issue even if the COUNTER is set again MAC reset after RRC reconfiguration message complete is sent even though it was set already upon RRC reconfiguration reception, so that we do not need different handling in the DRX section for HO.

Proposal 1: “set DRX_SFN_COUNTER to 0” is added to MAC reset upon transmission of RRCReconfigurationComplete for reconfiguration with sync.

## 5.12 MAC Reset … Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall: … 1> set DRX_SFN_COUNTER to 0 upon transmission of RRCReconfigurationComplete for reconfiguration with sync.

# 3	Conclusion

The following observations and proposal are made in this contribution with corresponding CR provided in R2-xxxxx.

Observation 1: initiation of the SFN counter happens only upon HO command reception at the source cell, but no re-initiation upon HO execution or HO complete.

Observation 2: using target cell SFN instead does not help as the initiation of the SFN counter time point is based on HO command reception at the source cell.

Observation 3: no issue with asynchronized system if HO is completed in 10s and if the target gNB takes the SFN time offset into account, but it might not always be guaranteed.

Observation 4: the initiation performed upon HO command reception does not work for CHO/LTM case.

Observation 5: changing the DRX_SFN_COUNTER initiation when the RRC reconfiguration is applied does not help when referring to obsolete reference SFN received long ago for CHO/LTM.

Observation 6: changing the DRX_SFN_COUNTER initiation when the RRC reconfiguration complete is sent is problematic for normal reconfiguration as it results in a period that the UE has applied the DRX configuration without a valid SFN counter value initiated.

Observation 7: it is undesirable to leave it to NW implementation as it implies the NW needs to perform RRC reconfiguration after HO with additional overhead as well ambiguity period before reconfiguration is done.

Proposal 1: “set DRX_SFN_COUNTER to 0” is added to MAC reset upon transmission of RRCReconfigurationComplete for reconfiguration with sync.

# Reference

[1] R2-2409015, MAC CR for DRX_SFN_COUNTER initiation at HO, Nokia, Nokia Shanghai Bell